3GPP TSG-WG SA2 Meeting #145E e-meeting	S2-2104268

Elbonia, May 17 – 28, 2021	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	Clarification on QoS handling for L2 and L3 Relay operations

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	5G_ProSe / Rel-17

Abstract: This contribution proposes to clarify several aspects on the QoS handling for L2 and L3 Relay operations.

# 1. Introduction/Discussion

This contribution proposes to clarify several aspects on the QoS handling for Relay operation:

1.	The QoS mapping is used by the Layer-3 UE-to-Network Relay without N3IWF to decide the PQI value of the PC5 QoS Flow, not to decide the GFBR and MFBR value for the PC5 GBR QoS flow. The GFBR and MFBR value for the PC5 GBR QoS flow are set equal to GFBR and MFBR value for the GBR QoS flow respectively.

2.	Only the procedure a Remote UE initiates when a QoS flow is setup in QoS handling for Layer-3 UE-to-Network Relay without N3IWF in clause 5.7.2.1 is described. Further description that UE-to-Network Relay also performs the UE requested PDU Session Modification to update the corresponding Uu QoS Flow when the Remote UE or Relay UE initiates the PC5 link modification procedure is added.

2.	In order to satisfy the end-end QoS requirements, if the PCF authorizes a different 5QI value, the Layer-3 UE-to-Network Relay without N3IWF may further update the PQI value based on the authorized 5QI value.

3.	Clarify that NG-RAN performing QoS enforcement for L2 Relay operation is specified in RAN2 specification.

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.304.

# * * * * First change * * * *

#### 5.7.2.1	QoS handling for Layer-3 UE-to-Network Relay without N3IWF

For a Remote UE accessing network via Layer-3 UE-to-Network Relay without N3IWF, the QoS requirement between Remote UE and UPF can be satisfied by the corresponding QoS control for the PC5 link between Remote UE and UE-to-Network Relay (PC5 QoS control) and the QoS control for the PDU session established between UE-to-Network Relay and UPF (Uu QoS control). The PC5 QoS is controlled with PC5 QoS rules and PC5 QoS parameters (e.g. PQI, GFBR, MFBR, PC5 LINK-AMBR, Range) as specified in clause 5.4 of TS 23.287 [2]. The Uu QoS is controlled with QoS rules and 5G QoS parameters (e.g. 5QI, GFBR, MFBR) as specified in clause 5.7 of TS 23.501 [4].

As shown in figure 5.7.2.1-1 below, the end-to-end QoS can be met only when the QoS requirements are properly translated and satisfied over the two legs respectively.

Figure 5.7.2.1-1: End-to-End QoS translation for Layer 3 UE-to-Network Relay solution

To achieve this, the QoS mapping can be pre-configured or provided to the UE-to-Network Relay by the PCF using Prose Policy as specified in clause 5.1.4.1. The QoS mapping includes combinations of the 5QIs and PQIs mapping as entries. The PQI shall have standardized values as defined in Table 5.7.1-1 and in Table 5.4.4-1 of TS 23.287 [2]. The 5QI shall have standardized values as defined in TS 23.501 [4] clause 5.7.4.

If the QoS Flows setup are initiated by network, the SMF can base on the PCC rules or its local configuration to generates the QoS rules and QoS Flow level QoS parameters (e.g. 5QI, GFBR, MFBR) and signal to the UE-to-Network Relay using PDU Session Establishment/Modification procedure. Then the UE-to-Network Relay decides the PC5 QoS parameters for the corresponding PC5 QoS Flow by determining the PQI based the QoS mapping, and the GFBR and MFBR values for the PC5 GBR QoS flow are set equal to the GFBR and MFBR values for the GBR QoS flow respectively. The PCF differentiates the relay traffic based on either local configuration, e.g.by DNN or S-NSSAI or by the traffic filters.

Editor's note:	The granularity of the QoS mapping rule , e.g. applicable for all the relay traffic or for the traffic of a specific service or a group of services is FFS.

If the Remote UE initiates PC5 QoS Flows setup or modification, the Remote UE provides PC5 QoS context as defined in TS 23.287 [2], clause 5.4.1.1.3 to the UE-to-Network Relay, the PC5 QoS context indicates the end-to-end QoS requirements for the traffic transmission between Remote UE and UPF. If end-end QoS requirements can be supported by an entry in QoS mapping, the UE-to-Network Relay uses the 5QI of the entry for the Uu QoS control, and uses the PQI of the entry for the PC5 QoS control. If end-end QoS requirements cannot be supported by any entries in QoS mapping, the UE-to-Network Relay, based on its implementation, decides the 5QI for the Uu QoS control and PQI for the PC5 QoS control. The UE-to-Network Relay performs the UE requested PDU session Modification as defined in TS 23.502 [5], clause 4.3.3 for authorizing the requested QoS including the 5QI and if the PCF authorizes the requested QoS with a different 5QI value, the UE-to-Network Relay may further update the PQI value based on the authorized 5QI value, and the UE-to-Network Relay performs the Layer-2 link modification procedure as defined in clause 6.4.3.4 to update the corresponding PC5 QoS Flow with the updated PQI value.

Alternatively, reflective QoS control over Uu as defined in TS 23.501 [4], clause 5.7.5.3 can be leveraged for dynamic QoS handling of Remote UE to save on signalling between SMF and Layer-3 UE-to-Network Relay. Upon reception of a DL packet with RQI on the Uu for the remote UE, based on the indicated QFI, the UE-to-Network Relay creates a new derived QoS rule or updates existing derived QoS rule corresponding to the remote UE, as defined in TS 23.501 [4]. The derived QoS rule is for UL packets from the remote UE at Uu interface.

Based on signalled QoS rules (via SMF) or derived QoS rules (Uplink Uu via reflective QoS), the UE-to-Network Relay may use the L2 Link Modification procedures as defined in clause 6.4.3.4 to either update existing PC5 QoS flow or to set up a new PC5 QoS flow (when the QFI to PC5 QoS flow mapping does not exist).

When the UE-to-Network relay deletes the derived QoS rule e.g. after the RQ Timer expires, the UE-to-Network Relay may perform L2 Link Modification procedures defined in clause 6.4.3.4 accordingly using the PQI mapped from the 5QI of the currently used QoS rule after the deletion of the derived QoS rule(s).

# * * * * Second change * * * *

#### 5.7.2.3	QoS handling for Layer-2 UE-to-Network Relay

For a Remote UE accessing network via Layer-2 UE-to-Network Relay, the existing 5G QoS control is reused between the Remote UE and the Remote UE's core network. The Remote UE's SMF provides QoS profiles to NG-RAN, how NG-RAN performs QoS enforcement for PC5 interface (between the Remote UE and UE-to-Network Relay) and Uu interface (between the UE-to-Network Relay and RAN) is specified in TS 38.XXX.

# * * * * End of changes * * * *