SA WG2 Meeting #145E (e-meeting)	S2-2104283

May 17 – 28, 2021, Elbonia

Source:	CATT

Title:	Update of local MBS service procedures

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract of the contribution: This contribution proposes some updates for local MBS service procedures.

- Introduction

This contribution proposes some updates to the description of procedures for support of local multicast service in clause 7.2.4 of TS 23.247, to align with descriptions in clauses 6.2, 7.1.2, 7.2.1, and etc.

# 2. Proposal

It is proposed to include the following changes in TS 23.247.

******************** 1st Change ********************

7.2.4	Support of local multicast service

7.2.4.1	General

The procedures for local multicast service contains the ones for local multicast service with the location-dependent content, and the ones for limited local multicast service distribution, as described in clause 6.2.

7.2.4.2	Support of local multicast service with the location-dependent content

7.2.4.2.1	Local multicast session join procedure

Editor's note:	Detailed additions to Multicast context and Multicast flow setup/modification via PDU Session Modification procedure are FFS.

The local multicast session join procedure is performed as defined in clause 7.2.1 with the following additions:

-	The local multicast session is configured as described in clause 7.2.4.2.2.

-	When SMF requests MB-SMF information via Nnrf_NFDiscovery request (MBS Session ID), the NRF provides information about the MB-SMF(s) serving the MBS service area(s), via Nnrf_NFDiscovery response (MB-SMF ID, Area Session ID, MBS service area). The SMF selects the MB-SMF based on the location area where the UE is residing.

-	If SMF has no information about the multicast session context for the indicated MBS Session ID and Area Session ID, SMF interacts with MB-SMF to retrieve QoS information of the multicast QoS flow(s) for the MBS Session ID and Area Session ID.

-	SMF requests the AMF to transfer N2 message to the RAN node using the Nsmf_PDUSession_UpdateSMContext response, to provide the NG-RAN with multicast session information which additionally includes the Area Session ID and MBS service area.

-	The RAN uses the received MBS Session ID and Area Session ID to determine the local multicast session context and whether the user plane for the local multicast session is already established.

Editor's note:	Services mentioned in this clause needs to be updated to match the agreements on clause 9 and other clauses.

7.2.4.2.2	Configuration for local MBS

Editor's note:	Further details of are FFS.

For local MBS, the configuration procedure for the UE is optional and performed as defined in clause 7.1.1.1 with the following additions:

-	Multiple AFs may register for the same multicast session but different MBS service areas. The NEF selects MB-SMF(s) as ingress control node(s) for different MBS service areas.

-	If presented, the NEF maps possible external identifiers for MBS service areas to network-internal identifiers (e.g. list of cells, TAIs).

-	MB-SMF allocates Area Session ID, and updates its NF profile towards the NRF with the MBS Session ID and Area Session ID.

-	The policy of Multicast session is determined based on MBS session ID, Area Session ID and MBS service area.

-	The MB-SMF may select the MB-UPF based on the MBS service area.

-	The MBS service area(s) can be indicated to the UE via Service Announcement.

7.2.4.2.3	Handover procedure

Editor's note:	Further details for Handover procedure are FFS.

The Handover procedure for the UE is performed as defined in clause 7.2.3 with the following additions:

-	Before the Handover, the UE is camping at Source RAN and receiving multicast data corresponding to the MBS Session ID and Area Session ID.

-	Source RAN includes MBS Session ID, Area Session ID and MBS service area to the Target RAN.

-	Target RAN determines whether to establish the forwarding resources and multicast distribution for MBS Session ID and Area Session ID provided by Source RAN, based on MBS Session ID, Area Session ID and MBS service area. To determine the forwarding resources for location-dependent contents delivery, Target RAN may only check whether or not the Area Session ID is the same if Target RAN already established the session context of the multicast session ID.

-	Target RAN responses to Source RAN, with the accepted MBS Session ID and Area Session ID. When Target RAN supports multicast but the UE is no longer in the MBS service area, Target RAN rejects the multicast session handover for the UE with a cause.

7.2.4.3	Support of multicast service available within a limited area

7.2.4.3.1	Local MBS service area information provided by AF

For local multicast services, the AF provides MBS service area information to the UE and the 5GC as specified in clause 7.1.1.1 with the following differences and clarifications:

-	In step 6 and step 18 for a Service Announcement, the AF provides MBS service area information to the UE. The local MBS service indication may be included to indicate that this multicast service is local service.

-	In step 7, the MBS service area information for a multicast session is provided by the AF.

7.2.4.3.2	Multicast Session join procedure for multicast service available within a limited area

For the case that the multicast service is only available within a limited area, the UEs outside the MBS service area are not allowed to join the multicast service.

The multicast session join for multicast service available within a limited area are performed as specified in clause 7.2.1.3 with the following differences and clarifications:

-	The local multicast session has been configured where the AF provided the MBS service area information as specified in clause 7.2.4.3.1.

-	The UE has information about local multicast service including MBS service area and local MBS service indication via Service Announcement as specified in clause 7.2.4.3.1.

-	In step 1, if the UE determines that it is in the MBS service area based on the information about local multicast service obtained via Service Announcement, the UE sends the PDU Session Modification Request (MBS Session ID) to join the multicast group. If the UE determines that it is outside the MBS service area, the UE does not send the Join Request.

-	In step 4, the SMF obtains the MBS service area (i.e. Cell ID list or TAI list) of the indicated MBS session from the MB-SMF.

-	In step 5, the SMF checks whether the UE is inside or outside the MBS service area.

-	If the SMF detects that the UE is inside the MBS service area, the SMF continues the process of multicast session join, i.e. sending the Nsmf_PDUSession_UpdateSMContext response to the AMF as specified in clause 7.2.1.3.

-	If the SMF detects that the UE is outside the MBS service area, the SMF rejects the multicast session join via the Nsmf_PDUSession_UpdateSMContext response. The response includes the reject reason (outside of local service area) and the MBS service area (i.e. Cell ID list or TAI list).

In this case, the MBS Session establishment (i.e. resources establishment for MBS traffic delivery) for the UE is not performed.

NOTE 1:	There may be the case that the UE determines that it is inside the MBS service area based on the geographical area information or civic address information as MBS service area information provided by the AF, so sends a Join Request but the UE is outside the MBS service area.

NOTE 2:	Which SM NAS message is used to deliver the multicast session join reject to the UE (e.g. PDU Session Modification Reject) is defined in stage 3 specifications.

-	For the UE that received the multicast session join reject from the SMF, later, if the UE detects that it is inside the MBS service area based on the MBS service area provided in the multicast session join reject message, the UE sends the PDU Session Modification Request (MBS Session ID) to join the multicast group.

-	The multicast session information sent by the CN to the NG-RAN additionally includes the MBS service area information.

******************** End of Change ********************