3GPP TSG-WG SA2 Meeting #145E e-meeting 	S2-2104313

Elbonia, May 17 – 28, 2021

Source:	Nokia, Nokia Shanghai-Bell

Title:	5MBS Information in UDR

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract: This contribution defines 5MBS information stored in the UDM, following proposals in S2-2103840

# Introduction

From TR 23.757 key issue 1 conclusions:

For multicast service parameters storage, the UDR shall be able to store the AF provisioned or preconfigured service parameters per MBS session

Such information is helpful for policy decisions by the PCF

The proposed text is derived from clause 6.2.1.3 of TS 23.503.

# 2. Text Proposal

It is proposed to agree the following changes for TR 23.757.

## 6.10	Policy control for Multicast and Broadcast services

### 6.10.1.	General

The policy and charging control framework as defined in TS 23.503 [7] applies to Multicast and Broadcast services in the following aspects:

Editor's note:	Whether session binding is applicable for MBS Session policy is FFS.

-	QoS Flow binding: For an MBS Session, QoS Flow binding is the association of a PCC rule to a QoS Flow within an MBS Session. The MB-SMF performs QoS Flow binding for an MBS Session in the same way as the SMF for a PDU Session.

Editor's note:	It is FFS whether any other aspects or differences compared to existing PCC framework are required for MBS policy control.

### 6.10.2.	MBS Session policy control data in UDR

The policy control profile information provided by the UDR at MBS Session establishment, using Nudr service for Data Set "Policy Data" and Data Subset "MBS Session policy control data" is described in Table 6.4.1-1.

Table 6.10.2-1: MBS Session policy control subscription information

| Information name | Description | Category |
| --- | --- | --- |
| Subscriber categories | List of category identifiers associated with the subscriber | Optional |
| Subscribed GBR | Maximum aggregate bitrate that can be provided across all GBR QoS Flows for the DNN and S-NSSAI. | Optional |
| QoS flows | Traffic filters and QoS characteristic of QoS flows associated with the multicast session | Optional |

Editor's note:	Parameters in the table are FFS.

The Subscriber category may comprise any number of identifiers associated with the subscriber (e.g. gold, silver, etc.). Each identifier associates operator defined policies to the subscriber that belong to that category.

The policy subscription profile may be extended with operator-specific information. Operator-specific extensions may be added both to any specific part of the policy control subscription information (e.g. to the subscriber category part), or as a new optional information block.

Handling of operator specific policy data by the PCF is out of scope of this specification in this release.