3GPP TSG-SA2 Meeting #145E	S2-2104321

# Electronic,  May 17 – May 28, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0593 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | eNA_ Update of Slice Data Rate Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the conclusion of Key Issue #5 in 23700-40, the following new functionality in the 5GS is needed: - Define a new AnalyticsID or reuse an available AnalyticsID，that provides UL/DL average throughput per S-NSSAI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update new AnalyticsID and related description to provide UL/DL average throughput per S-NSSAI, align with the conclution of 23700-40. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification of eNS and eNA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1, 6.x(new), 6.x.1(new), 6.x.2(new), 6.x.3(new), 6.x.4(new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------First Change---------------------------------------------

7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF, MFAF |
|  | Notify |  | NWDAF, DCCF, MFAF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF (i.e. NWDAF (AnLF), NWDAF (MTLF) or NWDAF (AnLF+MTLF)). |  |  |  |

Editor's note:	It is FFS to define the Nnwdaf_MLModelInfo service, i.e. a request-response mode to request the ML Model.

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level of a Network Slice Instance reported either as notification of crossing of a given threshold or as periodic notification (if no threshold is provided). |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| Slice Data Rate limitation | Analytics ID: Slice Data Rate limitation | Analytics on UL/DL average throughput per S-NSSAI over the measurement period. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |

----------------------------------------Next Change---------------------------------------------

6.X	Slice Data Rate limitation Analytics

6.X.1	General

This clause specifies how an NWDAF can provide Slice Data Rate limitation Analytics which provides Slice Data Rate analytics per slice (i.e. average throughput) to a service consumer (e.g. PCF).

The Slice Data Rate limitation Analytics may provide one or a combination of the following information:

-	UL average throughput per S-NSSAI for a UE, group of UEs, or any UE over a serving UPF.

-	DL average throughput per S-NSSAI for a UE, group of UEs, or any UE over a serving UPF.

The service consumer may be an NF (e.g. PCF).

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics ID set to "Slice Data Rate limitation Analytics";

-	The Target of Analytics Reporting: one or more SUPI(s) or Internal Group Identifier(s), or "any UE";

-	Analytics Filter Information optionally containing:

-	Area of Interest;

-	S-NSSAI;

-	DNN.

-	An Analytics target period indicates the time period requested.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

-	Optionally, Reporting Thresholds, which apply only for subscriptions and indicate conditions on the level to be reached for respective analytics subsets (see clause 6.X.3) in order to be notified by the NWDAF.

6.X.2	Input Data

NWDAF determine the analytics for the UL/DL throughput per S-NSSAI based on the following input data, collected from the UPF are defined in table 6.X.2-1:

Table 6.X.2-1: Data Collected from the UPF

| Information | Source | Description |
| --- | --- | --- |
| S-NSSAI | UPF | Identifies the Network Slice for which average throughput is retrieved. |
| Measurement period | UPF | Measurement period, the measurement is period. |
| Throughput UL/DL | UPF | Average Throughput UL/DL over the measurement period. |
| Timestamp | UPF | Time when measurements are taken. |

NOTE 1:	In the case of multiple UPFs are associated to an S-NSSAI, the NWDAF collects the data from all associated UPFs.

6.X.3	Output Analytics

The NWDAF generates Slice Data Rate limitation analytics. Depending on the Analytics Measurement period, the output detailed information provided by the NWDAF is defined in Table 6.X.3-1:

Table 6.X.3-1: Slice Data Rate limitation statistics

| Information | Description |
| --- | --- |
| S-NSSAI | Identifies the Network Slice for which analytics on average throughput are provided |
| Throughput UL/DL | Average Throughput UL/DL over the time window |
| Applicable time window | The time period that the analytics applies to |
| Confidence | Confidence on the prediction (application for predictions only) |

6.X.4	Procedures to request Slice Data Rate limitation Analytics

Figure 6.X.4-1: Procedure for NWDAF providing Slice Data Rate limitation Analytics

The procedure illustrated in figure 6.X.4-1 allows an analytics consumer to request Analytics ID "Slice Data Rate limitation" for a particular Application. The analytics consumer includes an S-NSSAI for which Slice Data Rate limitation is requested. The consumer indicates the Target of Analytics Reporting (e.g. "any UE").

1.	Analytics consumer sends an Analytics request/subscribe (Analytics ID = Slice Data Rate limitation Target of Analytics Reporting, Analytics Filter information = (Analytics target period, S-NSSAI, Area of Interest, UPF anchor ID) to NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe service.

2.	NWDAF subscribes to the network data from 5GC NF(s) (e.g. UPF) by invoking Nnf_EventExposure_Subscribe service.

2b.	The NWDAF generates Slice Data Rate limitation analytics, including the Average Throughput UL/DL over the time window.

3.	NWDAF provides the data analytics, to the analytics consumer by means of either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

NOTE 1:	For simplicity, the call flow only shows a request-response model for the interaction of NWDAF and analytics consumer instead of both request-response model and subscription-notification model.

If the analytics consumer is a PCF, PCF consumes the average throughput as input to determine the Slice-MBR for each UE.

----------------------------------------End of Changes--------------------------------------------