3GPP TSG-SA WG2 Meeting #145E 	S2-2104392

Elbonia, 17 - 28 May, 2021

Source:	Huawei, HiSilicon

Title:	Clarification of the Service Announcement

Document for:	Agreement

Agenda Item: 	8.9

Work Item / Release:	5MBS/Rel-17

Abstract of the contribution: This contribution clarify the EN in the service announcement.

- Discussion

There is an FFS on whether DNN and S-NSSAI in service announcement is required is FFS.

The UE sends MBS JOIN to SMF via an existing PDU Session. Thus the UE needs to select one dedicated PDU Session to trigger the MBS JOIN procedure. The UE cannot send MBS JOIN via any PDU Session. The Source Specific IP Multicast Address is optional IE, i.e. it may be unknown to the UE before UE join the 5MBS session.  So even from the URSP UE may not be able to derive which PDU session is to be used.

So the S-NSSAI and DNN shall be mandatory in the service announcement. It is proposed to remove the related FFS, and clarify this part.

- Proposal

It is proposed to capture the following changes vs. TS23.247.

**************************** 1st Change ***************************

6.11	Service Announcement

Service Announcement provides the UE with descriptions specifying the multicast or broadcast services to be delivered as part of MBS Session.

The Service Announcement includes the MBS Session ID(s), which is represented by TMGI or a Source Specific IP Multicast Address, for the service. When the MBS Session ID is Source Specific IP Multicast Address, the Service Announcement includes the PLMN ID of the PLMN in which the service is delivered.

The Service Announcement may include an MBS Session Type, which indicates whether the MBS Session for the service is multicast or broadcast.

For local MBS service, the Service Announcement may include the MBS service area information.

If the MBS Session is multicast, the Service Announcement includes the DNN and S-NSSAI of the PDU Session that the UE uses to join the MBS Session for the service via the PLMN.

The Service Announcement may be provided to UE by AF or MBSF.

NOTE:	For multicast, AF or MBSF provides Service Announcement only after the MBS information is available to 5GC or the start time need be included, to avoid potential rejection sent by SMF of the MBS session join request.

Editor’s Note: 	Other entities that can send Service Announcement to UE is FFS.

Editor's note:	The details of Service Announcement will be defined with coordination with SA4/SA6, including which information is aware by UE.

********************************* End Change ****************************