SA WG2 Meeting #145E	S2-2104416

17 – 28 May, 2021, Electronic meeting

Source:	Samsung

Title:	Update on MBS Session Establishment for Broadcast

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract: This document updates the MBS Session Establishment for Broadcast procedure to use common procedure.

- Introduction

This document modifies the MBS Session Establishment for Broadcast procedure to use common procedure.

- Proposal

It is proposed to capture the following changes in TS23.247.

* * * * Start of 1st Change * * * *

7.3.1	MBS Session Establishment for Broadcast

The broadcast MBS Session Establishment follows initial MBS Session Configuration(e.g. TMGI Allocation and MBS Session Start) so that resource for shared MBS delivery can be reserved. It is possible for AF to allocate TMGI once but start&stop MBS Session for multiple times

AF obtains the MBS Session ID (i.e. TMGI) and triggers to start an MBS session during initial MBS Session configuration and performs service announcement towards UEs.

And, based on service type(i.e. broadcast service) from AF, MB-SMF triggers the broadcast MBS session establishment, so that resources for the MBS Session are set up in the MB-UPF and in the NG-RAN for shared MBS delivery.

NOTE 1:	When the multicast transport between NG-RAN and MB-UPF is described below, source specific multicasting is assumed. That is, the parameter "LL MC address" is assumed to be accompanied by a "Source host address" parameter in the descriptions below.

To receive the data of broadcast communication service, the UE is either preconfigured with needed configuration (e.g. USD) for the UE to receive MBS service, or provisioned with the configuration of broadcast session on application level (service announcement; the configuration may for instance be performed using SIP signalling, or methods described in TS 26.346 [13]). If the needed configuration is pre-configured, the UE does not need to interact with network.

Figure 7.3.1-1: MBS Session Establishment for Broadcast

Editor´s note:	MBSTF could optionally be in the user plane and the call flow should be updated to also cover that possibility.

Editor's note:	The services and messages used in this procedure are FFS.

0.  Initial MBS session configuration(service type = broadcast services) is triggered from AF, which consists of TMGI allocation and Session start.

1.	For broadcast service the MB-SMF may use NRF to discover the AMF(s) based on the MBS service area and select the appropriate one(s). Then the MB-SMF sends the MBS Session establishment Request (TMGI, LL MC Address and source host address, 5G Authorized QoS Profile, MBS service area) messages to the selected AMF(s) in parallel if the service type is broadcast service.

Editor´s note:	Providing address information at this stage is FFS. Reusing the multicast signalling from NG-RAN to MB-SMF to set up shared delivery could be an alternative.

2.	The AMF sends an MBS Session Resource Setup Request (TMGI, LL MC and source host address, 5G Authorized QoS Profile) message to all NG-RANs in the MBS service area. The AMF may include the MBS service area.

3.	NG-RAN creates an MBS Broadcast Session Context (if it not already exists), stores the TMGI, the QoS Profile in the MBS Session Context. The LL MC Address and Source Host Address are optional parameters and only provided by MB-SMF to NG-RAN if MB-N3 multicast transport is configured to be used in the 5GC.

Editor´s note:	It also requires further study if AMF IDs need to be stored.

Editor´s note:	Steps 15 to 17 are FFS. Are several MBS session start responses for different NG RAN nodes permissible in step 17 or is information from several RAN nodes combined in one response? Reusing the multicast signalling to set up shared delivery could be an alternative.

4.	If NG-RAN prefers to use MB-N3 multicast transport (and if LL MC Address is available in NG-RAN), the NG-RAN joins the multicast group (i.e. LL MC Address).

If NG-RAN prefers to use MB-N3 point-to-point transport (or if the LL MC Address is not available in NG-RAN) between the NG-RAN and MB-UPF, NG-RAN provides its MB-N3 DL Tunnel Info.

5.	The NG-RAN reports successful establishment of the MBS Session resources (which may include multiple MBS QoS Flows) by sending MBS Session Resource Setup Response (TMGI, MB-N3 DL Tunnel Info) message(s) to the AMF. MB-N3 DL Tunnel Info is only available when point-to-point transport applies between MB-UPF and NG-RAN.

6.	The AMF sends MBS Session establishment Response (TMGI, MB-N3 DL Tunnel Info) to the MB-SMF. The AMF should respond success when it receives the first success response form the NG-RAN(s). And if all NG-RAN(s) report failure, the AMF should respond failure.

7.	If MB-N3 point-to-point transport is to be used (i.e. MB-N3 DL Tunnel Info is present in the MBS Session Start Response message from AMF), the MB-SMF sends a session update request (an N4 request) to the MB-UPF to allocate the MB-N3 point-to-point transport tunnel for a replicated MBS stream for the MBS Session. Otherwise, step 19 can be skipped.

8.	NG-RAN advertises the TMGI representing the MBS service over radio interface. Step 21 can take place in parallel with step 5.

9.	The AF starts transmitting the DL media stream to MB-UPF using the MB-N6 Tunnel Info, or optionally un-tunnelled i.e. as an IP multicast stream using the HL MC address.

10.	The MB-UPF transmits the media stream to NG-RAN via MB-N3 multicast transport or point-to-point transport.

11.	The NG-RAN transmits the received DL media stream using DL PTM resources.

* * * * End of Changes * * * *