SA WG2 Meeting #145E	S2-2104420

17 – 28 May, 2021, Electronic meeting

Source:	Samsung

Title:	Update on MBS Session release for Broadcast

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract: This document updates the MBS Session release for Broadcast procedure to use common procedure.

- Introduction

This document modifies the MBS Session release for Broadcast procedure to use common procedure.

- Proposal

It is proposed to capture the following changes in TS23.247.

* * * * Start of 1st Change * * * *

7.3.2	MBS Session Release for Broadcast

The MBS Session Release for broadcast follows removal of MBS Session Configuration(e.g. TMGI De-allocation and MBS Session Stop) so that resource for shared MBS delivery is released. If the dynamic PCC is not deployed, it is possible for AF to stop MBS Session but kept TMGI allocated. A combined step may be used to perform both removal of  MBS Session Configuration and Resource release for shared MBS delivery. If the dynamic PCC is deployed, the MBS Session is released when the MBS Policy Association is released, then the TMGI is also released.

The MBS Session Stop procedure is used to stop media delivery for an MBS session i.e. to all UEs in a group defined by a TMGI. The MBS session may start later again, which is triggered by AF. The resources in the NG-RAN and 5GC are however released and the MBS Session Contexts in 5GC and NG-RAN are deleted.

The TMGI De-allocation procedure is used to release TMGI allocated for AF. The removal of MBS Session Configuration procedure can be used no matter whether the MBS Session has stopped or not. If the MBS Session has not been stopped yet, the MBS session will be stopped prior to the TMGI de-allocation.

Figure 7.3.2-1: MBS Session Release for Broadcast

1.	The AF may stop the media stream before sending the Deactivate MBS Session Request (TMGI) message to the 3GPP network.

2.	The AF starts MBS session stop procedure by sending a Session Stop message(step 1 ~11 in the figure 7.1.1-2).

3.	MB-SMF sends an MBS Session Stop Request (TMGI) message to the AMF(s) that has been involved in the MBS Session.

Editor´s note:	It is FFS if the MB-SMF needs stores the AMFs or can select AMFs based on service area and forwards the service area to AMFs to avoid that AMF needs to maintain information about broadcast session.

4.	The AMF sends an MBS Session Resource Release Request (TMGI) message to all RAN nodes that have been involved. If a NG-RAN node receives multiple MBS Session Resource Release Request messages for the same TMGI (e.g. from several AMFs the NG-RAN is connected to), NG-RAN only performs step 5 and step 6 once.

Editor´s note:	It is FFS if the RAN nodes are stored in the AMFs or selected based on service area.

5.	The NG-RAN stops the PTM transmission.

6.	If MB-N3 multicast transport has been used, the NG-RAN sends a Leave message (LL MC Address) to stop the media stream to this NG-RAN node. If MB-N3 point-to-point transport has been used, the NG-RAN release its MB-N3 Tunnel Info. NG-RAN deletes its MBS Session Context.

7.	The NG-RAN reports successful release of resources for the MBS Session by sending MBS Session Resource Release Response (TMGI) message(s) to the AMF(s).

8.	The AMF sends MBS Session Resource Release Response (TMGI) to the MB-SMF.

9.	The AF starts a TMGI de-allocation procedure(step 12~15 in the figure 7.1.1-2) .

* * * * End of Changes * * * *