3GPP TSG-WG SA2 Meeting #145E e-meeting  	S2-2104540

# Elbonia, May 17 – 28, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2941 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for selection of AUSF in CH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For Credentials Holder using AUSF and UDM for primary authentication and authorization, the description about selection of AUSF in CH is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If the AMF determines that the primary authentication is performed by AUSF and UDM in CH based on SUCI provided by UE, it performs AUSF discovery and selection as specified in clause 6.3.4. The SUPI is used to identify the UE during primary authentication and authorization towards the AUSF in CH. SUPI privacy is achieved according to existing methods in Annex I.5 of TS 33.501 [29]. After completing the primary authentication with the AUSF in CH successfully, the AMF and SMF shall retrieve the UE subscription data from UDM in CH using SUPI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear description and unclear selection of AUSF in CH. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.30.2.9.3	Credentials Holder using AUSF and UDM for primary authentication and authorization

Figure 5.30.2.9.3-1 depicts the 5G System architecture for SNPN with Credentials Holder using AUSF and UDM for primary authentication and authorization. Clarification on the primary authentication and authorization using AUSF and UDM in Credentials Holder is listed as below:

-	If the AMF determines that the primary authentication is performed by AUSF and UDM in CH based on SUCI provided by UE, it performs AUSF discovery and selection as specified in clause 6.3.4.

-	The SUPI is used to identify the UE during primary authentication and authorization towards the AUSF in CH. SUPI privacy is achieved according to existing methods in Annex I.5 of TS 33.501 [29].

-	After completing the primary authentication with the AUSF in CH successfully, the AMF and SMF shall retrieve the UE subscription data from UDM in CH using SUPI if AMF and SMF does not have subscription data for the UE.

NOTE:	The architecture for SNPN and Credentials Holder is depicted as a non-roaming reference architecture as the UE is not considered to be roaming, even though some of the roaming architecture reference points are also used.

Figure 5.30.2.9.3-1: 5G System architecture with access to SNPN using credentials from Credentials Holder using AUSF and UDM

# * * * * End of changes * * * *