SA WG2 Meeting #S2-145E	S2-2104617

# 17 – 28 May, 2021, Electronic

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2848 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates of Processing AF requests to influence AM policies procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_DCAMP |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Since in TS 23.503 R15 specifications, there is a sentence that : The BSF may be deployed standalone or may be collocated with other network functions, such as PCF, UDR, NRF, SMF. So when the BSF is deployed distributedly maybe collocate with SMF, the number of BSFs is much larger than centralized deployment. In DCAMP WID, the approved solution for scenario when UPF/SMF detect the application start/stop, introduces PDU session related BSF in the solution, which functionality has been defined in R15. In this solution (paper S2-2102060,clause 4.15.6.9.3) the SM-BSF(which is related with PDU session) should support the subscription from AM-PCF based on UE SUPI and (DNN, S-NSSAI). But the existing BSF,i.e.R15 BSF, is managed and deployed based on IP address range, it is impossible for AM-PCF find the SM-BSF based on SUPI. But based on DNN and S-NSSAI can not reduce the searching scope in a large scale because the operators may use some default (DNN+S-NSSAI)s to serve general UEs and in this case there are still multiple BSFs in this DNN+S-NSSAI. In general, under the 3GPP specification allowed deployment choice (i.e. BSF distributed deployed), it is impossible to use UE SUPI+DNN+S-NSSAI to find one BSF or less numbers of BSF in some cases, to support AM-PCF subscription (i.e.step 6a in S2-2102060 4.15.6.9.3). So we propose to enhance the solution with the following update proposal: Method 1: SM-PCF finds the AM-PCF through AM-BSF. This AM-BSF refers to the BSF which support AM-PCF registration. And the AM-BSF can be discovered through NRF based on UE SUPI/GPSI (introduced in DCAMP), which is different from SM-BSF (which can be discovered through NRF based on IP address range) which have been defined since R15. In this method, first step: the SM-PCF should get the UE’s AM policy requirement from UDR, which is already supported by today’s interaction between SM-PCF and UDR(this is because there is only one NF type for PCF, so if the AM-PCF can retrieve the UE’s AM policy, the SM-PCF can also get the information). Second step: the SM-PCF should discover the AM-PCF through the AM-BSF. Third step: the SM-PCF should start the application detection and report/notify the result to AM-PCF. Method 2: SM-PCF finds the AM-PCF through NRF In this method, first step is the same as the method 1. Second step: the SM-PCF discovers the AM-PCF based on UE SUPI, DNN+S-NSSAI and finds the AM-PCF set from NRF. Third step: the SM-PCF should start the application detection and report/notify the result to AM-PCF set. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update procedure of supporting SM-PCF influence AM policies, PCF for the PDU Session takes the initiative to find the PCF for UE by requiring BSF or NRF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The solution which based on SM-BSF subscription can not be used when SM-BSF has been distributed deployed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<< First change >>>

4.15.6.9.3	Processing AF requests to influence AM policies

With this procedure, the AF can provide its AM Policy related request (for one or multiple UEs) at any time.

Figure 4.15.6.9.3: Handling an AF request to influence AM Policy

This procedure concerns only non-roaming scenarios, i.e. to cases where the involved entities serving the UE (i.e. AF, NEF, PCF, BSF, UDR, AMF) belong to the home PLMN, or the AF belongs to a third party with which the home PLMN has an agreement.

The PCF for the UE and the PCF for the PDU Session can be the same entity, then steps 6a, 6b, 7a and 7c are not performed, while step 7b is performed by that entity.

1.	AM Policy Association establishment as described in clause 4.16.1.

2. The PCF for the UE subscribes to policy data related to AM influence (Data Set = Application Data; Data Subset = AM influence information, Data Key = S-NSSAI and DNN and/or Internal Group Identifier or SUPI).

3a.	To create a new request, the AF provides "AM influence information" data to the NEF using the Nnef_AMInfluence_Create service operation (together with the AF identifier and potentially further inputs as specified in clause 5.2.6.23.2), including a target (one UE identified by SUPI or GPSI, a group of UEs identified by an External Group Identifier, or all UEs, noting that the "all UEs" option is applicable only if an Application ID is also provided), an optional indication of target application traffic (DNN, S-NSSAI and optionally an Application ID) and requirements related to AM policy (e.g. service coverage requirements, throughput requirements). The request contains also an AF Transaction Id and may contain a timer on how long this policy shall last, in which case the system behaviour upon expiration of this timer is as specified in TS 23.503. If with this request the AF subscribes to events related with access and mobility management policies, the AF indicates also where it desires to receive the corresponding notifications. To update or remove an existing request, the AF invokes an Nnef_AMInfluence_Update or Nnef_AMInfluence_Delete service operation providing the corresponding AF Transaction Id.

3b.	The NEF stores, updates, or removes the policy data of step 3a in the UDR, having translated any External Group Identifier to an Internal Group Identifier and any GPSI to a SUPI.

3c.	The UDR informs the NEF about the result of the operation of step 3b.

3d.	The NEF informs the AF about the result of the Nnef_AMInfluence operation performed in step 3a.

NOTE 1:	Steps 1, 2 and 3 can occur in any order.

4.	The UDR notifies the PCF(s) that have a matching subscription (from step 2) about the data stored, updated, or removed in step 3. If matching entries already existed in the UDR when step 3b is performed, this shall be immediately reported to the PCF. The PCF may check that an SM Policy association is established for the SUPI, DNN,S-NSSAI then subscribe to the SMF to Policy Control Request Trigger to detect the application traffic that triggers the allocation of a service area coverage or an allocation of RFSP index value, then steps 8 follows.

5.	A PDU session may be established by the UE as described in clause 4.3.2, including the registration of the PCF for the PDU Session to the BSF as the PCF that manages this PDU Session providing as inputs the UE SUPI/GPSI, the UE address and the DNN, S-NSSAI. The PCF for the PDU Session will get the AM influence information information from UDR via Nudr_DM_Notify service.

NOTE 2:	Step 5 can occur any time after step 1 and before step 6b.

6a.	The PCF for the UE may search the PCF for the PDU Session using Nbsf_Management_Subscribe with SUPI and (DNN, S-NSSAI) as parameters.

6b.	The BSF provides to the PCF for the UE the identity of the PCF for the PDU Session and the UE address for the requested SUPI and (DNN, S-NSSAI) combination via an Nbsf_Management_Notify operation. If a matching entry already exists in the BSF when step 6a is performed, this shall be immediately reported to the PCF for the UE.

NOTE 3: When there are multiple BSFs for PDU session deployed distributedly, the PCF for the UE may need to subscribe all the BSFs for PDU session because the BSFs can not be selected by UE SUPI since the BSFs for PDU session is configured and managed by IP address range. In this case, step 6a, step 6b and 7a can be skipped and step 6c or 6d can be used. In Step 6c, the BSF support discovery by SUPI and (DNN,S-NSSAI) is used so this kind of new BSF can be deployed and managed independently from the BSFs for PDU session.

6c. The PCF for the PDU session may search the PCF for the UE using Nbsf_Management_Discovery with SUPI and (DNN, S-NSSAI) as parameters.

6d. The PCF for the PDU session may search the PCF for the UE using Nnrf_NFDiscovery_Request with SUPI and (DNN, S-NSSAI) as parameters.

7a.	The PCF for the UE may subscribe to the PCF for the PDU Session for the "application traffic start/stop" event (see TS 23.503 [20] clause 6.1.3.18), providing the UE address and the Application ID(s) and request notifications on start/stop of application traffic detected in a Npcf_PolicyAuthorization_Subscribe request.

7b.	Application traffic start/stop detection is performed as described in steps 6 and 7 of Figure 4.16.14.2-1.

7c.	The PCF for the PDU Session notifies the PCF or PCF set for the UE about the detected application traffic start/stop event using Npcf_PolicyAuthorization_Notify.

8.	The PCF takes a policy decision and then it may initiate an AM Policy Association Modification procedure initiated by the PCF for the UE may be performed as described in clause 4.16.2.2. If the AF has subscribed to access and mobility management related events. i.e. request for service area coverage outcome in step 3, then the PCF reports the event (i.e. outcome of the request for service area coverage) to the AF as described in clause 4.16.2.2.

<<< End of changes >>>