3GPP TSG-WG SA2 Meeting #145E (e-meeting) 	S2-2104618

# Elbonia, 17-28 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0365 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS23.288 KI#15 User consent for UE data collection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-05-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | New feature user consent for UE data collection standardisation based on the KI#15 (user consent for UE data collection) conclusions in TR 23.700-91: 8.15 Key Issue #15: User consent for UE data collection/analysis It is recommended for Key Issue #15 to use the concepts and procedures described in below as the bases for the normative work with further alignments based on the SA WG3 feedback: a) UDR (via UDM services) holds the user consent for user related data which is provisioned by MNO as a user subscription information. b) A service provider (external to MNO domain) may use NEF parameterProvision_Update service to update or to revoke the user consent to the UDM/ UDR (when applicable). c) Alternatively, the user consent parameter may be configured by MNO in the UDM/UDR (when applicable). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The user consent for UE data collection is added to TS23.288 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The user consent for UE data collection will not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.x (new), 6.2.x.1(new), 6.2.x.2(new), 6.2.x.3(new). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.502 CR 2847 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * start of 1st change * * * *

### 6.2.x	User consent for UE data collection

#### 6.2.x.1	General

Based on local policy or regulations a user consent may be required before the data collection for some or all of the user data, which includes UE level data from UE or application or NFs.

It is network operator's responsibility to collect and manage the user consent information which is stored in the UDR as subscription information.

NOTE: 	Which data and data processing require prior user consent is not in the scope of 3GPP specification, due to the multitude of different legislations and regulatory environments in which 3GPP systems are being deployed, each with their own set of privacy and teleservices regulations.

The user consent may be updated or revoked at any time. If the user consent is updated/revoked, the NWDAF may, based on operator policies and legal requirements, stop data collection and delete any collected data related to the specific UE.

* * * * end of 1st change * * * *

* * * * start of 2nd change * * * *

#### 6.2.x.2	Content of user consent information for data collection

The following table illustrates the content of the user consent information for data collection.

Table 6.2.x.2-1: User consent information for data collection

| User consent information for data collection | Description |
| --- | --- |
| User consent for UE data collection | Identifies whether user consent is given or not. Applicable per specific data per UE. |

* * * * end of 2nd change * * * *

* * * * start of 3rd change * * * *

#### 6.2.x.3	Procedure for provision of the User consent for UE data collection

AF

1). Nnef_ParameterProvision_Update request

(UE external Id, user consent for UE data collection)

UDM

3). Nudm_ParameterProvision_Update request

(UE Id, user consent for UE data collection)

NEF

2). AF authorisation

5). Nudm_ParameterProvision_Update response

6). Nnef_ParameterProvision_Update response

4). The user consent update in the UDM

Figure 6.2.x.3-1 Procedure for provision of the User consent for UE data collection

1).	The Service Provider (AF) may collect the user consent information for data collection (if applicable) for some or all of UE level data from its users and the AF may provide this information via the NEF with the UE external Id for which the user consent applies using Nnef_ParameterProvision_Update request (UE external Id, user consent information for data collection), see TS23.502, clause 5.2.6.4.2. The Service Provider may also update or revoke the user consent.

2).	The NEF checks whether the AF is authorized for user consent provision or update.

3).	The NEF may inquire with the UDM to translate UE external Id to a 3GPP internal identity. Then the NEF forwards the request from the AF to the UDM via Nudm_ParameterProvision_Update request (UE Id, user consent for data collection), see TS23.502, clause 5.2.3.6.2.

4).	The UDM updates the User consent for UE data collection in the subscription information for the UE. The User consent for UE data collection information may also be provisioned by the network operator.

5). The UDM responds to the request with Nudm_ParameterProvision_Update response. If the procedure failed, the cause value indicates the reason.

6.	The NEF forwards the responds to the AF with Nnef_ParameterProvision_Update response.

* * * * end of 3rd change * * * *