3GPP TSG-SA WG2 Meeting #145-e (e-meeting)	S2-2104728

# 17-28 May, 2021, Elbonia 	(revision of )

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2870 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of UTRAN UE radio capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS, 5G_SRVCC |  |  |  |  |  | Date: |  |  | 2021-05-24 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The Nucmf_Provisioning_Create service in TS 23.502 should be aligned with CR 2570 to TS 23.501 whose reason for change was: In LS R2-2011164 RAN2 indicated: “To avoid START values de-synchronization and the key replay, RAN2 agreed that o The gNB does not upload the UE UTRA-FDD capabilities to the AMF; o Prior to initiating mobility the gNB always retrieves the latest capabilities from the UE. I.e. would not use UTRA capabilities received from CN, if provided.” The impact of the above agreement in TS 23.501 is that the UE radio capability information stored in the AMF (as defined in clause 5.4.4.1) does not contain UMTS FDD capabilities In addition based on the question in R2-2011164 it needs to be clarified that based on the above even for RACS manufacturer assigned UE Radio Capability ID it does not add value to provision in UCMF UMTS FDD radio capabilities for manufacturer assigned UE radio capability ID |  |  |  |  |  |  |  |  |
| Summary of change: |  | - UMTS FDD radio capabilities are not provisioned in UCMF for manufacturer assigned UE Radio Capability |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Signalling increase and misalignment between RAN and SA2 specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.18.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### 5.2.18	UCMF Services

#### 5.2.18.1	General

The following table illustrates the UCMF Services.

Table 5.2.18.1-1: NF services provided by UCMF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nucmf_Provisioning | Create | Request/Response | AF, NEF |
|  | Delete | Request/Response | AF, NEF |
|  | Update | Request/Response | AF, NEF |
| Nucmf_UECapabilityManagement | Resolve | Request/Response | AMF |
|  | Assign | Request/Response | AMF |
|  | Subscribe | Subscribe/Notify | AMF |
|  | Unsubscribe | Subscribe/Notify | AMF |
|  | Notify | Subscribe/Notify | AMF |

#### 5.2.18.2	Nucmf_Provisioning service

##### 5.2.18.2.1	Nucmf_Provisioning_Create service operation

Service operation name: Nucmf_Provisioning_Create

Description: The consumer creates a UCMF dictionary entry (or more entries) for a Manufacturer-assigned UE Radio Capability ID(s). The UCMF dictionary entry shall not contain UTRAN radio capabilities. For each UE Radio Capability ID the following inputs are provided:

a)	a UE radio access capability set with respective coding format or the UE radio access capability set in both TS 36.331 [16] and TS 38.331 [12] coding formats; and

b)	the related UE model(s) IMEI/TAC value(s) the UE radio capability ID applies to.

Inputs, Required: (list of) [UE Radio Capability ID(s), set(s) of UE radio access capability set and respective Coding format(s), (list of) IMEI/TAC value(s)].

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

The Coding format(s) indicates the format of the respective UE radio access capabilities as defined in TS 36.331 [16] or TS 38.331 [12].

*** End of Changes ****