SA WG2 Meeting #145e	S2-2104746

# 17-28 May 2021                                                             		 	Revision of S2-210

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0374 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure for Historical Data and Analytics Storage via Notifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For an ADRF to receive notifications from a NWDAF, MFAF or DCCF in the same manner as any other consuming NF (as described above): - ADRF service operations are needed so the DCCF or NWDAF can request that the ADRF initiate a subscription request, and so the DCCF or NWDAF can request that the ADRF terminate (remove) a current subscription. - The ADRF needs to use a DCCF or NWDAF service to send a subscription request containing a Notification Correlation ID and destination address. - The ADRF needs to store subsequent notifications. - The ADRF is allowed as an option to “fetch” data from the DCCF/MFAF/NWDAF when the ADRF is notified the data is available.. Similarly in adopted procedures and text, the ADRF is treated as any other data source for data retreival, and hence subscription service operations are are defined to obtain data from an ADRF. Note the above methodology for both data storage and data retreival from an ADRF are reflected in the 8 adopted procedures in 23.288 clause 6.2.6.3 for data collection and 6.1.4 for analytics exposure. Procedure for DCCF or NWDAF to request that an ADRF subscribe to receive data is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A procedure is added to clearly show how the DCCF or NWDAF can request that an ADRF subscribe to receive data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 23.288 procedures involving ADRF are inconsistent with the ADRF description and services operations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (NEW) 6.x.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.288 CR 0370, TS 23.288 CR 0306 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE (ALL NEW TEXT)

### 6.x.3	Historical Data and Analytics Storage via Notifications

The procedure depicted in figure 6.x.3-1 is used by consumers (NWDAF, DCCF) to store received notifications in the ADRF. The consumer requests the ADRF to initiate a subscription for data or analytics. Data or analytics provided in notifications as a result of the subsequent subscription by the ADRF are stored in the ADRF.

Figure 6.x.3-1: Historical Data and Analytics Storage via Notifications

1a-d.  Based on provisioning or based on reception of a DataManagement subscription request (e.g. see clause 6.2.6.3.2), the DCCF or the NWDAF determines that notifications are to be stored in an ADRF.

2a-b.	The DCCF or the NWDAF determines the ADRF where data or analytics needs to be stored, and requests that the ADRF subscribes to receive notifications. The determination may be made based on configuration or information supplied by the data consumer as described in clauses 6.1.4 and 6.2.6.3. The request to the ADRF specifies the data or analytics to which the ADRF will subscribe.

3a-b ADRF subscribes to the DCCF or the NWDAF to receive notifications, providing its notification endpoint address and a notification correlation ID.

4.	The DCCF, the MFAF or the NWDAF send Analytics or Data notifications containing the notification correlation ID provided by the ADRF to ADRF notification endpoint address. The ADRF stores the notifications.

5a-b.	The DCCF or the NWDAF determines that notifications no longer need to be stored in the ADRF.

6a-b.	 The DCCF or the NWDAF requests that the ADRF unsubscribes to receive notifications.

7a-b.	The ADRF sends a request to the DCCF or the NWDAF to unsubscribe to data notifications.

The NWDAF may interact with the Data Source, and the DCCF may interact with the Data Source and/or MFAF. Delivery notifications from the DCCF/MFAF or NWDAF to the ADRF are subsequently halted.

END OF CHANGES