3GPP TSG-WG SA2 Meeting #145E e-meeting 	S2-2104763

Elbonia, May 17 – 28, 2021

Source:	Nokia, Nokia Shanghai-Bell

Title:	Corrections to Clause 7.2.1.3 to address PCF interactions

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract: This contribution updates clause Clause 7.2.1.3 to add PCF interactions as explained in S2-2103838.

# Introduction

Problem:

At the time when the UE joins, QoS flows associated to the multicast session will be added by the SMF to the unicast PDU session and marked as being associated with the multicast session in preparation to a fallback to individual delivery.

The SMF obtains the information about the QoS flows for multicast from the MB-SMF.

Other QoS flows are determined by the SMF based on Policy Rules supplied by the PCF

QoS flows contain QoS rules with Packet filters and priorities and QFIs that need to be harmonized and decided upon based on policies. Associated QoS flows should be kept separate from QoS flows where radio resources are always reserved. Thus, special 5QIs not used for the PDU session may need to be allocated.

Total available bandwidth also needs to be considered. Policy control should be able decide which services the user can obtain simultaneously

Proposal:

SMF informs PCF that UE joined a multicast session and provides QoS flows obtained from MB-SMF for that multicast session

The PCF determines Policy Rules from the QoS flow information for the multicast session and selects 5QIs and priorities taking into account other installed PCC rules.

PCF provides Policy rules for associated QoS flows that contain an indication that those rules are for QoS flows associated with a multicast session

# 2. Text Proposal

It is proposed to agree the following changes for TR 23.757.

#### 7.2.1.3	MBS join and Session establishment procedure

The following steps are executed before the UE requests to join the MBS session:

-	The MBS Session has been configured.

-	The UE registers in the PLMN and establishes a PDU session.

-	The UE has known at least the MBS Session ID of a multicast group that the UE can join, e.g. via announcement.

v

Figure 7.2.1.3-1: PDU Session modification for multicast

1.	To join the multicast group, the UE sends the PDU Session Modification Request (MBS Session ID). MBS Session ID indicates the multicast group that UE wants to join.

2.	Per the received MBS Session ID, the SMF recognize this is MBS Session join request. The SMF authorizes MBS Session join request, see clause 6.1.1.

3.	If SMF has no information about the multicast context for the indicated MBS Session, SMF checks at the NRF whether a multicast context for the indicated MBS Session exists in the system, by using Nnrf_NFDiscovery request (MBS Session ID). If a multicast context already exists in the NRF, the NRF responses with Nnrf_NFDiscovery response (MB-SMF ID).

Editor's note:	Whether SMF acting as the MB-SMF is needed and how it works if needed is FFS.

Editor's note:	More consideration on how to prevent denial of service attack type situation when first UE joining the multicast group, triggers the MB-UPF to join the multicast tree towards the content provider is FFS.

4.	By using Nsmf_MBSSession_Create request (MBS Session ID), SMF interacts with MB SMF to retrieve multicast QoS flow information of the indicated MBS session.

5.	The SMF informs the PCF that the UE joined the multicast session and provides information about the QoS flows for the multicast session.

6.	The PCF determines PCC rules corresponding to those QoS flows. To avoid the QoS flows associated to the multicast session are bound to the same QFI as unrelated flows, the PCF may assign special 5QIs to related PCC rules or update other PCC rules. The PCF indicates in the PCC rules that they relate to the multicast session.

7.	SMF responds to AMF through Nsmf_PDUSession_UpdateSMContext response(N2 SM information (PDU Session ID, MBS Session ID, MB-SMF ID, multicast QoS flow information, updated PDU Session information, mapping between unicast QoS flow and multicast QoS flow information), N1 SM container (PDU Session Modification Command) to:

-	create a MBS session context for the indicated MBS session in the RAN, if it does not exist already; and

-	inform about the relation including the mapping information between the multicast context and the UE's PDU session to RAN.

Based on operator policy, the SMF may prepare for individual delivery fall-back. The SMF maps the received QoS information of the multicast QoS Flow into PDU Session's QoS Flow information, and includes the information of the QoS Flows and the mapping information about the QoS Flows in the SM information sent to RAN.

Editor's note:	Details information included in N2 SM information will be aligned with RAN WG3.

Editor's note:	Whether it needs to inform about the relation including the mapping information between the multicast context and the UE's PDU session to UE is FFS.

8.	The N2 message, which includes the PDU session modification command information is sent to the RAN.

If the MBS is not supported by NG-RAN, 5GC individual MBS traffic delivery may be used. Otherwise, 5GC shared MBS traffic delivery is adopted.

Editor's note:	How the NG-RAN's 5MBS capability is made known is FFS.

The NG-RAN uses the MBS Session ID to determine that the PDU Session Modification procedures corresponds to the indicated multicast session.

If the multicast QoS information is received, the associated unicast QoS flow information is not used to allocate the radio resource.

NOTE:	It is NG-RAN that decides whether radio resource is allocated or not.

When the NG-RAN receives an MBS Session ID but MBS Session context does not exist for that MBS Session ID, the NG-RAN use the included MBS Session QoS information to allocate resources to serve this multicast session. Otherwise the indicated MBS Session has been established before. The NG-RAN can use those allocated resource for MBS Session data packet transferring to UE.

9.	The RAN, AMF, SMF, and MB-SMF performs resources reservation for individual delivery and shared delivery.  The SMF obtains the 5MBS capability of target NG-RAN via the accepted QFI information and determines the delivery mode between 5GC and RAN.

Editor's note:	Details of establishing the tunnel of 5GC Shared MBS traffic delivery and 5GC Individual MBS traffic delivery is FFS.