3GPP TSG-SA WG2 Meeting #145E (e-meeting)	S2-2104768

17 - 28 May 2021, Elbonia	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0180 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Storage of UE Positioning Capabilities by 5GCN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-05-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As described in S2-2102420 at SA2#144E, some reduction of latency for positioning of a UE is possible by storing UE positioning capabilities in an AMF or LMF and using the stored UE positioning capabilities for subsequent location sessions with the UE. Three solutions were described and evaluated in S2-2102420. Of these, Solution C is provided here for support in Release 17. |  |  |  |  |  |  |  |  |
| Summary of change: |  | With Solution C, in S2-2102420, during a location session with an LMF, a UE provides its positioning capabilities to the LMF using existing LPP messages. A UE also includes an indication as to whether its positioning capabilities are non-variable, meaning that they are fixed and cannot be changed by the UE or by a user of the UE. If the positioning capabilities are indicated as non-variable, the LMF returns the UE positioning capabilities to the AMF along with any other results of the location session with the UE. The AMF then stores the positioning capabilities and provides the positioning capabilities to an LMF for any new location request for the UE. For periodic or triggered location of a UE, storage of UE positioning capabilities by an LMF is included, as the same LMF can support positioning of the UE for all periodic or triggered event reports sent later by the UE. In the case of transfer of a periodic or triggered location request to a new LMF, the old LMF can include the UE positioning capabilities in the UE location context transferred to the new LMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The latency of location services for a UE will typically include an extra delay to obtain UE positioning capabilities from the UE that might otherwise have been avoided. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.7, 4.3.8, 6.X (new), 6.X.1 (new), 6.X.2 (new), 8.3.2.2, 8.3.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### 4.3.7	Access and Mobility Management Function, AMF

The AMF contains functionality responsible for managing positioning for a target UE for all types of location request. The AMF is accessible to the GMLC and NEF via the Namf interface, to the RAN via the N2 reference point and to the UE via the N1 reference point.

Functions which may be performed by an AMF to support location services include the following.

-	Initiate an NI-LR location request for a UE with an IMS emergency call.

-	Receive and manage location requests from a GMLC for a 5GC-MT-LR and deferred 5GC-MT-LR for periodic, triggered and UE available location events.

-	Receive and manage location requests from a UE for a 5GC-MO-LR.

-	Receive and manage Event Exposure request for location information from an NEF.

-	Select an LMF.

-	Receive updated privacy requirements from a UE and transfer to a UDR via UDM.

-	Support cancelation of periodic or triggered location reporting for a target UE.

-	Support change of a serving LMF for periodic or triggered location reporting for a target UE.

-	When assistance data is broadcast by 5GS in ciphered form, the AMF receives ciphering keys from the LMF and forwards to suitably subscribed UEs using mobility management procedures.

-	Store UE positioning capabilities.

**** Next Change ****

### 4.3.8	Location Management Function, LMF

The LMF manages the overall co-ordination and scheduling of resources required for the location of a UE that is registered with or accessing 5GCN. It also calculates or verifies a final location and any velocity estimate and may estimate the achieved accuracy. The LMF receives location requests for a target UE from the serving AMF using the Nlmf interface. The LMF interacts with the UE in order to exchange location information applicable to UE assisted and UE based position methods and interacts with the NG-RAN, N3IWF or TNAN in order to obtain location information.

The LMF shall determine the result of the positioning in geographical co-ordinates as defined in TS 23.032 [8]. If requested and if available, the positioning result may also include the velocity of the UE.

Additional functions which may be performed by an LMF to support location services include the following.

-	Support a request for a single location received from a serving AMF for a target UE.

-	Support a request for periodic or triggered location received from a serving AMF for a target UE.

-	Determine position methods based on UE and PLMN capabilities, QoS, UE connectivity state per access type and LCS Client type.

-	Report UE location estimates directly to a GMLC for periodic or triggered location of a target UE.

-	Support cancelation of periodic or triggered location for a target UE.

-	Support the provision of broadcast assistance data to UEs via NG-RAN in ciphered or unciphered form and forward any ciphering keys to subscribed UEs via the AMF.

-	Support change of a serving LMF for periodic or triggered location reporting for a target UE.

-	Store UE positioning capabilities.

**** Next Change ****

## 6.X	Procedures for Storage of UE Positioning Capabilities

### 6.X.1	Storage of Positioning Capabilities in an AMF

The following procedure is used by an AMF to store UE positioning capabilities. The procedure enables some reduction in latency of UE positioning by avoiding the need to request and/or transfer UE positioning capabilities at an LPP level between a UE and LMF.

Figure 6.X.1-1: Storage of Positioning Capabilities in an AMF

1a.	Either: The UE sends an MO-LR Request message to the serving AMF as described for step 2 of clause 6.2. The MO-LR request may include one or more UL Positioning messages.

1b.	Or: Some entity in the 5GC (e.g. a GMLC) requests some location service (e.g. positioning) for the UE to the serving AMF.

1c.	Or: The serving AMF determines the need for some location service for the UE (e.g. to locate the UE for an emergency call).

2.	The AMF retrieves the UE positioning capabilities if stored previously in the AMF (according to step 6 for a previous location related request for the UE).

3.	The AMF selects a suitable LMF as described in clause 5.1 and  invokes the Nlmf_Location_DetermineLocation service operation towards the LMF to request location service appropriate to the location related request at step 1. The service operation includes a LCS Correlation identifier, the UE positioning capabilities if retrieved at step 2, one or more UL Positioning messages if received at step 1a and other parameters applicable to the location related request received at step 1.

4.	The LMF performs one or more of the positioning procedures described in clauses 6.11.1, 6.11.2 and 6.11.3 as follows.

4a.	If the LMF did not receive the UE positioning capabilities from the AMF at step 3 and if no UL Positioning message was included at step 3 containing the UE positioning capabilities, the LMF may send a DL Positioning message to the UE as described in clause 6.11.1 to request the UE positioning capabilities.

4b.	If a request was received at step 4a for the UE positioning capabilities, or if the UE decides to send its positioning capabilities unsolicited after received a DL positioning message from the LMF at step 4c, the UE sends an UL positioning message containing the UE positioning capabilities to the LMF as described in clause 6.11.1.

4c.	The UE and LMF may exchange other UL and DL Positioning messages as described in clause 6.11.1

4d.	The UE and NG-RAN may exchange Network positioning messages as described in clause 6.11.2 and clause 6.11.3.

4e.	Depending on the type of location service request at step 1, the LMF may determine a location for the UE.

5.	The LMF returns the Nlmf_Location_DetermineLocation Response towards the AMF. The service operation includes the LCS Correlation identifier and other parameters applicable to the location service request at step 1 such as a UE location if obtained at step 4e. If the LMF received the positioning capabilities of the UE at step 4b or as part of an UL positioning message transferred at step 3 and if the positioning capabilities include an indication that the positioning capabilities are non-variable, the LMF includes the positioning capabilities at step 5.

NOTE:	UE positioning capabilities are non-variable if the positioning capabilities are fixed and cannot be modified by an operator, by the UE or by a user of the UE. Positioning capabilities which can occasionally be extended but not reduced, e.g. by a software upgrade to an ME, can be considered as non-variable. To allow for occasional extension, storage of UE positioning capabilities might be limited in time, e.g. to a few days or few weeks.

6.	If the UE positioning capabilities are included at step 5, the AMF stores the UE positioning capabilities and overwrites any positioning capabilities that may already be stored for the UE.

7a.	If step 1a was performed, the AMF forwards the response from step 5 to the UE.

7b. If step 1b was performed, the AMF forwards the response to the 5GC LCS Entity.

7c.	If step 1c was performed, the AMF uses the response in step 5 to support the location service for step 1c.

### 6.X.2	Storage of Positioning Capabilities in an LMF

For a Deferred 5GC-MT-LR Procedure for Periodic or Triggered Location Events as described in clause 6.3.1, an LMF may store the UE positioning capabilities if obtained at the start of the positioning procedure. The UE positioning capabilities may be obtained by the LMF at the start of the procedure as described in Figure 6.X.1-1, where step 1b in Figure 6.X.1-1 is performed to start the procedure and corresponds to step 5 in Figure 6.3.1-1 and where steps 3, 4 and 5 in Figure 6.X.1-1 correspond to steps 14, 15 and 18, respectively, in Figure 6.3.1-1. The LMF may then obtain the UE positioning capabilities in two alternative ways: from the AMF at step 14 in Figure 6.3.1-1 or from the UE at step 15 in Figure 6.3.1-1. If the positioning capabilities include an indication that the positioning capabilities are non-variable, the LMF may store the positioning capabilities and use the positioning capabilities for each instance of step 27 in Figure 6.3.1-1 when positioning of the UE occurs.

**** Next Change ****

#### 8.3.2.2	Nlmf_Location_DetermineLocation service operation

Service operation name: Nlmf_Location_DetermineLocation

Description: Provides UE location information to the consumer NF.

NOTE:	For deferred location request, this service operation is used to implicitly subscribe to the notification of the UE location information.

Input, Required: Client Type, LCS Correlation Identifier.

Input, Optional: serving cell identifierof the Primary Cell in the Master RAN node and the Primary Cell in the Secondary RAN node when available based on Dual Connectivity scenarios if the UE is using 3GPP access, Location QoS, Supported GAD shapes, AMF identity if a UE associated Namf_Communication service is to be invoked by LMF, Type of request for a 5GC-MO-LR, Embedded LPP message for a 5GC-MO-LR, subscribed assistance data for a 5GC-MO-LR, Deferred location type, Deferred location parameters, indication if UE supports LPP or not, Notification Target Address, Notification Correlation ID, UE Positioning Capabilities.

Output, Required: Success/Failure indication

Output, Optional: Geodetic Location, Local Location including Coordinate ID, Civic Location, Position Methods Used (in the case of success indication provided), Serving LMF identification, Failure Cause (in the case of failure indication provided), UE Positioning Capabilities.

See clause 6.1, clause 6.2. and clause 6.3.1 for examples of usage of this service operation.

**** Next Change ****

#### 8.3.2.5	Nlmf_Location_LocationContextTransfer service operation

Service operation name: Nlmf_Location_LocationContextTransfer

Description: Transfers location context information for location event reporting for a target UE from the consumer NF.

Input, Required: AMF identity, Location QoS, Supported GAD shapes, Deferred location type, Deferred location parameters, Notification Target Address, Notification Correlation ID, Embedded event report message.

Input, Optional: Event reporting status, UE location information, UE Positioning Capabilities.

Output, Required: Success/Failure indication

Output, Optional: None.

See clause 6.4 for an example of usage of this service operation.

**** End of Changes ****