3GPP TSG-WG SA2 Meeting #145E e-meeting 	S2-2104926

Elbonia, May 17 – 28, 2021	(revision of S2-2104161)

Source:	China Mobile

Title:	Clarifications on one EN and one NOTE in the TS 23.256

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	ID_UAS / Rel-17

Abstract: This contribution introduces to make clarifications on one EN and one NOTE in the TS 23.256.

1. Introduction/Discussion

This contribution introduces to make clarifications on one EN and one Note in the TS 23.256.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.256.

# * * * * First change * * * *

5.3.1	UAV Tracking Model

3GPP network supports the functionality of UAV Tracking via the service exposure support towards USS. The USS invokes 3GPP network service through a UAS-NF for UAV tracking. The UAS-NF acts as an NEF and interacts with other network functions (e.g. GMLC and AMF/MME) to support UAV tracking. The USS shall use 3GPP UAV ID (e.g. GPSI) for identifying an individual target UAV. The USS does not include the 3GPP UAV ID for tracking unknown UAV. For further details on the architecture reference model, see clause 4.2.

Three UAV tracking modes are supported:

-	UAV location reporting mode,

-	UAV presence monitoring mode, and

-	Unknown UAV tracking mode.

USS/TPAE could at any time choose a UAV tracking mode and provide the corresponding request to UAS NF. The USS/TPAE logic on its choice of UAV Tracking Model is out of scope of 3GPP.

The 3GPP network may also provide the UAV location to the USS during the UUAA procedures, as described in clause 5.2.2 and clause 5.2.3.

# * * * * End of changes * * * *