3GPP TSG-SA WG2 Meeting #145-e (e-meeting)	S2-2105114

# 17-28 May 2021, Elbonia 	(revision of S2-2104638r01)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2962 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of applicability of port/bridge management information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clause 5.28.3.1 has the following Editor's note: Editor's note: It is FFS which of the information elements in the tables below are exchanged between NEF and DS-TT/NW-TT. Related to that, SA4 informed SA2 in S4-210619 that "typically professional audio and video applications already contain de-jitter buffers to accommodate time variability of inter-packet arrivals at the receiving device" and that for this reason "professional audio and video applications do not expect nor require the 5GS to de-jitter the traffic (e.g., by use of hold/forward buffering)." In conclusion, VIAPA scenarios do not require hold/forward buffer support from 5GS. It is also worth re-emphasizing the purpose of the hold and forward buffer functionality supported by 5GS, which is based on IEEE 802.1Q clause 8.6.8.4 (Enhancements for scheduled traffic) and which was introduced into 5GS to support scenarios when 5GS integrated into an IEEE 802.1Q network that supports scheduled traffic controlled by a CNC. The goal of scheduled traffic operation as defined in IEEE 802.1Q clause 8.6.8.4 is to guarantee a maximum end-to-end delay for TSN flows. This is achieved by avoiding that TSN frames have to be buffered because of other traffic blocking the egress queues of switches on the path from source to sink. In other words, the hold and forward functionality as defined by IEEE 802.1Q clause 8.6.8.4 is not to avoid that a frame would otherwise reach a receiver too early but solely to prevent traffic from blocking egress queues of intermediate switches on the path and by this, to guarantee the maximum end-to-end delay for TSN flows. However, if 5GS is not integrated into an IEEE 802.1 TSN network, then there is no need to artificially delay frames from being forwarded by holding them in a hold and forward buffer. In conclusion, there is no need for 5GS to support hold/forward functionality for scenarios other than when 5GS is integrated as a bridge into a managed IEEE 802.1 TSN network. In line with this, it is clear that NEF only needs to support the PMIC/BMIC information for time synchronization. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 5.28.3.1 Remove Editor's note and clarify which PMIC/BMIC information applies to TSN AF case (i.e. when 5GS is integrated into an IEEE 802.1 TSN network using scheduled traffic operation as defined in IEEE 802.1Q clause 8.6.8.4) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear applicability of port and bridge management information (TSN AF vs. NEF). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.28.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

#### 5.28.3.1	General

Port number of the DS-TT for the PDU Session is assigned by the UPF during PDU session establishment. The port number of the DS-TT port for a PDU Session shall be reported to the SMF from the UPF and further stored at the SMF. The SMF provides the DS-TT port number via PCF to the TSN AF or NEF. TSN AF or NEF maintains an association between the DS-TT port number and the MAC address (with Ethernet type PDU session) or IP address (with IP type PDU Session) of the UE. If a PDU session for which SMF has reported a DS-TT port number to TSN AF or NEF is released, then SMF informs TSN AF or NEF accordingly.

NOTE:	Port number can refer either to Ethernet port or PTP port. In Ethernet type PDU Sessions, it is assumed that the PTP port number is the same as the associated Ethernet port number.

5GS shall support transfer of standardized and deployment-specific port management information transparently between TSN AF or NEF and DS-TT or NW-TT, respectively inside a Port Management Information Container. NW-TT may support one or more ports. In this case, each port uses separate Port Management Information Container. 5GS shall also support transfer of standardized and deployment-specific bridge management information transparently between TSN AF or NEF and NW-TT, respectively inside a Bridge Management Information Container. Table 5.28.3.1-1 and Table 5.28.3.1-2 list standardized port management information and bridge management information, respectively.

If TSN AF is deployed, i.e. if 5GS is integrated with an IEEE TSN network, the port and bridge management information is exchanged between CNC and TSN AF. The port management information, is related to ports located in DS-TT or NW-TT.

If TSN AF is not deployed, the port and bridge management information is exchanged between NEF and DS-TT/NW-TT.

Table 5.28.3.1-1: Standardized port management information

| Port management information | Applicability (see NOTE 6) |  | Supported operations by TSN AF (see NOTE 1) | Supported operations by NEF (see NOTE 1) | Reference |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DS-TT | NW-TT |  |  |  |  |
| General |  |  |  |  |  |  |
| Port management capabilities (see NOTE 2) | X | X | R | R |  |  |
| Bridge delay related information |  |  |  |  |  |  |
| txPropagationDelay | X | X | R | ― | IEEE Std 802.1Qcc [95] clause 12.32.2.1 |  |
| Traffic class related information |  |  |  |  |  |  |
| Traffic class table | X | X | RW | ― | IEEE Std 802.1Q [98] clause 12.6.3 and clause 8.6.6. |  |
| Gate control information |  |  |  |  |  |  |
| GateEnabled | X | X | RW | ― | IEEE Std 802.1Q [98] Table 12-29 |  |
| AdminBaseTime | X | X | RW | ― | IEEE Std 802.1Q [98] Table 12-29 |  |
| AdminControlList | X | X | RW | ― | IEEE Std 802.1Q [98] Table 12-29 |  |
| AdminCycleTime (see NOTE 3) | X | X | RW | ― | IEEE Std 802.1Q [98] Table 12-29 |  |
| AdminControlListLength (see NOTE 3) | X | X | RW | ― | IEEE Std 802.1Q [98] Table 12-28 |  |
| Tick granularity | X | X | R | ― | IEEE Std 802.1Q [98] Table 12-29 |  |
| General Neighbor discovery configuration (NOTE 4) |  |  |  |  |  |  |
| adminStatus | D | X | RW | ― | IEEE Std 802.1AB [97] clause 9.2.5.1 |  |
| lldpV2LocChassisIdSubtype | D | X | RW | ― | IEEE Std 802.1AB [97] Table 11-2 |  |
| lldpV2LocChassisId | D | X | RW | ― | IEEE Std 802.1AB [97] Table 11-2 |  |
| lldpV2MessageTxInterval | D | X | RW | ― | IEEE Std 802.1AB [97] Table 11-2 |  |
| lldpV2MessageTxHoldMultiplier | D | X | RW | ― | IEEE Std 802.1AB [97] Table 11-2 |  |
| NW-TT port neighbor discovery configuration |  |  |  |  |  |  |
| lldpV2LocPortIdSubtype |  | X | RW | ― | IEEE Std 802.1AB [97] Table 11-2 |  |
| lldpV2LocPortId |  | X | RW | ― | IEEE Std 802.1AB [97] Table 11-2 |  |
| DS-TT port neighbor discovery configuration |  |  |  |  |  |  |
| lldpV2LocPortIdSubtype | D |  | RW | ― | IEEE Std 802.1AB [97] Table 11-2 |  |
| lldpV2LocPortId | D |  | RW | ― | IEEE Std 802.1AB [97] Table 11-2 |  |
| Neighbor discovery information for each discovered neighbor of NW-TT |  |  |  |  |  |  |
| lldpV2RemChassisIdSubtype |  | X | R | ― | IEEE Std 802.1AB [97] Table 11-2 |  |
| lldpV2RemChassisId |  | X | R | ― | IEEE Std 802.1AB [97] Table 11-2 |  |
| lldpV2RemPortIdSubtype |  | X | R | ― | IEEE Std 802.1AB [97] Table 11-2 |  |
| lldpV2RemPortId |  | X | R | ― | IEEE Std 802.1AB [97] Table 11-2 |  |
| TTL |  | X | R | ― | IEEE Std 802.1AB [97] clause 8.5.4 |  |
| Neighbor discovery information for each discovered neighbor of DS-TT (NOTE 5) |  |  |  |  |  |  |
| lldpV2RemChassisIdSubtype | D |  | R | ― | IEEE Std 802.1AB [97] Table 11-2 |  |
| lldpV2RemChassisId | D |  | R | ― | IEEE Std 802.1AB [97] Table 11-2 |  |
| lldpV2RemPortIdSubtype | D |  | R | ― | IEEE Std 802.1AB [97] Table 11-2 |  |
| lldpV2RemPortId | D |  | R | ― | IEEE Std 802.1AB [97] Table 11-2 |  |
| TTL | D |  | R | ― | IEEE Std 802.1AB [97] clause 8.5.4.1 |  |
| Stream Parameters (NOTE 11) |  |  |  |  |  |  |
| MaxStreamFilterInstances | X |  | R | ― | IEEE Std 802.1Q [98] clause 12.31.1.1 |  |
| MaxStreamGateInstances | X |  | R | ― | IEEE Std 802.1Q [98] clause 12.31.1.2 |  |
| MaxFlowMeterInstances | X |  | R | ― | IEEE Std 802.1Q [98] clause 12.31.1.3 |  |
| SupportedListMax | X |  | R | ― | IEEE Std 802.1Q [98] clause 12.31.1.4 |  |
| Per-Stream Filtering and Policing information (NOTE 10) |  |  |  |  |  |  |
| Stream Filter Instance Table (NOTE 8) |  |  |  | ― | IEEE Std 802.1Q [98] Table 12-32 |  |
| > StreamFilterInstanceIndex | X | X | RW | ― | IEEE Std 802.1Q [98] Table 12-32 |  |
| > Stream Identification type | X | X | RW | ― | IEEE 802.1CB [83] clause 9.1.1.6 |  |
| > Stream Identification Controlling Parameters | X | X | RW | ― | IEEE 802.1CB [83] clauses 9.1.2, 9.1.3, 9.1.4 (NOTE 12) |  |
| > PrioritySpec | X | X | RW | ― | IEEE Std 802.1Q [98] Table 12-32 |  |
| > StreamGateInstanceID | X | X | RW | ― | IEEE Std 802.1Q [98] Table 12-32 |  |
| Stream Gate Instance Table (NOTE 9) |  |  |  |  | IEEE Std 802.1Q [98] Table 12-33 |  |
| StreamGateInstanceIndex | X | X | R | ― | IEEE Std 802.1Q [98] Table 12-33 |  |
| PSFPAdminBaseTime | X | X | RW | ― | IEEE Std 802.1Q [98] Table 12-33 |  |
| PSFPAdminControlList | X | X | RW | ― | IEEE Std 802.1Q [98] Table 12-33 |  |
| PSFPAdminCycleTime | X | X | RW | ― | IEEE Std 802.1Q [98] Table 12-33 |  |
| PSFPTickGranularity | X | X | R | ― | IEEE Std 802.1Q [98] Table 12-33 |  |
| Time Synchronization Information |  |  |  |  |  |  |
| Supported PTP instance types (NOTE 13) | X |  | R | R | IEEE Std 1588-2019 [126] clause 8.2.1.5.5 |  |
| Supported transport types (NOTE 14) | X |  | R | R |  |  |
| Supported delay mechanisms (NOTE 15) | X |  | R | R | IEEE Std 1588-2019 [126] clause 8.2.15.4.4 |  |
| PTP grandmaster capable (NOTE 16) | X |  | R | R |  |  |
| gPTP grandmaster capable (NOTE 17) | X |  | R | R |  |  |
| Supported PTP profiles (NOTE 18) | X |  | R | R |  |  |
| Number of supported PTP instances | X |  | R | R |  |  |
| PTP Instance ID | X | X | RW | RW |  |  |
| > PTP profile (NOTE 19) | X | X | RW | RW |  |  |
| > Transport type (NOTE 20) | X | X | RW | RW |  |  |
| > Grandmaster enabled (NOTE 21) | X |  | RW | RW |  |  |
| IEEE Std 1588-2019 [126] data sets (NOTE 22) |  |  |  |  |  |  |
| > defaultDS.clockIdentity | X | X | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.1.2.2 |  |
| > defaultDS.clockQuality.clockClass | X | X | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.1.3.1.2 |  |
| > defaultDS.clockQuality.clockAccuracy | X | X | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.1.3.1.3 |  |
| > defaultDS.clockQuality.offsetScaledLogVariance | X | X | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.1.3.1.4 |  |
| > defaultDS.priority1 | X | X | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.1.4.1 |  |
| > defaultDS.priority2 | X | X | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.1.4.2 |  |
| > defaultDS.domainNumber | X | X | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.1.4.3 |  |
| > defaultDS.sdoId | X | X | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.1.4.5 |  |
| > defaultDS.instanceEnable | X | X | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.1.5.2 |  |
| > defaultDS.externalPortConfigurationEnabled |  | X | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.1.5.3 |  |
| > defaultDS.instanceType | X | X | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.1.5.5 |  |
| > portDS.portIdentity | X | X | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.15.2.1 |  |
| > portDS.portState | X | X | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.15.3.1 |  |
| > portDS.logMinDelayReqInterval | X | X | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.15.3.2 |  |
| > portDS.logAnnounceInterval | X | X | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.1 |  |
| > portDS.announceReceiptTimeout |  | X | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.2 |  |
| > portDS.logSyncInterval | X | X | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.3 |  |
| > portDS.delayMechanism | X | X | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.4 |  |
| > portDS.logMinPdelayReqInterval | X | X | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.5 |  |
| > portDS.versionNumber | X | X | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.6 |  |
| > portDS.minorVersionNumber | X | X | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.7 |  |
| > portDS.delayAssymetry | X | X | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.8 |  |
| > portDS.portEnable | X | X | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.15.5.1 |  |
| > timePropertiesDS.currentUtcOffset | X | X | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.4.2 |  |
| > timePropertiesDS.timeSource | X | X | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.4.9 |  |
| > externalPortConfigurationPortDS.desiredState |  | X | RW | RW | IEEE Std 1588-2019 [126] clause 15.5.3.7.15.1 |  |
| IEEE Std 802.1AS [104] data sets (NOTE 22) |  |  |  | RW |  |  |
| > defaultDS.clockIdentity | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.2.2 |  |
| > defaultDS.clockQuality.clockClass | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.2.4.2 |  |
| > defaultDS.clockQuality.clockAccuracy | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.2.4.3 |  |
| > defaultDS.clockQuality.offsetScaledLogVariance | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.2.4.4 |  |
| > defaultDS.priority1 | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.2.5 |  |
| > defaultDS.priority2 | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.2.6 |  |
| > defaultDS.timeSource | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.2.15 |  |
| > defaultDS.domainNumber | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.2.16 |  |
| > defaultDS.sdoId | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.2.4.3 |  |
| > defaultDS.externalPortConfigurationEnabled |  | X | RW | RW | IEEE Std 802.1AS [104] clause 14.2.18 |  |
| > defaultDS.instanceEnable | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.2.19 |  |
| > portDS.portIdentity | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.2 |  |
| > portDS.portState |  | X | R | R | IEEE Std 802.1AS [104] clause 14.8.3 |  |
| > portDS.ptpPortEnabled | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.4 |  |
| > portDS.delayMechanism | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.5 |  |
| > portDS.isMeasuringDelay | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.6 |  |
| > portDS.asCapable | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.7 |  |
| > portDS.meanLinkDelay | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.8 |  |
| > portDS.meanLinkDelayThresh | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.9 |  |
| > portDS.delayAssymetry | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.10 |  |
| > portDS.neighborRateRatio | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.11 |  |
| > portDS.initialLogAnnounceInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.12 |  |
| > portDS.currentLogAnnounceInterval | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.13 |  |
| > portDS.useMgtSettableLogAnnounceInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.14 |  |
| > portDS.mgtSettableLogAnnounceInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.15 |  |
| > portDS.announceReceiptTimeout |  | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.16 |  |
| > portDS.initialLogSyncInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.17 |  |
| > portDS.currentLogSyncInterval | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.18 |  |
| > portDS.useMgtSettableLogSyncInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.19 |  |
| > portDS.mgtSettableLogSyncInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.20 |  |
| > portDS.syncReceiptTimeout |  | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.21 |  |
| > portDS.syncReceiptTimeoutTimeInterval |  | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.22 |  |
| > portDS.initialLogPdelayReqInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.23 |  |
| > portDS.currentLogPdelayReqInterval | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.24 |  |
| > portDS.useMgtSettableLogPdelayReqInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.25 |  |
| > portDS.mgtSettableLogPdelayReqInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.26 |  |
| > portDS.initialLogGptpCapableMessageInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.27 |  |
| > portDS.currentLogGptpCapableMessageInterval | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.28 |  |
| > portDS.useMgtSettableLogGptpCapableMessageInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.29 |  |
| > portDS.mgtSettableLogGptpCapableMessageInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.30 |  |
| > portDS.initialComputeNeighborRateRatio | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.31 |  |
| > portDS.currentComputeNeighborRateRatio | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.32 |  |
| > portDS.useMgtSettableComputeNeighborRateRatio | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.33 |  |
| > portDS.mgtSettableComputeNeighborRateRatio | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.34 |  |
| > portDS.initialComputeMeanLinkDelay | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.35 |  |
| > portDS.currentComputeMeanLinkDelay | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.36 |  |
| > portDS.useMgtSettableComputeMeanLinkDelay | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.37 |  |
| > portDS.mgtSettableComputeMeanLinkDelay | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.38 |  |
| > portDS.allowedLostResponses | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.39 |  |
| > portDS.allowedFaults | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.40 |  |
| > portDS.gPtpCapableReceiptTimeout | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.41 |  |
| > portDS.versionNumber | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.42 |  |
| > portDS.nup | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.43 |  |
| > portDS.ndown | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.44 |  |
| > portDS.oneStepTxOper | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.45 |  |
| > portDS.oneStepReceive | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.46 |  |
| > portDS.oneStepTransmit | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.47 |  |
| > portDS.initialOneStepTxOper | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.48 |  |
| > portDS.currentOneStepTxOper | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.49 |  |
| > portDS.useMgtSettableOneStepTxOper | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.50 |  |
| > portDS.mgtSettableOneStepTxOper | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.51 |  |
| > portDS.syncLocked | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.52 |  |
| > portDS.pdelayTruncatedTimestampsArray | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.53 |  |
| > portDS.minorVersionNumber | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.54 |  |
| > timePropertiesDS.currentUtcOffset | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.5.2 |  |
| > externalPortConfigurationPortDS.desiredState |  | R | RW | RW | IEEE Std 802.1AS [104] clause 14.12.2 |  |
| NOTE 1: R = Read only access; RW = Read/Write access; ― = not supported. NOTE 2: Indicates which standardized and deployment-specific port management information is supported by DS-TT or NW-TT. NOTE 3: AdminCycleTime and AdminControlListLength are optional for gate control information. NOTE 4: If DS-TT supports neighbor discovery, then TSN AF sends the general neighbor discovery configuration for DS-TT Ethernet ports to DS-TT. If DS-TT does not support neighbor discovery, then TSN AF sends the general neighbor discovery configuration for DS-TT Ethernet ports to NW-TT using the Bridge Management Information Container (refer to Table 5.28.3.1-2) and NW-TT performs neighbor discovery on behalf on DS-TT. When a parameter in this group is changed, it is necessary to provide the change to every DS-TT and the NW-TT that belongs to the 5GS TSN bridge. It is mandatory that the general neighbor discovery configuration is identical for all DS-TTs and the NW-TTs that belongs to the bridge. NOTE 5: If DS-TT supports neighbor discovery, then TSN AF retrieves neighbor discovery information for DS-TT Ethernet ports from DS-TT. If DS-TT does not support neighbor discovery, then TSN AF retrieves neighbor discovery information for DS-TT Ethernet ports from NW-TT, using the Bridge Management Information Container (refer to Table 5.28.3.1-2), the NW-TT performing neighbor discovery on behalf on DS-TT. NOTE 6: X = applicable; D = applicable when validation and generation of LLDP frames is processed at the DS-TT. NOTE 7: Void. NOTE 8: There is a Stream Filter Instance Table per Stream. NOTE 9: There is a Stream Gate Instance Table per Gate. NOTE 10: TSN AF indicates the support for PSFP to the CNC only if each DS-TT and NW-TT of the 5GS bridge has indicated support of PSFP. DS-TT indicates support of PSFP using port management capabilities, i.e. by indicating support for the Per-Stream Filtering and Policing information and by setting higher than zero values for MaxStreamFilterInstances, MaxStreamGateInstances, MaxFlowMeterInstances, SupportedListMax parameters. When available, TSN AF uses the PSFP information for determination of the traffic pattern information as described in Annex I. The PSFP information can be used at the DS-TT (if supported) and at the NW-TT (if supported) for the purpose of per-stream filtering and policing as defined in IEEE Std 802.1Q [98] clause 8.6.5.1. NOTE 11: TSN AF composes a Stream Parameter Table towards the CNC. It is up to TSN AF how it composes the Stream Parameter Table based on the numerical values as received from DS-TT and NW-TT port(s) and for the bridge for each individual parameter. NOTE 12: The set of Stream Identification Controlling Parameters depends on the Stream Identification type value as defined in IEEE Std 802.1CB [83] Table 9-1 and clauses 9.1.2, 9.1.3, 9.1.4. NOTE 13: Enumeration of supported PTP instance types. Allowed values as defined in IEEE Std 1588-2019 [126] clause 8.2.1.5.5. NOTE 14: Enumeration of supported transport types. Allowed values: IPv4 (as defined in IEEE Std 1588-2019 [126] Annex C), IPv6 (as defined in IEEE Std 1588-2019 [126] Annex D), Ethernet (as defined in IEEE Std 1588-2019 [126] Annex E). NOTE 15: Enumeration of supported PTP delay mechanisms. Allowed values as defined in IEEE Std 1588-2019 [126] clause 8.2.15.4.4. NOTE 16: Indicates whether DS-TT supports acting as a PTP grandmaster. NOTE 17: Indicates whether DS-TT supports acting as a gPTP grandmaster. NOTE 18: Enumeration of supported PTP profiles, each identified by PTP profile ID, as defined in IEEE Std 1588-2019 [126] clause 20.3.3. NOTE 19: PTP profile to apply, identified by PTP profile ID, as defined in IEEE Std 1588-2019 [126] clause 20.3.3. NOTE 20: Transport type to use. Allowed values: IPv4 (as defined in IEEE Std 1588-2019 [126] Annex C), IPv6 (as defined in IEEE Std 1588-2019 [126] Annex D), Ethernet (as defined in IEEE Std 1588-2019 [126] Annex E). NOTE 21: Indicates whether to act as grandmaster or not, i.e. whether to send Announce, Sync and optionally Follow_Up messages. NOTE 22: The IEEE Std 802.1AS [104] data sets apply if the IEEE 802.1AS PTP profile is used; otherwise the IEEE Std 1588-2019 [126] data sets apply. |  |  |  |  |  |

Table 5.28.3.1-2: Standardized bridge management information

| Bridge management information | Supported operations by TSN AF (see NOTE 1) | Supported operations by NEF (see NOTE 1) | Reference |
| --- | --- | --- | --- |
| Information for 5GS Bridge |  |  |  |
| Bridge Address | R | R |  |
| Bridge ID | R | R |  |
| NW-TT port numbers | R | R |  |
| Traffic forwarding information |  |  |  |
| Static Filtering Entry (NOTE 3) | RW | ― | IEEE Std 802.1Q [98] clause 8.8.1 |
| General Neighbor discovery configuration (NOTE 2) |  |  |  |
| adminStatus | RW | ― | IEEE Std 802.1AB [97] clause 9.2.5.1 |
| lldpV2LocChassisIdSubtype | RW | ― | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2LocChassisId | RW | ― | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2MessageTxInterval | RW | ― | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2MessageTxHoldMultiplier | RW | ― | IEEE Std 802.1AB [97] Table 11-2 |
| DS-TT port neighbor discovery configuration for DS-TT ports (NOTE 4) |  |  |  |
| >DS-TT port neighbor discovery configuration for each DS-TT port |  |  |  |
| >> DS-TT port number | RW | ― |  |
| >> lldpV2LocPortIdSubtype | RW | ― | IEEE Std 802.1AB [97] Table 11-2 |
| >> lldpV2LocPortId | RW | ― | IEEE Std 802.1AB [97] Table 11-2 |
| Discovered neighbor information for DS-TT ports (NOTE 4) |  |  |  |
| >Discovered neighbor information for each DS-TT port (NOTE 4) |  |  |  |
| >> DS-TT port number | R | ― |  |
| >> lldpV2RemChassisIdSubtype | R | ― | IEEE Std 802.1AB [97] Table 11-2 |
| >> lldpV2RemChassisId | R | ― | IEEE Std 802.1AB [97] Table 11-2 |
| >> lldpV2RemPortIdSubtype | R | ― | IEEE Std 802.1AB [97] Table 11-2 |
| >> lldpV2RemPortId | R | ― | IEEE Std 802.1AB [97] Table 11-2 |
| >> TTL | R | ― | IEEE Std 802.1AB [97] clause 8.5.4.1 |
| Stream Parameters (NOTE 5) |  |  |  |
| MaxStreamFilterInstances | R | ― | IEEE Std 802.1Q [98] |
| MaxStreamGateInstances | R | ― | IEEE Std 802.1Q [98] |
| MaxFlowMeterInstances | R | ― | IEEE Std 802.1Q [98] |
| SupportedListMax | R | ― | IEEE Std 802.1Q [98] |
| Time synchronization information |  |  |  |
| Supported PTP instance types (NOTE 6) | R | R |  |
| Supported transport types (NOTE 7) | R | R |  |
| Supported delay mechanisms (NOTE 8) | R | R |  |
| PTP grandmaster capable (NOTE 9) | R | R |  |
| gPTP grandmaster capable (NOTE 10) | R | R |  |
| Supported PTP profiles (NOTE 11) | R | R |  |
| Number of supported PTP instances | R | R |  |
| Grandmaster candidate enabled | RW | RW |  |
| Time synchronization information for DS-TT ports |  |  |  |
| > Time synchronization information for each DS-TT port |  |  |  |
| > DS-TT port number | RW | RW |  |
| >> Time synchronization information for each PTP Instance |  |  |  |
| >> PTP Instance ID | RW | RW |  |
| >> PTP profile (NOTE 12) | RW | RW |  |
| >> Transport type (NOTE 13) | RW | RW |  |
| >> Grandmaster on behalf of DS-TT enabled (NOTE 14) | RW | RW |  |
| IEEE Std 1588-2019 [126] data sets (NOTE 15) |  |  |  |
| >> defaultDS.clockIdentity | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.1.2.2 |
| >> defaultDS.clockQuality.clockClass | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.1.3.1.2 |
| >> defaultDS.clockQuality.clockAccuracy | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.1.3.1.3 |
| >> defaultDS.clockQuality.offsetScaledLogVariance | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.1.3.1.4 |
| >> defaultDS.priority1 | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.1.4.1 |
| >> defaultDS.priority2 | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.1.4.2 |
| >> defaultDS.domainNumber | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.1.4.3 |
| >> defaultDS.sdoId | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.1.4.5 |
| >> defaultDS.instanceEnable | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.1.5.2 |
| >> defaultDS.externalPortConfigurationEnabled | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.1.5.3 |
| >> defaultDS.instanceType | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.1.5.5 |
| >> portDS.portIdentity | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.15.2.1 |
| >> portDS.portState | R | R | IEEE Std 1588-2019 [126] clause 8.2.15.3.1 |
| >> portDS.logMinDelayReqInterval | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.15.3.2 |
| >> portDS.logAnnounceInterval | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.1 |
| >> portDS.announceReceiptTimeout | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.2 |
| >> portDS.logSyncInterval | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.3 |
| >> portDS.delayMechanism | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.4 |
| >> portDS.logMinPdelayReqInterval | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.5 |
| >> portDS.versionNumber | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.6 |
| >> portDS.minorVersionNumber | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.7 |
| >> portDS.delayAssymetry | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.8 |
| >> portDS.portEnable | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.15.5.1 |
| >> timePropertiesDS.currentUtcOffset | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.4.2 |
| >> timePropertiesDS.timeSource | RW | RW | IEEE Std 1588-2019 [126] clause 8.2.4.9 |
| >> externalPortConfigurationPortDS.desiredState | RW | RW | IEEE Std 1588-2019 [126] clause 15.5.3.7.15.1 |
| IEEE Std 802.1AS [104] data sets (NOTE 15) |  |  |  |
| >> defaultDS.clockIdentity | RW | RW | IEEE Std 802.1AS [104] clause 14.2.2 |
| >> defaultDS.clockQuality.clockClass | RW | RW | IEEE Std 802.1AS [104] clause 14.2.4.2 |
| >> defaultDS.clockQuality.clockAccuracy | RW | RW | IEEE Std 802.1AS [104] clause 14.2.4.3 |
| >> defaultDS.clockQuality.offsetScaledLogVariance | RW | RW | IEEE Std 802.1AS [104] clause 14.2.4.4 |
| >> defaultDS.priority1 | RW | RW | IEEE Std 802.1AS [104] clause 14.2.5 |
| >> defaultDS.priority2 | RW | RW | IEEE Std 802.1AS [104] clause 14.2.6 |
| >> defaultDS.timeSource | RW | RW | IEEE Std 802.1AS [104] clause 14.2.15 |
| >> defaultDS.domainNumber | RW | RW | IEEE Std 802.1AS [104] clause 14.2.16 |
| >> defaultDS.externalPortConfigurationEnabled | RW | RW | IEEE Std 802.1AS [104] clause 14.2.18 |
| >> defaultDS.sdoId | RW | RW | IEEE Std 802.1AS [104] clause 14.2.4.3 |
| >> defaultDS.instanceEnable | RW | RW | IEEE Std 802.1AS [104] clause 14.2.19 |
| >> portDS.portIdentity | RW | RW | IEEE Std 802.1AS [104] clause 14.8.2 |
| >> portDS.portState | R | R | IEEE Std 802.1AS [104] clause 14.8.3 |
| >> portDS.ptpPortEnabled | RW | RW | IEEE Std 802.1AS [104] clause 14.8.4 |
| >> portDS.delayMechanism | RW | RW | IEEE Std 802.1AS [104] clause 14.8.5 |
| >> portDS.isMeasuringDelay | R | R | IEEE Std 802.1AS [104] clause 14.8.6 |
| >> portDS.asCapable | R | R | IEEE Std 802.1AS [104] clause 14.8.7 |
| >> portDS.meanLinkDelay | R | R | IEEE Std 802.1AS [104] clause 14.8.8 |
| >> portDS.meanLinkDelayThresh | RW | RW | IEEE Std 802.1AS [104] clause 14.8.9 |
| >> portDS.delayAssymetry | RW | RW | IEEE Std 802.1AS [104] clause 14.8.10 |
| >> portDS.neighborRateRatio | R | R | IEEE Std 802.1AS [104] clause 14.8.11 |
| >> portDS.initialLogAnnounceInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.12 |
| >> portDS.currentLogAnnounceInterval | R | R | IEEE Std 802.1AS [104] clause 14.8.13 |
| >> portDS.useMgtSettableLogAnnounceInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.14 |
| >> portDS.mgtSettableLogAnnounceInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.15 |
| >> portDS.announceReceiptTimeout | RW | RW | IEEE Std 802.1AS [104] clause 14.8.16 |
| >> portDS.initialLogSyncInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.17 |
| >> portDS.currentLogSyncInterval | R | R | IEEE Std 802.1AS [104] clause 14.8.18 |
| >> portDS.useMgtSettableLogSyncInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.19 |
| >> portDS.mgtSettableLogSyncInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.20 |
| >> portDS.syncReceiptTimeout | RW | RW | IEEE Std 802.1AS [104] clause 14.8.21 |
| >> portDS.syncReceiptTimeoutTimeInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.22 |
| >> portDS.initialLogPdelayReqInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.23 |
| >> portDS.currentLogPdelayReqInterval | R | R | IEEE Std 802.1AS [104] clause 14.8.24 |
| >> portDS.useMgtSettableLogPdelayReqInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.25 |
| >> portDS.mgtSettableLogPdelayReqInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.26 |
| >> portDS.initialLogGptpCapableMessageInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.27 |
| >> portDS.currentLogGptpCapableMessageInterval | R | R | IEEE Std 802.1AS [104] clause 14.8.28 |
| >> portDS.useMgtSettableLogGptpCapableMessageInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.29 |
| >> portDS.mgtSettableLogGptpCapableMessageInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.30 |
| >> portDS.initialComputeNeighborRateRatio | RW | RW | IEEE Std 802.1AS [104] clause 14.8.31 |
| >> portDS.currentComputeNeighborRateRatio | R | R | IEEE Std 802.1AS [104] clause 14.8.32 |
| >> portDS.useMgtSettableComputeNeighborRateRatio | RW | RW | IEEE Std 802.1AS [104] clause 14.8.33 |
| >> portDS.mgtSettableComputeNeighborRateRatio | RW | RW | IEEE Std 802.1AS [104] clause 14.8.34 |
| >> portDS.initialComputeMeanLinkDelay | RW | RW | IEEE Std 802.1AS [104] clause 14.8.35 |
| >> portDS.currentComputeMeanLinkDelay | R | R | IEEE Std 802.1AS [104] clause 14.8.36 |
| >> portDS.useMgtSettableComputeMeanLinkDelay | RW | RW | IEEE Std 802.1AS [104] clause 14.8.37 |
| >> portDS.mgtSettableComputeMeanLinkDelay | RW | RW | IEEE Std 802.1AS [104] clause 14.8.38 |
| >> portDS.allowedLostResponses | RW | RW | IEEE Std 802.1AS [104] clause 14.8.39 |
| >> portDS.allowedFaults | RW | RW | IEEE Std 802.1AS [104] clause 14.8.40 |
| >> portDS.gPtpCapableReceiptTimeout | RW | RW | IEEE Std 802.1AS [104] clause 14.8.41 |
| >> portDS.versionNumber | RW | RW | IEEE Std 802.1AS [104] clause 14.8.42 |
| >> portDS.nup | RW | RW | IEEE Std 802.1AS [104] clause 14.8.43 |
| >> portDS.ndown | RW | RW | IEEE Std 802.1AS [104] clause 14.8.44 |
| >> portDS.oneStepTxOper | R | R | IEEE Std 802.1AS [104] clause 14.8.45 |
| >> portDS.oneStepReceive | R | R | IEEE Std 802.1AS [104] clause 14.8.46 |
| >> portDS.oneStepTransmit | R | R | IEEE Std 802.1AS [104] clause 14.8.47 |
| >> portDS.initialOneStepTxOper | RW | RW | IEEE Std 802.1AS [104] clause 14.8.48 |
| >> portDS.currentOneStepTxOper | RW | RW | IEEE Std 802.1AS [104] clause 14.8.49 |
| >> portDS.useMgtSettableOneStepTxOper | RW | RW | IEEE Std 802.1AS [104] clause 14.8.50 |
| >> portDS.mgtSettableOneStepTxOper | RW | RW | IEEE Std 802.1AS [104] clause 14.8.51 |
| >> portDS.syncLocked | R | R | IEEE Std 802.1AS [104] clause 14.8.52 |
| >> portDS.pdelayTruncatedTimestampsArray | RW | RW | IEEE Std 802.1AS [104] clause 14.8.53 |
| >> portDS.minorVersionNumber | RW | RW | IEEE Std 802.1AS [104] clause 14.8.54 |
| >> timePropertiesDS.currentUtcOffset | RW | RW | IEEE Std 802.1AS [104] clause 14.5.2 |
| >> externalPortConfigurationPortDS.desiredState | RW | RW | IEEE Std 802.1AS [104] clause 14.12.2 |
| NOTE 1: R = Read only access; RW = Read/Write access; ― = not supported. NOTE 2: General neighbor discovery information is included only when NW-TT performs neighbor discovery on behalf of DS-TT. When a parameter in this group is changed, it is necessary to provide the change to every DS-TT and the NW-TT that belongs to the 5GS TSN bridge. NOTE 3: If the Static Filtering Entry information is present for uplink traffic, UPF/NW-TT uses Static Filtering Entry information to determine the NW-TT egress port for forwarding UL TSC traffic. The user plane forwarding is further specified in clause 5.8.2.5.3. NOTE 4: DS-TT discovery configuration and DS-TT discovery information are used only when DS-TT does not support LLDP and NW-TT performs neighbor discovery on behalf of DS-TT. NOTE 5: TSN AF indicates the support for PSFP to the CNC only if each DS-TT and NW-TT of the 5GS bridge have indicated support of PSFP. The support of PSFP at the NW-TT ports is expressed by setting higher than zero values for MaxStreamFilterInstances, MaxStreamGateInstances, MaxFlowMeterInstances, SupportedListMax parameters. NOTE 6: Enumeration of supported PTP instance types. Allowed values as defined in IEEE Std 1588-2019 [126] clause 8.2.1.5.5. NOTE 7: Enumeration of supported transport types. Allowed values: IPv4 (as defined in IEEE Std 1588-2019 [126] Annex C), IPv6 (as defined in IEEE Std 1588-2019 [126] Annex D), Ethernet (as defined in IEEE Std 1588-2019 [126] Annex E). NOTE 8: Enumeration of supported PTP delay mechanisms. Allowed values as defined in IEEE Std 1588-2019 [126] clause 8.2.15.4.4. NOTE 9: Indicates whether NW-TT supports acting as a PTP grandmaster. NOTE 10: Indicates whether NW-TT supports acting as a gPTP grandmaster. NOTE 11: Enumeration of supported PTP profiles, each identified by PTP profile ID, as defined in IEEE Std 1588-2019 [126] clause 20.3.3. NOTE 12: PTP profile to apply, identified by PTP profile ID, as defined in IEEE Std 1588-2019 [126] clause 20.3.3. NOTE 13: Transport type to use. Allowed values: IPv4 (as defined in IEEE Std 1588-2019 [126] Annex C), IPv6 (as defined in IEEE Std 1588-2019 [126] Annex D), Ethernet (as defined in IEEE Std 1588-2019 [126] Annex E). NOTE 14: Indicates whether to act as grandmaster on behalf of a DS-TT port or not if 5GS is determined to be the grandmaster clock, i.e. whether to send Announce, Sync and optionally Follow_Up messages on behalf of DS-TT. NOTE 15: The IEEE Std 802.1AS [104] data sets apply if the IEEE 802.1AS PTP profile is used; otherwise the IEEE Std 1588-2019 [126] data sets apply. |  |  |  |

Exchange of port and bridge management information between TSN AF or NEF and NW-TT or between TSN AF or NEF and DS-TT allows TSN AF or NEF to:

1)	retrieve port management information for a DS-TT or NW-TT port or bridge management information;

2)	send port management information for a DS-TT or NW-TT port or bridge management information;

3)	subscribe to and receive notifications if specific port management information for a DS-TT or NW-TT port changes or bridge management information changes.

Exchange of port management information between TSN AF or NEF and NW-TT or DS-TT is initiated by DS-TT or NW-TT to:

-	notify TSN AF if port management information has changed that TSN AF has subscribed for.

Exchange of bridge management information between TSN AF and NW-TT is initiated by NW-TT to:

-	notify TSN AF or NEF if bridge management information has changed that TSN AF or NEF has subscribed for.

Exchange of port management information is initiated by DS-TT to:

-	provide port management capabilities, i.e. provide information indicating which standardized and deployment-specific port management information is supported by DS-TT.

TSN AF or NEF indicates inside the Port Management Information Container or Bridge Management Information Container whether it wants to retrieve or send port or bridge management information or intends to (un-)subscribe for notifications.

*** End of Changes ***