3GPP TSG-WG SA2 Meeting #145E e-meeting 	S2-2105192

Elbonia, May 17– 28, 2021	(revision of S2-2104309)

Source:	Huawei, HiSilicon

Title:	TS 23.247: Session Update Procedures

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract: This contribution resolves the issues regarding session update procedures.

1. Discussion

Session update procedures for 5MBS are with several ENs and it is proposed to resolve the update procedures by clarifying the ENs and adding the procedures for multicast.

In general, the companies believes that QoS flow could be added/removed during update procedure. Therefore, it is proposed to document such aspect.

Proposal 1: The update procedure is proposed to cover the case of adding/removing the QoS flow.

For the update for common procedure, see the agreed version of S2-2103955.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.247.

# * * * * First change * * * * (All Text New)

7.2.X	Multicast session update procedure

Multicast session update procedure is invoked by the AF to update the ARP for an ongoing multicast session. For the interaction between AF and MB-SMF, see clause 7.1.1.Y.

RAN

AMF

SMF

MB-SMF

2. MBS Session update Request ()

3. NGAP Session update Request ()

7. Session update Request (TMGI)

5. NGAP Session update Response

6. MBS Session update Response

UE

8. PDU Session Modification procedure defined in TS 23.502

4. RAN updates MBS parameters.

1. MB-SMF triggers session update

Figure 7.2.X-1: Multicast session update procedure.

Editor's Note: names of messages are FFS.

1.	This procedure is triggered by the MB-SMF receiving the updated policy for MBS, see clause 7.1.1.Y.

2.	The MB-SMF generates the QoS profile for the multicast, and sends MBS Session update Request (N2 SM message (TMGI, QoS profiles for multicast)) to the AMF(s).

3.	The involved AMF forwards the N2 SM information received from MB-SMF to the RAN nodes via NGAP Session update Request (N2 SM message (TMGI, QoS profiles for multicast)) message.

Editor's Note: Whether it is the AMF or the MB-SMF that stores RAN ID for the interaction with RAN in step 3 needs to align with clause 7.2.1.

4.	NG-RAN updates the MBS session context, and if only the ARP of QoS parameters is updated, NG-RAN node also updates the QoS parameters of the associating PDU Sessions.

Editor's Note: Updating the associated PDU Session’s QoS based on the update of MBS Session QoS by NG-RAN requires RAN WGs collaboration.

5.	The NG-RAN acknowledges NGAP Session update Request by sending an NGAP Session update Response message to the AMF.

6.	The AMF sends MBS Session update Response to the MB-SMF.

7.	MB-SMF sends Session update Request (TMGI, QoS profiles for multicast) to SMF.

8.	SMF determines the UE list regarding the TMGI included in the message. If the QoS parameters other than ARP needs to be updated, SMF triggers PDU Session Modification procedure for each UE as defined in TS 23.502 [6].

# * * * * End of changes * * * *