3GPP TSG-WG SA2 Meeting #145E e-meeting  	S2-2105203

# Elbonia, May 17 – 28, 2021	(revision of S2-2104574, 3481)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2676 | rev | 3 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PDU Session establishment and modification admission control based on network Slice maximum data rate |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Xiaomi, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  |  | Date: |  | 2021-05-28 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR is proposed to implement in TS 23.502 the following conclusion for KI#5 in clause 8.5 in TR 23.700-40. - Enforcement: the PCF ensures that no new PDU sessions or QoS flows of network slices with registered UEs are admitted if this may cause overflowing the NW Slice maximum data rate parameter stored in the UDR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implement the above conclusion. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eNS_Ph2 feature not implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.12.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Rev 2 removes an unnecessary addition and updates the reference. |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

##### 5.2.12.2.1	General

The operations defined for Nudr_DM service use following set of parameters defined in this clause:

-	Data Set Identifier: uniquely identifies the requested set of data within the UDR (see clause 4.2.5).

-	Data Subset Identifier: it uniquely identifies the data subset within each Data Set Identifier. As specified in the procedures in clause 4, e.g. subscription data can consist of subsets particularised for specific procedures like mobility, session, etc.

-	Data Keys defined in Table 5.2.12.2.1-1

For Nudr_DM_Subscribe and Nudr_DM_Notify operations:

-	The Target of Event Reporting is made up of a Data Key and possibly a Data Sub Key both defined in Table 5.2.12.2.1-1. When a Data Sub Key is defined in the table but not present in the Nudr_DM_Subscribe this means that all values of the Data Sub Key are targeted.

-	The Data Set Identifier plus (if present) the (set of) Data Subset Identifier(s) corresponds to a (set of) Event ID(s) as defined in clause 4.15.1

An NF Service Consumer may include an indicator when it invokes Nudr_DM Query/Create/Update service operation to subscribe the changes of the data, to avoid a separate Nudr_DM_Subscribe service operation.

Depending on the use case, it is possible to use a Data Key and/or one or multiple Data sub keys to further identify the corresponding data, as defined in Table 5.2.12.2.1-1 below.

Table 5.2.12.2.1-1: Data keys

| Data Set | Data Subset | Data Key | Data Sub Key |
| --- | --- | --- | --- |
|  | Access and Mobility Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | SMF Selection Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | UE context in SMF data | SUPI | PDU Session ID or DNN |
| Subscription Data (see clause 5.2.3.3.1) | SMS Management Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | SMS Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | Session Management Subscription data | SUPI | S-NSSAI |
|  |  |  | DNN |
|  |  |  | Serving PLMN ID and optionally NID |
|  | Slice Selection Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | Group Data | Internal Group Identifier or External Group Identifier | - |
|  | Identifier translation | GPSI |  |
|  |  | SUPI | Application Port ID |
|  | Intersystem continuity Context | SUPI | DNN |
|  | LCS privacy | SUPI | - |
|  | LCS mobile origination | SUPI | - |
|  | UE reachability | SUPI | - |
|  | Group Identifier Translation | Internal Group Identifier or External Group Identifier | - |
|  | UE context in SMSF data | SUPI | - |
|  | V2X Subscription data | SUPI | - |
|  | ProSe Subscription data | SUPI | - |
| Application data | Packet Flow Descriptions (PFDs) | Application Identifier | - |
|  | AF traffic influence request information | AF transaction internal ID |  |
|  | (See clause 5.6.7 and clause 6.3.7.2 in TS 23.501 [2]). | S-NSSAI and DNN and/or Internal Group Identifier or SUPI |  |
|  | Background Data Transfer (NOTE 3) | Internal Group Identifier or SUPI |  |
|  | Service specific information (See clause 4.15.6.7) | S-NSSAI and DNN or Internal Group Identifier or SUPI |  |
|  | AM policy influence request information (See clause 4.15.6.9.3) | AF transaction internal ID |  |
|  |  | S-NSSAI and DNN and/or Internal Group Identifier or SUPI |  |
| Policy Data | UE context policy control data (See clause 6.2.1.3 in TS 23.503 [20]) | SUPI |  |
|  | PDU Session policy control data | SUPI | S-NSSAI |
|  | (See clause 6.2.1.3 in TS 23.503 [20]) |  | DNN |
|  | Policy Set Entry data (See clause 6.2.1.3 in TS 23.503 [20]) | SUPI (for the UDR in HPLMN) |  |
|  |  | PLMN ID (for the UDR in VPLMN) |  |
|  | Remaining allowed Usage data | SUPI | S-NSSAI |
|  | (See clause 6.2.1.3 in TS 23.503 [20]) |  | DNN |
|  | Sponsored data connectivity profiles (See clause 6.2.1.6 in TS 23.503 [20]) | Sponsor Identity |  |
|  | Background Data Transfer data (See clause 6.2.1.6 in TS 23.503 [20]) | Background Data Transfer Reference ID. (NOTE 2) |  |
|  |  | None. (NOTE 1) |  |
|  | Remaining Data Rate (See clause 6.2.1.3 in TS 23.503 [20]) | S-NSSAI |  |
| Exposure Data | Access and Mobility Information | SUPI or GPSI | PDU Session ID or |
| (see clause 5.2.12.1) | Session Management information | SUPI or GPSI | UE IP address or DNN |
| NOTE 1: Retrieval of the stored Background Data Transfer References for all ASP identifiers in the UDR requires Data Subset but no Data Key or Data Subkey(s). NOTE 2: Update of a Background Data Transfer Reference in the UDR requires a Data key to refer to a Background Data Transfer Reference as input data. NOTE 3: The Background Data Transfer includes the Background Data Reference ID and the ASP id that requests to apply the Background Data Reference ID to the UE(s). Furthermore, the Background Data Transfer includes the relevant information received from the AF as defined in clause 6.1.2.4, TS 23.503 [20]. |  |  |  |

The content of the UDR storage for (Data Set Id= Application Data, Data Subset Id = AF TrafficInfluence request information) is specified in TS 23.501 [2], clause 5.6.7, Table 5.6.7-1. This information is written by the NEF and read by the PCF(s). PCF(s) may also subscribe to changes onto this information.

* * * * End of Changes * * * *