3GPP TSG-SA2 Meeting # 145E (e-meeting)	S2-2103807

# Online, May  17 – 28, 2021										  				   (revision of S2-2103069)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2813 | rev | 2 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of support of NG.116 attribute "Simultaneous Use of a Network Slice" |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, (Nokia, Nokia Shanghai Bell, Telecom Italia, Convida Wireless, T-Mobile, Apple, Verizon UK ltd., NEC) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduction of support of NG.116 attribute "Simultaneous Use of a Network Slice" |  |  |  |  |  |  |  |  |
| Summary of change: |  | General description : Revisions with respect to previsou revisions Provide complete backward compatibilty for PLMSN not supporting the feature as well UEs not supporting the feature. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing function description |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.15.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR …CR … |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START CHANGE ***************

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5G-VN	5G Local Area Network

5GS	5G System

5G-AN	5G Access Network

5G-AN PDB	5G Access Network Packet Delay Budget

5G-EIR	5G-Equipment Identity Register

5G-GUTI	5G Globally Unique Temporary Identifier

5G-BRG	5G Broadband Residential Gateway

5G-CRG	5G Cable Residential Gateway

5G GM	5G Grand Master

5G-RG	5G Residential Gateway

5G-S-TMSI	5G S-Temporary Mobile Subscription Identifier

5G VN	5G Virtual Network

5QI	5G QoS Identifier

ADRF	Analytics Data Repository Function

AF	Application Function

AKMA	Authentication and Key Management for Applications

AnLF	Analytics Logical Function

AMF	Access and Mobility Management Function

AS	Access Stratum

ATSSS	Access Traffic Steering, Switching, Splitting

ATSSS-LL	ATSSS Low-Layer

AUSF	Authentication Server Function

BMCA	Best Master Clock Algorithm

BSF	Binding Support Function

CAG	Closed Access Group

CAPIF	Common API Framework for 3GPP northbound APIs

CH	Credentials Holder

CHF	Charging Function

CN PDB	Core Network Packet Delay Budget

CP	Control Plane

DAPS	Dual Active Protocol Stacks

DCCF	Data Collection Coordination Function

DL	Downlink

DN	Data Network

DNAI	DN Access Identifier

DNN	Data Network Name

DRX	Discontinuous Reception

DS-TT	Device-side TSN translator

ePDG	evolved Packet Data Gateway

EBI	EPS Bearer Identity

EUI	Extended Unique Identifier

FAR	Forwarding Action Rule

FN-BRG	Fixed Network Broadband RG

FN-CRG	Fixed Network Cable RG

FN-RG	Fixed Network RG

FQDN	Fully Qualified Domain Name

GFBR	Guaranteed Flow Bit Rate

GIN	Group ID for Network Selection

GMLC	Gateway Mobile Location Centre

GPSI	Generic Public Subscription Identifier

GUAMI	Globally Unique AMF Identifier

HMTC	High-Performance Machine-Type Communications

HR	Home Routed (roaming)

IAB	Integrated access and backhaul

IMEI/TAC	IMEI Type Allocation Code

IPUPS	Inter PLMN UP Security

I-SMF	Intermediate SMF

I-UPF	Intermediate UPF

LADN	Local Area Data Network

LBO	Local Break Out (roaming)

LMF	Location Management Function

LoA	Level of Automation

LPP	LTE Positioning Protocol

LRF	Location Retrieval Function

MFAF	Messaging Framework Adaptor Function

MCX	Mission Critical Service

MDBV	Maximum Data Burst Volume

MFBR	Maximum Flow Bit Rate

MICO	Mobile Initiated Connection Only

ML	Machine Learning

MPS	Multimedia Priority Service

MPTCP	Multi-Path TCP Protocol

MTLF	Model Training Logical Function

N3IWF	Non-3GPP InterWorking Function

N5CW	Non-5G-Capable over WLAN

NAI	Network Access Identifier

NEF	Network Exposure Function

NF	Network Function

NGAP	Next Generation Application Protocol

NID	Network identifier

NPN	Non-Public Network

NR	New Radio

NRF	Network Repository Function

NSAC	Network Slice Admission Control

NSACF	Network Slice Admission Control Function

NSI ID	Network Slice Instance Identifier

NSSAA	Network Slice-Specific Authentication and Authorization

NSSAAF	Network Slice-Specific Authentication and Authorization Function

NSSAI	Network Slice Selection Assistance Information

NSSF	Network Slice Selection Function

NSSP	Network Slice Selection Policy

NW-TT	Network-side TSN translator

NWDAF	Network Data Analytics Function

PCF	Policy Control Function

PDB	Packet Delay Budget

PDR	Packet Detection Rule

PDU	Protocol Data Unit

PEI	Permanent Equipment Identifier

PER	Packet Error Rate

PFD	Packet Flow Description

PNI-NPN	Public Network Integrated Non-Public Network

PPD	Paging Policy Differentiation

PPF	Paging Proceed Flag

PPI	Paging Policy Indicator

PSA	PDU Session Anchor

PTP	Precision Time Protocol

QFI	QoS Flow Identifier

QoE	Quality of Experience

RACS	Radio Capabilities Signalling optimisation

(R)AN	(Radio) Access Network

RG	Residential Gateway

RIM	Remote Interference Management

RQA	Reflective QoS Attribute

RQI	Reflective QoS Indication

RSN	Redundancy Sequence Number

SA NR	Standalone New Radio

SBA	Service Based Architecture

SBI	Service Based Interface

SCP	Service Communication Proxy

SD	Slice Differentiator

SEAF	Security Anchor Functionality

SEPP	Security Edge Protection Proxy

SMF	Session Management Function

SMSF	Short Message Service Function

SN	Sequence Number

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

SRG	Simultaneous Registration Group

SSC	Session and Service Continuity

SSCMSP	Session and Service Continuity Mode Selection Policy

SST	Slice/Service Type

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

SV	Software Version

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TNL	Transport Network Layer

TNLA	Transport Network Layer Association

TSC	Time Sensitive Communication

TSCAI	TSC Assistance Information

TSN	Time Sensitive Networking

TSN GM	TSN Grand Master

TSP	Traffic Steering Policy

TT	TSN Translator

TWIF	Trusted WLAN Interworking Function

UCMF	UE radio Capability Management Function

UDM	Unified Data Management

UDR	Unified Data Repository

UDSF	Unstructured Data Storage Function

UL	Uplink

UL CL	Uplink Classifier

UPF	User Plane Function

URLLC	Ultra Reliable Low Latency Communication

URRP-AMF	UE Reachability Request Parameter for AMF

URSP	UE Route Selection Policy

VID	VLAN Identifier

VLAN	Virtual Local Area Network

W-5GAN	Wireline 5G Access Network

W-5GBAN	Wireline BBF Access Network

W-5GCAN	Wireline 5G Cable Access Network

W-AGF	Wireline Access Gateway Function

****************** NEXT CHANGES ***************

## 5.15.x	Support of subscription-based restrictions to simultaneous registration of network slices

#### 5.15.x.1	General

The subscription information for a UE may include for each S-NSSAI one or more scalar values reflecting  simultaneous usability of an S-NSSAI with other S-NSSAIs. These scalar values are called Simultaneous Registration grouping (SRG), and enables a UE to identify groups/lists of S-NSSAIs that can be requested/used simultaneously.

The SRG is a HPLMN-assigned value which has the scope for a single subscription. Two or more S-NSSAIs can be simultaneously used, if they share at least one identical SRG value. For backward compatibility, the SRG Information, defining the association of S-NSSAIs to SRG, shall be additional and separate information from the S-NSSAI information.

For SRG enabled subscriptions,

at least one SRG value shall be associated with each of the S-NSSAIs in the SRG enabled subscription. At any time, if the subscription information of a UE includes SRG values, the Allowed NSSAI for the UE can only include S-NSSAIs which share the same SRG value.

To enable interworking  with non-supporting UEs, and non-supporting PLMNs, UDM subscriptions must equally include subscribed S-NSSAIs without SRG information for all UE subscriptions. Hence UDM maintains UE subscribed S-NSSAIs with SRG information and without SRG information.

If SRG information is not present in UE subscriptions, it is assumed the PLMN does not support this feature.

If the serving network does not support SRG information, the UDM determines based on the subscription information which S-NSSAIs can be sent to the AMF, e.g. such that all S-NSSAIs provided to the serving network are allowed to be present simultaneously in the Allowed NSSAI or that the serving network is known to apply SLA logic to limit what S-NSSAIs are included in the Allowed NSSAI. The AMF/NSSF performs additional logic

prior to providing the Configured NSSAI, Allowed NSSAI, etc. to the UE

A supporting AMF/NSSF, when it receives a Requested NSSAI, checks the SRG-compatibility of the S-NSSAIs by evaluating the S-NSSAIs of the HPLMN (in the mapping information of the Requested NSSAI, when a mapping information is applicable) based on received SRG values for these S-NSSAIs in the subscription information.  The supporting AMF/NSSF determines the Allowed NSSAI so that all S-NSSAIs of the HPLMN share at least one common SRG value, if SRGs were provided as part of the subscription information.

An AMF/NSSF not supporting SRG information applies the Network Slice selection and provides the Configured NSSAI, Allowed NSSAI to the UE based on the SLA between the serving network and the HPLMN and the received Requested NSSAI and Subscribed S-NSSAIs.

#### 5.15.x.2 UE and UE configuration aspects

A UE that supports the SRG enabled subscription to simultaneous registration of network slice feature indicates its support in the Registration Request message in the Initial Registration and the Mobility Registration Update. The AMF stores in the UE context if the UE supports the feature.

When a serving PLMN provides the Configured NSSAI to the UE, and the UE supports the subscription-based restrictions to simultaneous registration of network slices feature, the AMF/NSSF also provides the UE with the SRG information related to the S-NSSAIs of the configured S-NSSAIs. A supporting UE which receives the SRG values in the network slicing configuration information shall only include SRG-compatible S-NSSAIs in the Requested NSSAI.

If the HPLMN changes SRG information in the subscription information for a UE, the UDM updates the AMF serving the UE with the new SRG information and the AMF updates the UE network slicing configuration by means of the UE Configuration Update procedure (this can include changes in the Configured NSSAI (and related mapping information) and changes in the Allowed NSSAI as applicable). The UE acknowledges this UE Configuration Update as described in TS 23.502 [3], clause 4.2.4.2.

An AMF which supports the subscription-based restrictions to simultaneous registration of network slice feature shall configure a non-supporting UE with a Configured NSSAI based on the Subscribed S-NSSAIs received as part of UE subscription information as described in clause 5.15.4.1.1.

When an AMF receives a Requested NSSAI including S-NSSAIs that are supported in the Tracking Area but are not  SRG-compatible from a UE that indicated support for the feature, the AMF assumes the UE configuration is out of date. In this case, the AMF provides the UE with an Allowed NSSAI including only the Default S-NSSAIs and with an updated configuration including, the up to date  SRG values for the S-NSSAIs in the Configured NSSAI.

****************** END CHANGES ***************