SA WG2 Meeting #145E (e-meeting)	S2-2103830

# May 17 – 28, 2021, Electronic, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0298 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve ENs for clause 5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Regarding “Editor's note: How the selected NWDAF and / or NRF is aware of the tracking area where the UE is located is FFS”, since the TA where the UE is located is only known by the AMF serving the UE, it should be allowed that the NWDAF finds the AMF serving the UE and then request UE location from the AMF. When the NWDAF get the TA that UE is located, it can use it as a filter for NWDAF discovery. So the EN is replaced by a Note to clarify this. Regarding “Editor's note: If multiple NWDAFs are serving the same tracking area, it is FFS to determine which one should be selected”, it is a normal procedure for a consumer to determine a target NWDAF from the candidate NWDAFs returned by NRF, nothing special here. So we propose to delete the EN directly. Regarding “Editor's note: It is FFS if other mechanism can be used to determine the target NWDAF, e.g. other parameters in the query to NRF.” from our perpective, using the service area of NF to be contacted to make a query would be the most efficient way, we don’t see other possible parameters for NRF query, and then propose to remove this EN. Additionally, according to the agreement in the last meeting, a NWDAF consumer may query UDM to obtain the NWDAF serving the UE, it is proposed to capture it as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. removing some ENs 2. clarify how to discover target NWDAFs if the selected NWDAF can not provide the required analytics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The issues indicated by the ENs still exist |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First change * * *

5.2	NWDAF Discovery and Selection

The NWDAF service consumer selects an NWDAF that supports requested analytics information and required analytics capabilities and/or requested Machine Learning (ML) model information by using the NWDAF discovery principles defined in clause 6.3.13, TS 23.501 [2].

If the required analytics is related to NF(s) (e.g. NF load analytics) and from the NWDAF discovery response it is not clear which NWDAF can provide such analytics, the NWDAF service consumer should select an NWDAF with large serving area from the candidate NWDAFs indicated in the discovery response, preferably with analytics aggregation capability and the largest serving area. If the selected NWDAF cannot provide the requested data analytics, e.g. due to the NF(s) to be contacted being out of serving area of the NWDAF, the selected NWDAF queries the NRF with the service area of the NF to be contacted to determine another target NWDAF.

If the required analytics is related to UE(s) (e.g. UE mobility analytics), the NWDAF service consumer can:

-	query UDM to discover the target NWDAF serving the UE if it supports; or

-	query NRF to discover the target NWDAF to serve the UE.

When querying NRF to discover the target NWDAF to serve the UE, if the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, it should select an NWDAF with large serving area from the candidate NWDAFs indicated in the discovery response, preferably with analytics aggregation capability and the largest serving area.

If the selected NWDAF cannot provide analytics for the UE(s) (e.g, the NWDAF serves a different serving area), the selected NWDAF determines the AMF serving the UE as specified in the clause 6.2.2.1, requests UE location information from the AMF, and queries the NRF with the tracking area where the UE is located to discover another target NWDAF serving the area where the UE(s) is located.

* * * End of changes * * *