3GPP TSG-SA WG2 Meeting #144E e-meeting	S2-2104040

# Elbonia, 12-16 April, 2021														  (revision of S2-2103478)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2838 | rev | 2 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS23.501 KI#4 NSACF event notification definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC, Apple, Nokia, Nokia Shanghai Bell, LG Electronics, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-05-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This contribution proposes a new feature addition to TS23.501 related to eNS_Ph2 KI#4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New eNS_Ph2 feature addition for KI#4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | KI#4 conclusion is not implemented in TS 23.501 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7; 5.15.11; 5.15.11.x (new); 5.31.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 23.502 CR 2715 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | On top of S2-2103478 agreed at SA2#144E; fixing the reference point number to avoid the numbering conflict |  |  |  |  |  |  |  |  |

* * * * start of 1st change * * * *

4.2.7	Reference points

The 5G System Architecture contains the following reference points:

N1:	Reference point between the UE and the AMF.

N2:	Reference point between the (R)AN and the AMF.

N3:	Reference point between the (R)AN and the UPF.

N4:	Reference point between the SMF and the UPF.

N6:	Reference point between the UPF and a Data Network.

NOTE 1:	The traffic forwarding details of N6 between a UPF acting as an uplink classifier and a local data network are not specified in this Release of the specification.

N9:	Reference point between two UPFs.

The following reference points show the interactions that exist between the NF services in the NFs. These reference points are realized by corresponding NF service-based interfaces and by specifying the identified consumer and producer NF service as well as their interaction in order to realize a particular system procedure.

N5:	Reference point between the PCF and an AF.

N7:	Reference point between the SMF and the PCF.

N8:	Reference point between the UDM and the AMF.

N10:	Reference point between the UDM and the SMF.

N11:	Reference point between the AMF and the SMF.

N12:	Reference point between AMF and AUSF.

N13:	Reference point between the UDM and Authentication Server function the AUSF.

N14:	Reference point between two AMFs.

N15:	Reference point between the PCF and the AMF in the case of non-roaming scenario, PCF in the visited network and AMF in the case of roaming scenario.

N16:	Reference point between two SMFs, (in roaming case between SMF in the visited network and the SMF in the home network).

N16a:	Reference point between SMF and I-SMF.

N17:	Reference point between AMF and 5G-EIR.

N18:	Reference point between any NF and UDSF.

N19:	Reference point between two PSA UPFs for 5G LAN-type service.

N22:	Reference point between AMF and NSSF.

N23:	Reference point between PCF and NWDAF.

N24:	Reference point between the PCF in the visited network and the PCF in the home network.

N27:	Reference point between NRF in the visited network and the NRF in the home network.

N28:	Reference point between PCF and CHF.

N29:	Reference point between NEF and SMF.

N30:	Reference point between PCF and NEF.

NOTE 2:	The functionality of N28 and N29 and N30 reference points are defined in TS 23.503 [45].

N31:	Reference point between the NSSF in the visited network and the NSSF in the home network.

NOTE 2:	In some cases, a couple of NFs may need to be associated with each other to serve a UE.

N32:	Reference point between SEPP in the visited network and the SEPP in the home network.

NOTE 5:	The functionality of N32 reference point is defined in TS 33.501 [29].

N33:	Reference point between NEF and AF.

N34:	Reference point between NSSF and NWDAF.

N35:	Reference point between UDM and UDR.

N36:	Reference point between PCF and UDR.

N37:	Reference point between NEF and UDR.

N38:	Reference point between I-SMFs.

N40:	Reference point between SMF and the CHF.

N41:	Reference point between AMF and CHF in HPLMN.

N42:	Reference point between AMF and CHF in VPLMN.

NOTE 3:	The reference points from N40 up to and including N49 are reserved for allocation and definition in TS 23.503 [45].

N50:	Reference point between AMF and the CBCF.

N51:	Reference point between AMF and NEF.

N52:	Reference point between NEF and UDM.

N55:	Reference point between AMF and the UCMF.

N56:	Reference point between NEF and the UCMF.

N57:	Reference point between AF and the UCMF.

NOTE 4:	The Public Warning System functionality of N50 reference point is defined in TS 23.041 [46].

N58:	Reference point between AMF and the NSSAAF.

N59:	Reference point between UDM and the NSSAAF.

N60:	Reference point between AMF and NSACF.

N82:	Reference point between NSACF and NEF.

The reference points to support SMS over NAS are listed in clause 4.4.2.2.

The reference points to support Location Services are listed in TS 23.273 [87].

The reference points to support SBA in IMS (N5, N70 and N71) are described in TS 23.228 [15].

The reference points to support AKMA (N61, N62 and N63) are described in TS 33.535 [124].

The reference points to support ProSe (N64, N65 and N66) are described in TS 23.304 [128].

* * * * end of 1st change * * * *

* * * * start of the 2nd change * * * *

5.15.11	Network Slice Admission Control Function

The Network Slice Admission Control Function (NSACF) monitors and controls the number of registered UEs per network slice for the network slices that are subject to Network Slice Admission Control (NSAC).

The NSACF also provides event based Network Slice status notifications and reports to the consumer NF (e.g. AF).

The NSACF is configured with the maximum number of UEs per network slice which are allowed to be served by each network slice that is subject to NSAC.

* * * * end of 2nd  change * * * *

* * * * start of 3rd change * * * *

5.15.11.x	Network Slice status notifications and reports to a consumer NF

A consumer NF (e.g. AF) can subscribe with the NSACF for Network Slice status notifications and reports. Upon such subscription, the NSACF can provide event based notifications and reports to the consumer NF (e.g. to AF via NEF) related to the current number of UEs registered for a network slice or the current number of PDU Sessions established on a network slice or both.

* * * * end of 3rd change * * * *

* * * * start of 4th change * * * *

5.31.9	Support for Monitoring Events

The Monitoring Events feature is intended for monitoring of specific events in the 3GPP system and reporting such Monitoring Events via the NEF. The feature allows NFs in 5GS to be configured to detect specific events and report the events to the requested party. Clause 5.20 further discusses the Monitoring capabilities of the NEF.

For CIoT, the list of supported monitoring events is specified in Table 4.15.3.1-1 of TS 23.502 [3].

Support for Monitoring Events can be offered via AMF, UDM, NSACF and SMF, and can be reported via the NEF, as specified in clause 4.15.3 of TS 23.502 [3].

* * * * end of 4th change * * * *