3GPP TSG-WG SA2 Meeting #145E 	S2-2104104

# 17th May – 28th May 2021, Elbonia	(revision of S2-210xxxx)

Source:	Nokia, Nokia Shanghai Bell

Title:	TS 23.256:		Updates to procedure for UAV location reporting

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	ID_UAS / Rel-17

Abstract of the contribution: This contribution proposes some corrections in UAV location reporting procedure

# 1 Introduction

This pCR proposes some correcctions to the "UAV location reporting" procedure documented in clause 5.3.2 of TS 23.256.

1.	Currently in step 4 of Figure 5.3.2-1 of TS 23.256, it is mentioned that Nnef_EventExposure_Subscribe response is sent. But as per the 5GC-MT-LR procedure step 24b-2 in Figure 6.1.2-1 and deferred 5GC-MT-LR procedure step 21b-2 in Figure 6.3.1-1, this should be a Nnef_EventExposure_Notify operation. The Figure 5.3.2-1 and corresponding description is corrected to align with the procedure in TS 23.273.

2.	The clause 5.3.2 of TS 23.256 is about UAV location reporting. The procedure should not explain what does the USS/UTM do with the location information. Hence it is proposed to remove step 6 from the clause as it is not relevant.

3.	There are also few editorial corrections like using "UAS NF/NEF" which is aligned with other sections of the TS 23.256.

# 2 Proposal

It is proposed to approve the below content. All changes are marked with revision marks.

* * * * Begin of Changes * * * *

### 5.3.2	Procedure for UAV location reporting

The following procedures describe the 5GC UAV's location reporting service to USS.

Figure 5.3.2-1: UAV Location Reporting

1.	USS to UAS NF/NEF: The USS sends Nnef_EventExposure_Subscribe request to the UAS NF/NEF as described in step 1b-1 from clause 6.1.2 of TS 23.273 [8] for immediate location reporting (i.e. 5GC-MT-LR) and step 1b-1 from clause 6.3.1 of TS 23.273 [8] for periodic, triggered and UE available location reporting (i.e. deferred 5GC-MT-LR).

2.	UAS NF/NEF determines the relevant NF, i.e. GMLC for location reporting based on the UAV's capability or network capability, location accuracy etc.

3.	UAS NF/NEF sends request to GMLC with the GPSI (i.e. 3GPP UAV ID) provided by USS to retrieve the UAV location via the current location services supported by GMLC. The UAS NF/NEF performs 5GC-MT-LR Procedure as described in clause 6.1.2 of TS 23.273 [8] or deferred 5GC-MT-LR procedure as described in clause 6.3.1 (up to step 21b-1) of TS 23.273 [8] depending on whether the request received in step 1 was for immediate location reporting or deferred location reporting respectively.

4.	UAS NF/NEF to USS: UAS NF/NEF provides the UAV location to USS/TPAE in Nnef_EventExposure_Notify operation as described in step 24b-2 of Figure 6.1.2-1, if the request in step 1 was for immediate location reporting. The UAS NF/NEF includes the GPSI in the location reporting message to USS/TPAE as well as the UAV's location information (in the form of geo co-ordinates) which is understood by USS/TPAE (not assuming the knowledge of TA and Cell Id).

If the request in step 1 was for deferred 5GC-MT-LR,  the UAS NF/NEF sends Nnef_EventExposure_Notify indicating whether or not the periodic or triggered location was successfully activated in the target UE, as described in step 21b-2 of Figure 6.3.1-1.

5.	For deferred 5GC-MT-LR with periodic or triggered location request steps 22 to 30b-2 of Figure 6.3.1-1 are executed and the UAS NF/NEF provides the location report to USS/TPAE in Nnef_EventExposure_Notify operation.

* * * * End of Changes * * * *