SA WG2 Meeting #145E	S2-2104210

17 – 28 May, 2021, Electronic meeting

Source:	Samsung

Title:	update of MBS session configuration

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract: This document adds the MBS session update configuration procedure, which incorporate broadcast session update procedure including PCF-related steps.

- Introduction

This document provides the MBS session update configuration procedure, which incorporate broadcast session update procedure including PCF-related steps.

- Proposal

It is proposed to capture the following changes in TS23.247.

* * * * Start of 1st Change( all new text) * * * *

7.1.1.3	update of MBS session configuration

The update of configuration steps for MBS Session are used by the AF to modify the MBS Session towards 5GC e.g. MBS QoS, etc, which the MBS session modification procedure may follow the update of configuration procedure to modify the reserved resources toward NG-RAN.

Figure 7.1.1.3-1: Update of configuration for MBS Session

Editor´s note:	MBSTF could optionally be in the user plane and the call flow should be updated and aligned with the MBS session configuration procedure.

Editor's note:	The services and messages used in this procedure are FFS.

- AF of content provider may request update contents for the MBS session (TMGI) to NEF.

If dynamic PCC is deployed, the steps 6,7 and 8 are skipped, otherwise, the steps 2,3, 4 and 5 are skipped.

2.	The NEF/MBSF may decide to do application/transport layer treatment (e.g. adding FEC, MBS media transcoding) and change the MBS QoS Requirements from the AF. The NEF/MBSF sends the MB Session Update (MBS Session ID, MBS QoS Requirements, MBS service area) to the PCF. The NEF/MBSF may need to translate MBS service area as internal format(e.g. Cell IDs, TAIs) if it is provided in external format.

3/4. The PCF sends the MB Session Policy Association Update Request (MBS Session ID, 5G QoS Profile, MBS service area) to the MB-SMF.

5a/b. If there is service requirement update, MB-SMF sends the updated 5G QoS Profile to the MB-UPF.

6.	The NEF/MBSF sends MBS Session Update to the MB-SMF. The NEF/MBSF may to translate MBS service area as internal format (e.g. Cell IDs, TAIs) if it is provided in external format.

7a/b. If there is service requirement update, MB-SMF sends the updated 5G QoS Profile to the MB-UPF.

8.	MB-SMF responses to the NEF/MBSF.

9.	The NEF/MBSF responses to the AF.

* * * * End of Changes * * * *