SA WG2 Meeting #145E	S2-2104222

17 – 28 May, 2021, Electronic meeting

Source:	Samsung

Title:	Update on initial MBS session configuration

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract: This document updates the initial MBS session configuration procedure to incorporate broadcast session establishment procedure including PCF-related steps.

- Introduction

This document modifies the initial MBS session configuration procedure to incorporate broadcast session establishment procedure including PCF-related steps.

- Proposal

It is proposed to capture the following changes in TS23.247.

* * * * Start of 1st Change * * * *

7.1.1	Configuration for MBS Session

Editor's note:	The heading and content needs to be revisited.

7.1.1.1	Initial MBS session configuration

The configuration steps for MBS Session are used by the AF to start the MBS Session towards 5GC and consist of TMGI allocation and MBS session start procedures, and they apply to both multicast and broadcast communications unless otherwise stated. MBS session establishment/activation procedure may follow the MBS session configuration procedure per its service type (multicast or broadcast service) to reserve resources towards NG-RAN.

Figure 7.1.1.1-1: Configuration for MBS Session

Editor´s note:	MBSTF could optionally be in the user plane and the call flow should be updated to also cover that possibility.

Editor's note:	The services and messages used in this procedure are FFS.

Steps 1 to 5 are optional and only applicable if TMGI is used as MBS Session ID and required to be pre-allocated.

1.	AF sends Allocate TMGI Request () message to NEF/MBSF to request allocation of a TMGI to identify a new MBS session.

NOTE 1:	Depending on the configuration, MB-SMF may receive requests from AF directly, or via NEF, or via MBSF, or via NEF and MBSF.

2. AF checks authorization of AF.

3.	NEF/MBSF discovers and selects an MB-SMF using NRF or based on local configuration, and then sends an Allocate TMGI Request () message to the MB-SMF.

4.	MB-SMF allocates a TMGI and returns the TMGI to the NEF/MBSF.

5.	The NEF or MBSF responds to the AF by sending an Allocate TMGI Response (TMGI) message.

6.	The AF may perform a Service Announcement towards UEs. The AF informs UEs about MBS Session information with MBS Session ID, e.g., TMGI, source specific multicast address, and possibly other information e.g., MBS service area, session description information, etc.

The MBS service area information can be Cell ID list, TAI list, geographical area information or civic address information. Amongst them, Cell ID list and TAI list shall only be used by AFs who reside in trust domain, and when the AFs are aware of such information.

The UE needs to be aware if the service is broadcast or multicast to decide if JOIN is to be performed.

Editor's note:	How to do service announcements requires SA WG4 /WG6 coordination.

7.	AF of content provider may provide contents for an MBS session (possibly providing information for a previously allocated TMGI; e.g. service type of either multicast service or broadcast service) to NEF. If step 1-4 has not been executed before, the AF may provide a source specific multicast address or it may request that the network allocates an identifier for the MBS session (i.e., TMGI) and its service type of either multicast service or broadcast service. MBS information may further include QoS requirements, UE authorization information (e.g. a GPSI or an External Group Id or a UE ID to identify UEs authorized to join the multicast service), MBS service area (see step 6 for detail) identifying the service scope.

If geographical area information or civic address information was provided by the AF as MBS service area, NEF/MBSF translates the MBS service area to Cell ID list or TAI list.

Editor's note:	What other information is to be sent by AF is FFS.

NEF/MBSF checks authorization of content provider.

If dynamic PCC is deployed, the steps 8, 12,13 and 14 are executed.

8.	The NEF or MBSF checks if the input parameters e.g. HL MC address are valid. The MBSF may decide to do application/transport layer treatment (e.g. adding FEC, MBS media transcoding) and change the MBS QoS Requirements from the AF. The NEF/MBSF sends an MBS Session Start (MBS session ID, Service type, Service Requirement, MBS service area, etc) message to the PCF.

9.	NEF/MBSF discovers MB-SMF candidates and selects MB-SMF as ingress control node, possibly based on location area.

10.	NEF/MBSF requests MB-SMF to reserve ingress resources for a MBS distribution session and provides MBS Session ID or request allocation, and indicate its service type (either multicast service or broadcast service), so that the MB-SMF determines whether it starts MBS session establishment for broadcast or waits for MBS session join for multicast. It also indicates if the allocation of an ingress transport address is requested.

The MBS service area is provided by NEF/MBSF to the MB-SMF if provided by the AF in step 7.

11.	MB-SMF updates NF profile to NRF with the serving MBS Session ID.

12.	[Optional] The MB-SMF sends SM MBS Policy Association Request to MB-PCF with the MBS Session ID, AF Identifier, and the QoS requirements.

13.	[Optional] The MB-PCF registers at the BSF that it handles the multicast session. It provides an identifier that the policy association is for multicast and the MBS Session ID, it own PCF ID and optionally its PCF set ID.

14.	[Optional] The MB-PCF responds with SM MBS Policy Association Response with policies for the MBS Session ID.

In addition, determines whether the request is authorized.

If the request is authorized, the PCF derives the required QoS parameters based on the information provided by the NEF and determines whether this QoS is allowed (according to the PCF configuration for this Application), and notifies the result to the MB-SMF. The PCF notifies the MB-SMF whether the transmission resources corresponding to the QoS request are established or not.

If the request is not authorized, the required QoS is not allowed, or transmission resources are not established, MB-SMF responds to the NEF in step 16 with a Result value indicating the failure cause, and NEF further notifies AF in step 17.

15.	MB-SMF selects the MB-UPF and requests it to reserve user plane ingress resources. If multicast transport of the MBS data towards RAN nodes is to be used, the MB-SMF also request the MB-UPF to reserve for the outgoing data a tunnel endpoint and the related identifiers (source IP address, source specific multicast address and GTP Tunnel ID) and to forward data received at the user plane ingress resource using that tunnel endpoint.

If ingress address is not requested, the MB-SMF configure MB-UPF to handle the multicast data distribution and request the MB-UPF to join the multicast tree towards the content provider. MB-UPF can also join the distribution tree of the content provider in the subsequent procedures e.g., session establishment procedure.

16.	If requested, MB-UPF selects an ingress address (IP address and port) and a tunnel endpoint for the outgoing data and provides it to MB-SMF.

17.	MB-SMF indicates the possibly allocated ingress address to the NEF/MBSF. MB-SMF may include TMGI if it is allocated in step 9. It also indicates the success or failure of reserving transmission resources.

18.	The NEF/MBSF-C indicates the possibly allocated ingress address and other parameters (e.g. TMGI) to the AF.

19.	Same as step 6. The AF may also perform a service announcement at this stage.

* * * * End of Changes * * * *