3GPP TSG-WG SA2 Meeting #145E e-meeting  	S2-2104248

# Elbonia, May 17 – 28, 2021	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2792 | rev |  | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify the BDT warning procedure with degraded Network performance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The step 2 in the procedure for BDT warning notification in clause 4.16.7.3, TS 23.502 is as follows “2. The PCF is notified when the network performance in the area of interest goes below the criteria set by the operator from the NWDAF as described for the Network Performance analytics in TS 23.288 [50].” It gives people an impression that NWDAF will provide a "network performance is degraded" indication to the PCF when the NWDAF determines the network performance is degraded based on a criteria in the NWDAF itself set by the operator. However, if it looks at the Network Performance Analytics in clause 6.7.3 of TS 23.288, there is not any descriptions about the NW performance degraded in the Network Performance analytics. The correct understanding is that the NWDAF will provide the degraded network performance analytics when the NWDAF determines the network performance goes below the criteria (i.e. the Reporting Threshold) set by the PCF in the Analytics Reporting Information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the PCF is notified the network performance analytics in the area of interest from the NWDAF when the NWDAF determines the network performance goes below the criteria (i.e. the Reporting Threshold) set by the PCF in the Analytics Reporting Information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current descriptions is not aligned with the Network Performance Analytics in clause 6.7.3 of TS 23.288. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.16.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.16.7.3	Procedure for BDT warning notification

Figure 4.16.7.3-1: The procedure for BDT warning notification

1.	The negotiation for Background Data Transfer (BDT) described in clause 4.16.7.2 is completed. In addition, the PCF has subscribed to analytics on "Network Performance" from NWDAF for the area of interest and time window of a background data transfer policy following the procedure and services described in TS 23.288 [50] using the Reporting Threshold in the Analytics Reporting information set by the PCF to reflect the criteria set by the operator.

2.	The PCF is notified with the network performance analytics in the area of interest from the NWDAF when the NWDAF determines the network performance goes below the criteria as described for the Network Performance analytics in TS 23.288 [50].

3.	The H-PCF may request from the UDR the stored BDT policies using Nudr_DM_Query (Policy Data, Background Data Transfer) service operation.

4.	The UDR provides all the Background Transfer Policies together with the relevant information received from the AF (as defined in clause 6.1.2.4, TS 23.503 [20]) to the H-PCF.

5.	The H-PCF identifies the BDT policies affected by the notification received from NWDAF. For each of them, the H-PCF determines the ASP of which the background traffic will be influenced by the degradation of network performance and which requested the H-PCF to send the notification. The PCF then performs the following steps for each of the determined ASPs, i.e. Steps 6 - 16 can occur multiple times (i.e. once per ASP).

6.	The PCF decides based on operator policies, whether a new list of candidate Background Data Transfer policies can be calculated for the ASP. If the PCF does not find any new candidate BDT policy, the previously negotiated BDT policy shall be kept, no interaction with that ASP shall occur and the procedure stops for that BDT policy.

NOTE 1:	The BDT policies of an ASP which did not request to be notified are kept and no interaction with this ASP occurs.

7.	The PCF sets the no longer valid BDT policy in the UDR as invalidated by invoking Nudr_DM_Update (Background Data Transfer Reference ID, invalidation flag) service.

NOTE 2:	The BDT policies that are applicable for future sessions are checked by the PCF in step 6.

8.	The UDR sends a response to the H-PCF as acknowledgement.

9.	The PCF sends the notification to the NEF by invoking Npcf_PolicyControl_Notify (Background Data Transfer Reference ID, list of candidate Background Data Transfer policies) service operation.

10.	The NEF sends the BDT warning notification to the AF by invoking Nnef_BDTPNegotiation_Notify (Background Data Transfer Reference ID, list of candidate Background Data Transfer policies) service operation.

11.	The AF checks the new Background Data Transfer policies included in the candidate list in the BDT warning notification.

12.	If the AF selects any of the new Background Data Transfer policies, the steps 8-13 from clause 4.16.7.2 are executed.

13.	If the AF doesn't select any of the new Background Data Transfer policies, the steps 8-11 from clause 4.16.7.2 are executed, with the AF indicating that none of the candidate Background Data Transfer policies is acceptable.

14.-15.	If the step 13 is executed, the PCF removes the no longer valid BDT policy from UDR for the corresponding Background Data Transfer Reference ID.

NOTE 3:	The PCF can also remove the no longer valid BDT policy after an operator configurable time for the case that the AF does not respond.

16.	If there is a new Background Data Transfer policy stored in the UDR or a BDT policy removed from the UDR, the PCFs are notified by the UDR accordingly. The PCFs check if the corresponding URSP rules need to be updated or removed and if so, use the procedure defined in clause 4.16.12.2 to update URSP rules for the relevant UEs.

The AF can send a Stop notification by invoking Nnef_BDTPNegotiation_Update service, when the AF requests not to receive the BDT warning notification anymore. Then, the NEF invokes Npcf_BDTPolicyControl_Update service in order to provide this information for the H-PCF.

# * * * * End of changes * * * *