SA WG2 Meeting #S2-145E	S2-2104259

May 17 - 28, 2021, Electronic, Elbonia	(revision of S2-2103246)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0280 | rev | 02 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving EN on analytics metadata request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is an editor's note for contents of analytics exposure in current specification: Editor's note: Whether the "analytics metadata request" is a flag or it allows to request specific sub-parameters of the "analytics metadata information" is FFS. This needs to be resolved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Propose further clarification on the "analytics metadata request"and "analytics metadata information". Rev02: Add "analytics metadata type" parameter to indicated the requested sub-parameters in "analytics metadata information". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The request and provision of "analytics metadata information" is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3 |  |  |  |  |  |  |  |  |
| “ |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************** 1st Change ********************

### 6.1.3	Contents of Analytics Exposure

The consumers of the Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operations described in clause 7 provide the following input parameters listed below.

-	A list of Analytics ID(s): identifies the requested analytics.

-	Analytics Filter Information: indicates the conditions to be fulfilled for reporting Analytics Information. This set of optional parameter types and values enables to select which type of analytics information is requested. Analytics Filter Information are defined in the analytics related procedures.

-	Target of Analytics Reporting: indicates the object(s) for which Analytics information is requested, entities such as specific UEs, a group of UE(s) or any UE (i.e. all UEs).

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from NWDAF with this subscription.

-	Analytics Reporting Information with the following parameters:

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Analytics Reporting Parameters as per Event Reporting parameters defined in Table 4.15.1-1, TS 23.502 [3].

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Reporting Thresholds, which indicate conditions on the level of each requested analytics that when reached shall be notified by the NWDAF. A matching direction may be provided such as below, above, or crossed. If no matching direction is provided, the default direction is crossed.

-	Analytics target period: time interval [start..end], either in the past (both start time and end time in the past) or in the future (both start time and end time in the future). An Analytics target period in the past is a request or subscription for statistics. An Analytics target period in the future is a request or subscription for predictions. The time interval is expressed with actual start time and actual end time (e.g. via UTC time). When the Analytics Reporting Parameters indicate a periodic reporting mode, the time interval can also be expressed as positive or negative offsets to the reporting time, which indicates a subscription for predictions or statistics respectively. By setting start time and end time to the same value, the consumer of the analytics can request analytics or subscribe to analytics for a specific time rather than for a time interval.

-	[OPTIONAL] Data time window: if specified, only events that have been created in the specified time interval are considered for the analytics generation.

-	Preferred level of accuracy of the analytics ("Low", "Medium", "High" or "Highest").

-	[OPTIONAL] Accuracy level per analytics subset ("Low", "Medium", "High" or "Highest"). When an accuracy level is expressed for a given analytics subset, it takes precedence for this subset over the above preferred level of accuracy. Analytics subsets are defined in the "Output Analytics" clause of applicable analytics.

-	[OPTIONAL] Dataset Statistical Properties: NWDAF may use the dataset statistical properties information in order to influence the data selection mechanisms to be used for the generation of an analytics ID, assuring that the generated analytics ID reflects the statistical characteristics of the data that are relevant for the NWDAF consumer in all different NWDAFs.

Editor's note:	The specific parameters that are part of the "dataset statistical properties" are FFS.

-	Time when analytics information is needed (if applicable): indicates to the NWDAF the latest time the analytics consumer expects to receive analytics data provided by the NWDAF. If the time is reached the consumer does not need to wait for the analytics information any longer, yet the NWDAF may send an error response or error notification to the consumer. "Time when analytics information is needed" is a relative time interval as the gap with respect to analytics request /subscription (e.g. "in 10 minutes").

-	[OPTIONAL] Maximum number of objects requested by the consumer (max) to limit the number of objects in a list of analytics per Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response.

-	[OPTIONAL] Preferred order of results when a list of analytics is returned, possibly with a criterion for identifying the property of the results to which the preferred ordering is applied.

-	[OPTIONAL] Maximum number of SUPIs (SUPImax) requested by the consumer to limit the number of SUPIs in an object. When SUPImax is not provided, the NWDAF shall return all SUPIs concerned by the analytics object. When SUPImax is set to 0, the NWDAF shall not provide any SUPI.

-	[OPTIONAL] Output strategy: indicates the output strategy for the analytics reporting. The following values are allowed:

-	Binary output strategy: if preferred level of accuracy is more important than providing an output, then the binary strategy is used such that all analytics outputs have equivalent confidence in the prediction.

-	Gradient output strategy: if having an analytics output is more important than reaching the preferred level of accuracy, then the gradient output strategy is used such that each NWDAF will timely provide the output indicating the level of accuracy at the moment of the output generation.

-	[OPTIONAL] Analytics metadata request: indicates a request from one NWDAF to another NWDAF to provide the "Analytics metadata information" related to the produced output analytics. This input parameter indicates if analytics metadata information is required to aggregate the output analytics for the requested Analytics ID(s).

-	[OPTIONAL] Analytics metadata type: indicates what sub-parameter(s), e.g. Number of data samples, Dataset Statistical Properties, in "analytics metadata information" are requested. This input parameter is only used when "Analytics metadata request" indicates analytics metadata information is required.

The NWDAF provides to the consumer of the Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request service operations described in clause 7, the output information listed below:

-	(Only for Nnwdaf_AnalyticsSubscription_Notify) The Notification Correlation Information.

-	For each Analytics ID the analytics information in the requested Analytics target period.

-	In addition, the following additional information:

-	Timestamp of analytics generation, which allows consumers to decide until when the received information shall be used. For instance, an NF can deem a received notification from NWDAF for a given feedback as invalid based on this timestamp;

-	Validity period, which defines the time period for which the analytics information is valid.

-	Probability assertion: confidence in prediction.

-	(Only when analytics aggregation is required and Gradient Output Strategy is used) Level of accuracy reached for the specific notification for a cycle of the periodicity or for the request.

Editor's note:	It is FFS whether the "level of accuracy" is also to be provided for other cases.

-	[OPTIONAL] Analytics metadata information: additional information required to aggregate the output analytics for the requested Analytics ID(s). This parameter shall be provided if the "Analytics metadata request" parameter was provided in the corresponding Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operation.

-	number of data samples used for the generation of the output analytics,

-	time window of the data samples,

-	Dataset Statistical Properties.

Editor's note:	The specific parameters that are part of the "dataset statistical properties" are FFS.

Editor's note:	The specific parameters that are part of the "analytics metadata information" are FFS. Whether "dataset statistical properties" and "time window" are part of the analytics metadata information is FFS.

-	(Only for error response or error notification) Revised waiting time: indicates to the consumer a revised waiting value for "Time when analytics information is needed". Each NWDAF may include this as part of error response or error notification to "Time when analytics information is needed" as described in clause 6.2.5. Revised waiting time is the minimum time interval recommended by NWDAF to use as "Time when analytics information is needed" for similar future analytics requests/subscriptions.

Editor's note:	The specification of "error response" and "error notification" is FFS, e.g. whether the other parameters of the output information are optional / not included in such error response/notification.

******************** End of Change ********************