3GPP TSG-WG SA2 Meeting #145E e-meeting	S2-2104266

Elbonia, May 17 – 28, 2021	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	Identifiers for Layer-2 UE-to-Network Relay

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	5G_ProSe / Rel-17

Abstract: This contribution proposes to add identifiers for Layer-2 UE-to-Network Relay.

# 1. Introduction/Discussion

It is agreed in last meeting that the RSC can indicate if the UE-to-Network Relay is a Layer-3 or Layer-2 UE-to-Network Relay. For Layer-3 UE-to-Network Relay, RSC contained in the discovery message is associated with PDU session parameters. For Layer-2 UE-to-Network Relay, there is no need to use RSC to differentiate the services and a Remote UE can get the PLMN ID of the UE-to-Network Relay during the Relay discovery process based on clause 6.7.2.3 of TR 23.752 and RAN2 agreement, so there is also no need to associate different RSCs with different PLMNs.

Proposal 1: No information needs to be associated with the RSC for Layer-2 UE-to-Network Relay, and the RSC only indicates the Layer-2 UE-to-Network Relay service.

In addition, RAN2 agreements on Relay (re)selection part include

Proposal 16: Include the information required for agreed additional AS criteria in discovery message.

Proposal 2-2 [easy]: For L2 relay, PLMN ID supported as additional AS criteria for relay (re)selection.  Whether cell ID is used can be further discussed by RAN2.

Proposal 2: PLMN ID of the UE-to-Network Relay can be provided to Remote UE during the discovery. The other identifiers requires cooperation with RAN2.

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.304.

# * * * * First change * * * *

#### 5.9.3.3	Identifiers for Layer-2 UE-to-Network Relay

Editor’s note:	Any identifiers that are used in addition to the identifiers specified in 5.9.3.1 for Layer-2 UE-to-Network Relay requires cooperation with RAN2.

The following parameters are used in addition to the parameters specified in 5.9.3.1, for both Model A and Model B, to assist the UE-to-Network Relay (re)selection:

-	Serving PLMN ID of the UE-to-Network Relay.

# * * * * End of changes * * * *