SA WG2 Meeting #145E (e-meeting)	S2-2104280

May 17 – 28, 2021, Elbonia

Source:	CATT

Title:	MBS Session and Service Context

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract of the contribution: This contribution proposes the update to the description of MBS Session and Service Context.

- Introduction

In clause 6.9.1 of TS 23.247, MBS Session/Service Context is described. This contribution proposes some updates to the description.

# 2. Proposal

It is proposed to include the following changes in TS 23.247.

******************** 1st Change ********************

### 6.9.1	MBS Session/Service Context

The MBS Session Context contains all information describing a particular MBS session in the 5GS and is created in each node involved in the delivery of the MBS data.

The content of the MBS Multicast Session Context is described in Table 6.9.1-1.

Table 6.9.1-1: Multicast MBS Session context

| Parameter | Description | NG-RAN | MB-SMF | SMF | AMF |
| --- | --- | --- | --- | --- | --- |
| MBS Session type | broadcast or multicast session. Here is “multicast session”. | X | X |  | X |
| State | State of MBS session 'Configured multicast session' |  | X |  |  |
| State | State of MBS session ('Active multicast session' or 'Inactive multicast session') | X | X |  |  |
| SSM (source specific IP multicast address) | IP multicast address identifying the MBS session. |  | X (note 1) | X (note 1) | X (note 1) |
| TMGI | Temporary Mobile Group Identity allocated to the MBS Session. | X | X | X | X |
| Area Session Identifier | Used for MBS session with location dependent content. When present, the Area Session Identifier together with the TMGI uniquely identify the MBS Session in a specific MBS service area. | X (note 1) | X (note 1) | X (note 1) | X (note 1) |
| MB-SMF | The MB-SMF that handles the MBS session. |  |  | X | X |
| QoS information | QoS information of the MBS session. | X | X | X |  |
| MBS Service Area | Area over which the MBS session data is distributed. | X (note 1) | X (note 1) | X (note 1) | X (note 1) |
| UEs in multicast session | UEs hosted by the node that have joined the multicast session. | X |  | X |  |
| NG-RAN Node ID(s) | NG-RAN node(s) serving the UEs which joined in the multicast session |  |  |  | X |
| IP multicast and source address for data distribution | IP addresses identifying the SSM user plane transport for shared delivery between MB-UPF and NG-RAN | X (note 1) | X (note 1) | ) |  |
| IP unicast address for data distribution | IP addresses identifying the user plane transport for individual delivery between NG-RAN and UPF, and between the UPF and MB-UPF | X (note 1) | X (note 1) | X (note 1) |  |
| NOTE 1: It is an optional parameter. |  |  |  |  |  |

In Broadcast MBMS mode, an MBS Session Context is created in the NG-RAN, AMF, MB-SMF and MBSF as a result of the MBS Session Start procedure.

The content of the Broadcast MBS Session Context is described in Table 6.9.1-2.

Table 6.9.1-2: Broadcast MBS Session context

| Parameter | Description | NG-RAN | AMF | MB-SMF |
| --- | --- | --- | --- | --- |
| MBS Session type | broadcast or multicast session. Here is “broadcast session” | X | X | X |
| TMGI | Temporary Mobile Group Identity allocated to the MBS Session. | X | X | X |
| Area Session Identifier | Used for MBS session with location dependent content. When present, the Area Session Identifier together with the TMGI uniquely identify the MBS Session in a specific MBS service area. | X (note 1) | X (note 1) | X (note 1) |
| AMF | The AMF(s) which are selected for the MBS session |  |  | X |
| QoS information | QoS information for the MBS Session. | X |  | X |
| MBS Service Area | Area over which the MBS session data is distributed. | X | X | X |
| NG-RAN Node ID(s) | NG-RAN nodes which are selected for the broadcast session |  | X |  |
| IP multicast address for data distribution | IP addresses identifying the user plane transport between MB-UPF and NG-RAN when the IP multicast transport is used. | X (note 1) |  | X (note 1) |
| NG-RAN IP Address and TEID for User Plane | The IP address and TEID of NG-RAN used for the user plane between NG-RAN and MB-UPF when Point to Point tunnel is used. | X (note 1) |  | X (note 1) |
| List of cell ID(s) | Cell(s) for which the MBS service may be distributed. | X |  |  |
| NOTE 1: It is an optional parameter. |  |  |  |  |

******************** End of Change ********************