SA WG2 Meeting #145E (e-meeting)	S2-2104281

May 17 – 28, 2021, Elbonia

Source:	CATT

Title:	MBS Session state model

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract of the contribution: This contribution proposes the update to the description of MBS Session state model.

- Introduction

This contribution proposes some updates to the description of MBS Session state model in clause 4.3 of TS 23.247, to remove the EN, etc.

# 2. Proposal

It is proposed to include the following changes in TS 23.247.

******************** 1st Change ********************

4.3	Multicast session state model

The following states are defined for the multicast session:

-	Configured state: Multicast session is established in 5GC, i.e. information about the multicast session (e.g. QoS information) is configured in 5GC NFs (e.g. MB-SMF) serving the multicast session, but no User Plane resources towards NG-RAN are reserved and no multicast data is transmitted. Optionally, TMGIs can be allocated as the MBS Session ID. UEs are allowed to join (subject to authorization check and configuration), and the first accepted UE join request will trigger the multicast session establishment towards the NG-RAN and the UE.

NOTE 1:	The SMF is not involved in the multicast session while the multicast session  in configured state.

NOTE 2:	There may be several sub-states in the configured state, e.g. only TMGIs requested, or information about the multicast session provided.

-	Active state: Established multicast session is in active state. UEs are allowed to join the multicast session (subject to authorization check). 5GC resources for the multicast session are reserved for UEs that joined the multicast session. Corresponding radio resources are reserved depending on participating UE locations.  Multicast data can be transmitted to UEs that joined the multicast session.

-	Inactive state: Established multicast session in inactive state. No multicast data are transmitted. UEs that joined the multicast session may be in CM CONNECTED or CM IDLE state. UEs are allowed to join the multicast session (subject to authorization check).

The following procedures are defined which result in transition between the multicast session states:

-	Multicast Session Configuration: The AF provides information about the multicast session and optionally request the allocation of a TMGI. Alternatively, there is network-internal configuration of the multicast session. Only resources at MB-SMF, NEF and MB-UPF are reserved and no multicast data are transmitted. The configuration may indicate when a multicast session can become active. The AF may provide configuration in several steps, e.g. to first request TMGIs and then provide full information about the multicast session and allow it to be established. Multicast session state transitions from NULL to Configured state in 5GC NFs.

-	Multicast Session Join/Establishment: The UE initiates multicast session join towards the network. When the join request of the first UE for multicast session in the NG-RAN node is accepted, the multicast session is established towards the NG-RAN node and the UE. User Plane resources for the multicast session are allocated between 5GC and the NG-RAN. Multicast session state transitions from NULL or Configured state (in 5GC NFs) or NULL state (in UE and NG-RAN) to either Inactive or Active state, depending on whether there is multicast data to be delivered to the UE when the multicast session join is accepted.

-	Multicast Session Activation: Triggered by the 5GC, the radio resources for the multicast session are established and multicast session data starts to be transmitted to the UE. CM IDLE UEs that joined the multicast session are paged. Activation can be triggered by AF request, or data notification from the MB-UPF. Multicast session state transitions from Inactive state to Active state.

-	Multicast Session Deactivation: Triggered by the 5GC, the radio resources for the multicast session are released and multicast session data stops to be transmitted to the UE. Deactivation can be triggered by AF request, or no reception of multicast data by the MB-UPF. Multicast session state transitions from Active to Inactive state.

-	Multicast Session Leave/Release: The UE initiates multicast session leave towards the network. The UE can leave an active or inactive multicast session. Multicast session state transitions from Active or Inactive state to NULL in the UE. When the last UE of the multicast session in the NG-RAN node left the multicast session, the multicast session is released between the 5GC and NG-RAN node. User Plane resources for the multicast session between 5GC and the NG-RAN node are released. Multicast session state transitions from active or inactive state to NULL in the NG-RAN node. When the last UE of the multicast session left the multicast session (i.e. User Plane resources for the multicast session between 5GC and the last NG-RAN node are released), multicast session state transitions from Active or Inactive state to Configured in 5GC NFs if Multicast Session Configuration was performed, and from Active or Inactive state to NULL in 5GC NFs if Multicast Session Configuration was not performed. .

-	Multicast Session Deconfiguration: All information about the multicast session is removed from the 5GC, and TMGIs (if allocated during Multicast Session Configuration) are deallocated. The deconfiguration may be triggered by an AF request. Multicast session state transitions from active or inactive state to NULL in the 5GC NFs (e.g. MB-SMF).

Figure 4.3-1, Figure 4.3-2, Figure 4.3-3 and Figure 4.3-4 show the multicast session states and state transitions in MB-SMF (with and without Configured state), UE and NG-RAN respectively.

Figure 4.3-1: Multicast session states and state transitions in MB-SMF (with Configured state)

Figure 4.3-2: Multicast session states and state transitions in MB-SMF (without Configured state)

Figure 4.3-3: Multicast session states and state transitions in UE

Figure 4.3-4: Multicast session states and state transitions in NG-RAN

******************** End of Change ********************