3GPP TSG-WG SA2 Meeting #145E e-meeting 	S2-2104310

Elbonia, May 17– 28, 2021	(revision of S2-210xxxx)

Source:	Huawei, HiSilicon, CBN

Title:	TS 23.247: Clarification on description of the NFs

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract: This contribution proposes to remove the ENs in clause 5.3.

1. Discussion

In the last meeting, some ENs were added:

-	Editor's note:	Whether N6mb and Nmb5 are with the same functionalities or not is FFS.

In fact such issue is not controversial since from the protocol point of view they are same, but since different NFs, companies suggested to name them differently and therefore such EN provide it can be removed.

-	Editor’s note:	Whether AMF or SMF discovers MB-SMF is FFS.

Given that the technically endorsed document S2-2103074, it should be the SMF discovers MB-SMF. Therefore it is suggested to remove this EN.

-	Editor’s note:	Whether more functionalities are required is FFS.

In addition, on the role of MB-SMF, there were some discussions (see the discussion of S2-2102282). The should be no obstacle for the implementation that using enhanced SMF to realize the functionalities of MB-SMF, therefore it is suggested to add an NOTE to clarify that.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.247.

# * * * * First change * * * *

5.3	Reference point and functional entities

5.3.1	Reference point

The 5MBS System Architecture contains the following new reference points:

N3mb:	Reference point between the (R)AN and the MB-UPF.

N4mb:	Reference point between the MB-SMF and the MB-UPF.

N6mb:	Reference point between the MB-UPF and the AF/AS.

N7mb:	Reference point between the MB-SMF and the PCF.

N11mb:	Reference point between the AMF and the MB-SMF.

N16mb:	Reference point between the SMF and the MB-SMF.

N19mb:	Reference Point between the UPF and the MB-UPF.

N29mb:	Reference point between the MB-SMF and the NEF.

Nmb1:	Reference point between the MB-SMF and the MBSF.

Nmb2:	Reference point between the MBSF and the MBSTF.

Nmb3:	Reference point between the MBSF and the NEF.

Nmb4:	Reference point between the MBSTF and the AF.

Nmb5:	Reference point between the MB-UPF and the MBSTF.

Nmb6:	Reference point between the MBSF and the AF.

Nmb7:	Reference point between the MBSF and the PCF

Editor's note:	The Nmb7 reference point is FFS.

The 5MBS System Architecture reuses the existing reference points of N1, N2, N10, N11, and N33 with enhancement to support 5G MBS.

5.3.2	Functional entities

Editor's Note: functional entities and functional requirement are not completed.

5.3.2.1	PCF

The PCF performs the following functions to support 5MBS if dynamic PCC for 5MBS is needed:

-	Supporting QoS handling for MBS Session.

-	Providing policy information regarding the MBS Session to MB-SMF for authorizing the related QoS profiles.

-	Interacting with UDR for QoS information retrieval.

Editor's Note: The following is ffs: The PCF receives MB service information from AF, NEF or MBSF

5.3.2.2	MB-SMF

The MB-SMF performs the following functions to support 5MBS:

-	General for multicast and broadcast sessions:

-	Supporting MBS session management (including QoS control).

-	Configuring the MB-UPF for multicast and broadcast flows transport based on the policy rules for multicast and broadcast services from PCF or local policy.

-	Specific for broadcast sessions:

-	Interacting with RAN (via AMF) to control data transport using 5GC Shared MBS traffic delivery method.

-	Specific for multicast sessions:

-	Interacting with SMF to modify PDU Session associated with 5MBS.

-	Interacting with RAN (via AMF and SMF) to establish data transmission resources between MB-UPF and RAN nodes for 5GC Shared MBS traffic delivery method.

-	Controlling multicast data transport using 5GC Individual MBS traffic delivery method.

NOTE:	MB-SMF can be implemented by enhancing SMF, given that MB-SMF supports the provisioning of N2 message and N4 configuration.

5.3.2.3	SMF

The SMF performs the following functions to support 5MBS:

-	Discovering MB-SMF for multicast session.

-	Authorizing multicast session join operation if needed.

-	Interacting with MB-SMF to obtain and manage multicast session context.

-	Interacting with RAN for shared data transmission resource establishment.

NOTE:	SMF and MB-SMF may be co-located or deployed separately.

5.3.2.4	MB-UPF

The MB-UPF performs the following functions to support 5MBS:

-	General for multicast and broadcast sessions:

-	Packet filtering of incoming downlink packets for multicast and broadcast flows.

-	QoS enforcement (MFBR) and counting/reporting based on existing means.

-	Interaction with MB-SMF for receiving multicast and broadcast data.

-	Delivery of multicast and broadcast data to RAN nodes for 5GC Shared MBS traffic delivery method.

-	Specific for multicast sessions:

-	Delivery of multicast data to UPF for 5GC Individual MBS traffic delivery method.

5.3.2.5	UPF

The UPF performs the following functions to support 5MBS:

-	Interacting with SMF for receiving multicast data from MB-UPF for 5GC Individual MBS traffic delivery method.

-	Delivering multicast data to UEs via PDU Session for 5GC Individual MBS traffic delivery method.

NOTE:	UPF and MB-UPF may be co-located or deployed separately.

5.3.2.6	AMF

The AMF performs the following functions to support 5MBS:

-	Signalling with NG-RAN and MB-SMF for MBS Session management.

-	Selection of NG-RANs for notification of multicast session activation toward UEs in CM-IDLE state.

-	Selection of NG-RANs for broadcast.

5.3.2.7	NG-RAN

The NG-RAN performs the following functions to support 5MBS:

-	Management of MBS QoS flows via N2.

-	Delivery of MBS data packets from 5GC shared for multiple UEs over radio using PTM or PTP.

-	Configuration of UE for MBS QoS flow reception at AS layer.

-	Control switching between PTM and PTP delivery per UE.

-	Support for multicast sessions continuity during Xn Handover and N2 Handover.

-	Support notification of multicast session activation over radio toward UEs in CM-IDLE state.

5.3.2.8	UE

The UE may perform the following functions to support 5MBS:

-	Reception of multicast data using PTM/PTP.

-	Reception of multicast and broadcast data using PTM.

-	Handling of incoming MBS QoS flows.

-	Support of signalling for joining and leaving multicast session.

-	5MBS resource management support at AS layer.

-	Reception of notification in CM-IDLE state from 5GS for multicast data transmission.

5.3.2.9	AF

The AF performs the following functions to support 5MBS:

-	Requesting multicast or broadcast service from the 5GC by providing service information including QoS requirement to 5GC.

-	Instructing MBS session operation towards 5GC if needed.

-	Interacting with NEF for MBS related service exposure.

5.3.2.10	NEF

The NEF performs the following functions to support 5MBS:

-	Providing an interface to AFs for 5MBS procedures including service provisioning, MBS session and QoS management.

-	Interacting with AF and MB-SMF for MBS session operations, determination of transport parameters, and session transport.

-	Selection of serving MB-SMF for an MBS Session.

5.3.2.11	MBSF

The MBSF performs the following functions to support 5MBS:

-	Service level functionality to support 5MBS, and interworking with LTE MBMS

-	Interacting with AF and MB-SMF for MBS session operations, determination of transport parameters, and session transport.

-	Selection of serving MB-SMF for an MBS Session.

-	Controlling MBSTF if the MBSTF is used.

-	Determination of sender IP multicast address for the MBS session if IP multicast address is sourced by MBSTF.

NOTE:	MBSF functionality related to service and MBS data handling (e.g. encoding) is to be determined with SA WG4.

5.3.2.12	MBSTF

The MBSTF performs the following functions to support 5MBS if deployed:

-	Media anchor for MBS data traffic if needed.

-	Sourcing of IP Multicast if needed.

-	Generic packet transport functionalities available to any IP multicast enabled application such as framing, multiple flows, packet FEC (encoding).

-	Multicast/broadcast delivery of input files as objects or object flows.

NOTE: MBSTF functionality related to MBS data handling (e.g. encoding) is to be determined with SA4.

5.3.2.13	UDM

The UDM performs the following functions to support 5MBS:

-	Support management of subscription for authorization for multicast sessions.

5.3.2.14	UDR

The UDR performs the following functions to support 5MBS if deployed:

-	Support management of UE authorization information for multicast session.

#### 5.3.2.15	NRF

The NRF performs the following functions to support 5MBS:

-	Support management of MB-SMF information serving multicast sessions.

# * * * * End of changes * * * *