3GPP TSG-WG SA2 Meeting #145E e-meeting 	S2-2104322

Elbonia, May 17 – 28, 2021

Source:	Nokia, Nokia Shanghai-Bell

Title:	5MBS Information in UDM

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract: This contribution updates clause 7.2.1.2 following proposals in S2-2103840.

# Introduction

Technically endorsed S2-2103074 against TS 23.247: 
based on the indication DNN and S-NSSAI use for of establishing a PDU Session associated with multicast session(s), and/or an indication that the user is allowed to join multicast sessions in the UDM the AMF selects an SMF capable of handling multicast sessions based on DNN and S-NSSAI, or locally configured data or a corresponding SMF capability stored in the NRF. For indirect discovery, the AMF requests the SCP to select an SMF capable of handling multicast sessions.
Editor's note:	Whether the indication in the UDM is needed or not is FFS.

Taking only DNN and S-NSSAI seems to assume that operator uses specific slides for PDU sessions that allow a multicast join. On the other hand, the PDU session can also contain contents unrelated to multicast. It seems too restrictive to require all operators to deploy specific slices to support multicast. On the other hand, not all UEs may be multicast capable or have a subscription for multicast services. Thus an entry in the SMF selection subscription data about need for multicast support seems appropriate. This can be considered by the AMF in combination with DNN and S-NSSAI to decide whether to select an 5MBS capable SMF

It seems reasonable to separate the Establishment of a PDU Session into a separate CR and focus in S2-2103074 on the join operation.

The present contribution copies the text from technically endorsed S2-2103074 but removes the editor´s note.

# 2. Text Proposal

It is proposed to agree the following changes for TR 23.757.

#### 7.2.1.2	Establishment of a PDU Session that can be associated with multicast session(s)

A PDU Session associated with multicast session(s) is established using the procedures as specified in TS 23.502 [6] clause 4.3.2.2 with the following differences:

NOTE:	The DNN and S-NSSAI are used to establish the PDU session which can carry the Multicast operation, i.e. join/leave, and can be associated with multicast MBS session(s).

-	In step 2, based on the DNN and S-NSSAI use for establishing a PDU Session associated with multicast session(s), and/or an indication that the user is allowed to join multicast sessions in the UDM the AMF selects an SMF capable of handling multicast sessions based on DNN and S-NSSAI, or locally configured data or a corresponding SMF capability stored in the NRF. For indirect discovery, the AMF requests the SCP to select an SMF capable of handling multicast sessions.