3GPP TSG-WG SA2 Meeting #145E e-Meeting	S2-2104366

# Elbonia,  May 17 - May 28, 2021													(revision of S2-2102737r01)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2805 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of ECS address configuration parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC |  |  |  |  |  | Date: |  |  | 2021-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The ECS address can be provided by AF via parameter provision procedure. A UE may host EEC(s) as defined in TS 23.558 [5] and support the ability to receive ECS address(es) from the 5GC and to transfer the ECS address(es) to the EEC(s). In this case, the ECS address provisioning via 5GC is described in clause 6.5.2 of TS 23.548. As indicated in clause 8.3.3.2.2 of TS 23.558, ahead of EEC triggers service provision request to ECS, the EEC should be authorized to communicate with ECS. It means that even if the EEC receives the ECS address, not all of the EEC has the right to use the service from ECS. So, it can be concluded that multiple ECS addresses should be delivered to one EEC or multiple EECs on UE. But not all of the EECs are able to receive the ECS address. And how the UE to identify which ECS addresses should be sent to which EEC is still FFS now. The only way to indicate the endpoint of ECS address delivery is using the EEC ID that defined in TS 23.558[x]. The EEC ID is a globally unique value that identifies an EEC. And as a UE capability, that it can resolve the packet of ECS Address Configuration Information in ePCO and identify the EEC ID in ECS Address Configuration Information. Finally, UE delivers the ECS address to EEC which is identified by EEC ID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The 3rd party AF provides both ECS address and other relation parameters to 5GC and UE in parameter provision. The relation parameters include address provider related (ECSP, ECS ID) parameters and target consumer related (EEC ID). During ePCO procedure, UE obtains FQDN(s) and/or IP Address(es) of ECSs and delivers them to the dedicated EEC according to EEC ID in ECS Address Configuration Information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE would not be able to identify which EECs on UE is able to receive the ECS address. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.3d |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

4.15.6.3d	ECS Address Configuration Information Parameters

ECS Address Configuration Information parameters that an AF may provide are described in Table 4.15.6.3d-1. ECS Address Configuration Information is stored as Session Management Subscription data. The ECS Address Configuration Information is associated with a DNN and S-NSSAI included in the message from UDM.

The ECS Address Configuration Information consists of one or more FQDN(s) and/or IP Address(es) of Edge Configuration Server(s), optionally the corresponding ECS Provider ID, ECS ID and EEC ID.

The ECS Provider ID and ECS ID are used to identify and differentiate the provision source of ECS address. And the EEC ID are used to identify which EECs on UE is able to receive the ECS address.

Table 4.15.6.3d-1: Description of ECS Address Configuration Information parameters

| Parameters | Description |
| --- | --- |
| ECS Address(es) | Consists of one or more FQDN(s) and/or IP Address(es) of Edge Configuration Server(s). |
| ECS ID | This IE indicates to the Identifier of ECS. The may be used to differentiate ECSs. |
| ECSP ID | The identifier of the ECS Provider |
| EEC ID | The EEC ID is a globally unique value that identifies an EEC. |

* * * End of Change * * *