3GPP TSG-WG SA2#145E  	S2-2104474

# emeeting, May 17-28, 2021	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2827 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add N3 tunnel information in SM context for I-SMF reselection per DNAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Ericsson, Huawei, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are some scenarios that NG-RAN UP is not changed but there is change of I-SMF/I-UPF, in this case, the target I-SMF is expected to send PDU Session Resource Modification Request instead of PDU Session Resource Setup Request to the NG-RAN. Below is an example: Example: I-SMF change due to requested DNAI as described in 4.23.5.4. In this case when the UE is in CM-CONNECTED state the AMF may reselect a new I-SMF supporting the requested DNAI. To address the above scenario without impacting other scenarios, the following is proposed : - The AMF sends indication of no NG-RAN change to the new I-SMF when the UE is in CM-CONNECTED state; - The new I/V-SMF then send this indication to old I-SMF; - Based on the indication the old I-SMF then sends NG-RAN tunnel to target I-SMF. - If received NG-RAN tunnel info, the target I-SMF then initiates PDU Session Resource Modify Request to NG-RAN |  |  |  |  |  |  |  |  |
| Summary of change: |  | When the AMF determines that I-SMF need a change but UE context is valid in NG-RAN, - The AMF sends indication of no NG-RAN change to the new I-SMF when the UE is in CM-CONNECTED state; - The new I/V-SMF then send this indication to old I-SMF; - Based on the indication the old I-SMF then sends NG-RAN tunnel to target I-SMF. - If received NG-RAN tunnel info, the target I-SMF then initiates PDU Session Resource Modify Request to NG-RAN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The I-SMF relocation procedure may fail. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.23.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The corresponding changes on service operation are captured in CR#2826 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.23.5.4	I-SMF selection per DNAI

Figure 4.23.5.4-1: I-SMF selection per DNAI

1.	The PCF sends to the SMF PCC rule(s) including the DNAI(s) for the PDU sessions by invoking Npcf_SMPolicyControl_UpdateNotify service operation.

Based on the received DNAI(s) information, the SMF may subscribe to the UE mobility event notification from the AMF (e.g. UE moves into or out of Area of Interest).

The SMF determines the target DNAI(s) which are applicable to the current UE location. Then the SMF may decide the target DNAI finally.

2.	The SMF invokes a Nsmf_PDUSession_SMContextStatusNotify service operation (or Nsmf_PDUSession_StatusNotify) if it (or the associated old I-SMF) cannot serve the target DNAI and the content of the message includes the target DNAI information which may indicate that the I-SMF selection is expected. This is to trigger the AMF to (re)select a suitable I-SMF for the PDU Session. The target DNAI is used for selecting (I-)SMF, which controls UPF connecting to that DNAI.

If there is an I-SMF serving the PDU session, the SMF invokes Nsmf_PDUSession_StatusNotify and then the I-SMF invokes Nsmf_PDUSession_SMContextStatusNotify message to send the target DNAI information for existing PDU session to AMF.

3.	If the I-SMF selection is expected, the AMF selects a new I-SMF which can serve the target DNAI for the existing PDU Session as described in clause 5.34.3 of TS 23.501 [2].

4.	The AMF sends a Nsmf_PDUSession_CreateSMContext Request to the new I-SMF, as described in step 3 of clause 4.23.4.3 with the following enhancement:

The AMF includes target DNAI received from SMF in the message. When the UE is in CONNECTED state the AMF also include indication of no NG-RAN change.

5.	The procedure described in clauses 4.23.4.3 (case: I-SMF insertion or I-SMF change) starting from step 4 takes place with the following difference:

Step 4a, the (target )new I-SMF sends the indication of no NG-RAN change to the old I-SMF or SMF as it is received from AMF .

Step 4b. When the old I-SMF or SMF receives indication of no NG-RAN change it include the additional Downlink Tunnel Info of NG-RAN in the SM context of the PDU Session.

In step 5, the I-SMF selects the I-UPF based on target DNAI.

In step  6, The target I-SMF should reuse the downlink Tunnel Info of serving RAN node if received from old I-SMF/SMF as described in clause 4.23.4.3.

In step  9, if the new I-SMF receives the Downlink Tunnel Info of NG-RAN the N2 SM information includes PDU Session Resource Modification message

If the UE is in IDLE state the step 17-21 are skipped. Steps 17a and 17b are still performed to release the old I-SMF.

6.	The PSA and UL CL/BP controlled by I-SMF is inserted as described from steps 2 to 11 in figure 4.23.9.1-1 is performed with the following difference:

In step 2, the I-SMF selects a new PDU Session Anchor (PSA2) based on the target DNAI received in step 4.

# * * * * end of change * * * *