3GPP TSG-WG SA2 Meeting #145E e-meeting 	S2-2104509

Elbonia, May 17– 28, 2021

Source:	Huawei, HiSilicon

Title:	Update on EAS IP replacement procedures

Document for:	Approval

Agenda Item:	eEDGE_5GC

Work Item / Release:	Rel-17

Abstract: This contribution introduces updates on EAS IP replacement procedures.

# 1. Discussion

This contribution introduces the following update on EAS IP replacement procedures:

- In SA2#144e meeting, it clarified that information for EAS IP Replacement in 5GC indicates the EAS IP Replacement in 5GC is enabled or disabled. The existence of "source EAS IP address and port number, target EAS IP address and port number" can already indicate the EAS IP replacement is requested, so the indication "the EAS IP Replacement in 5GC is enabled or disabled" is not needed.

- For Enabling EAS IP replacement procedure by AF specified in clause 6.3.3.1.1, step 5 refers to clause 4.3.5.4 or 4.3.5.6 of TS 23.502. For the condition of UL CL change, clause 4.3.5.7 should also be referred, otherwise when step 5 is referred by clause 6.3.3.1.2, UL CL change cannot be dealt with.

- For EAS IP replacement update upon DNAI and EAS IP change specified in clause 6.3.3.1.2, since UL CL and L-PSA are changed, SMF should configure the new UL CL with forwarding rule, and the new L-PSA with FARs "Outer Header Creation" and "Outer Header Removal". This contribution proposes to add a step to do such configuration.

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.548.

# * * * * First change * * * *

### 6.3.3	Edge Relocation Using EAS IP Replacement

EAS IP replacement enables the Local PSA UPF to replace the source/old Target EAS IP address and port number with the target/new target EAS IP address and port number for the Destination IP address and Destination Port number field of the uplink traffic and replace the target/new target EAS IP address and port number with the source/old Target EAS IP address and port number for the Source IP address and Source Port number field of the downlink traffic based on the enhanced AF Influence information for EAS IP replacement (i.e. source EAS IP address and port number, target EAS IP address and port number). The source AS IP address and port number are the destination IP address and port number of the uplink traffic, generated by UE, for a service subject to Edge Computing. The source EAS IP address is the one discovered by UE for a service subject to Edge Computing.

EAS IP replacement requires support of TCP/TLS/QUIC context transfer between EASs.

NOTE 1:	The feasibility of this requirement, i.e. TCP/TLS/QUIC context transfer between EASs, is unclear and whether third party platforms would support this TCP/TLS/QUIC context transfer between EASs is unknown/not clear.

#### 6.3.3.1	EAS IP Replacement Procedures

##### 6.3.3.1.1	Enabling EAS IP Replacement Procedure by AF

Figure 6.3.3.1.1-1: Enabling EAS IP Replacement Procedure by AF

NOTE 1:	This procedure covers the scenarios that the UE moves from non-EC to EC or the AF decides to enable the EAS IP replacement in the middle of a session.

1.	UE requests to establish a PDU Session.

2.	UE is preconfigured with the Source EAS IP address or discovers the IP address of the application server for the service subject to Edge Computing and the Source EAS IP address is returned to the UE via EAS Discovery procedure as described in clause 6.2.

3.	UE communicates with the Source EAS.

4.	When AF detects that the EAS is capable of runtime context mirroring and an optimal EAS is found, then AF decides to influence the traffic routing in 5GC. The EAS IP replacement information (i.e. source EAS IP address and port number, target EAS IP address and port number) is sent to the SMF within the AF Influence information and the SMF reconfigures the UL CL UPF for local traffic routing and Local PSA with EAS IP replacement information. Or when UE moves to an area where the Local PSA has been configured to enforce EAS IP address replacement.

UL CL is configured by SMF to forward UL packet to Local PSA if the destination IP address is the Source EAS IP address.

Local PSA is configured by SMF to enforce the "Outer Header Creation" and "Outer Header Removal" as described in step 5. FARs "Outer Header Creation" and "Outer Header Removal" are reused for such an instruction from SMF to UPF.

Detailed enhancement to the AF Influence procedure is described in clause 6.3.3.2.

If a new Local PSA is selected by SMF, the SMF may configure the new Local PSA to buffer the uplink traffic per clause 6.3.5 and enforce the "Outer Header Creation" and "Outer Header Removal" as described in step 6.

5.	When Early/Late Notification procedure with enhancement described in clause 6.3.3.2 is triggered, the SMF notifies AF about the target DNAI. Based on the target DNAI, the AF selects a proper target EAS, then the AF triggers to mirror the runtime context between Source EAS and Target EAS. Once the Target EAS is ready, AF responds to SMF about the Target EAS IP information (i.e. Target EAS IP address and port number). During the addition or change of UL CL and Local PSA as described in clause 4.3.5.4, 4.3.5.6 or 4.3.5.7 of TS 23.502 [3], SMF may (re)configure Local PSA for EAS IP address replacement between Source EAS and Target EAS.

6.	Local PSA starts to perform "Outer Header Creation" and "Outer Header Removal" FARs as instructed by SMF, which results in EAS IP address replacement:

-	For UL traffic, the destination IP address and port number are replaced with the Target EAS IP address and port number;

-	For DL traffic, the source IP address and port number are replaced back with the Source EAS IP address and port number.

NOTE 2:	In this solution, the PSA UPF need not to understand the logic of EAS IP replacement.

Then all subsequent uplink traffic of this EC service for this UE is forwarded to the target EAS.

NOTE 3:	AF decides when and how to stop the Source EAS from serving the UE based on its local configuration.

##### 6.3.3.1.2	EAS IP Replacement Update upon DNAI and EAS IP Change

UE

SMF

Local PSA2

Local PSA1

PCF

Target UL CL

AF

Old Target EAS

New Target EAS

Source UL CL

Steps 3-4 are same as steps 5-6 described in clause 6.3.3.1.1

1a. UL traffic (Src IP: UE IP, Dst IP: Source EAS IP)

DL traffic (Src IP: Source EAS IP, Dst IP: UE IP)

1b. UL traffic (Src IP: UE IP, Dst IP: Old Target EAS IP)

DL traffic (Src IP: Old Target EAS IP, Dst IP: UE IP)

2. SMF reconfigures UL CL and Local PSA

Figure 6.3.3.1.2-1: EAS IP Replacement Update upon DNAI and EAS IP change

1.	For UL traffic, the destination IP address is replaced with the old Target EAS IP address at Local PSA; for DL traffic, the source IP address is replaced back with the Source EAS IP address at Local PSA.

2. SMF configures Target UL CL with forwarding rules and Local PSA2 with FARs, as described in step 4 of clause 6.3.3.1.1.

Steps 3-4 are same as steps 5-6 described in clause 6.3.3.1.1 except that the UL CL, Local PSA and Target EAS in clause 6.3.3.1.1 are replaced by Target UL CL, Local PSA2 and new Target EAS respectively.

##### 6.3.3.1.3	Disabling EAS IP Replacement Procedure

Figure 6.3.3.1.3-1: Disabling EAS IP Replacement Procedure

1.	Local PSA performs "Outer Header Creation" and "Outer Header Removal" FARs as instructed by SMF, which results in EAS IP address replacement:

-	For UL traffic, the destination IP address and port number are replaced with the old Target EAS IP address and port number;

-	For DL traffic, the source IP address and port number are replaced back with the Source EAS IP address and port number.

2.	Due to UE Mobility to a Non-EC environment, when Early/Late Notification is triggered for the change from the UP path status where a DNAI applies to a status where no DNAI applies, AF knows the UE moves out of EC environment and mirrors the runtime session context from old Target EAS to Source EAS. Once ready, the AF indicates SMF without providing source/target EAS IP addresses and port numbers, so the SMFdisables the local routing at UL CL and the EAS IP replacement at Local PSA for this PDU Session.

3.	UL and DL traffic goes through Remote PSA, no EAS IP address replacement happens at Remote PSA.

NOTE 1:	AF decides when and how to stop the old Target EAS from serving the UE based on its local configuration. In case of AF relocation, AF doesn't have to disable the EAS IP Replacement in 5GC.

#### 6.3.3.2	Enhancement to AF Influence

The AF may additionally include Source and Target EAS IP address(es) and Port number(s) in the Nnef_TrafficInfluence_Create/Update request. Based on the Source EAS IP address(es) and Port number(s), the SMF knows which service flow(s) is(are) subject to EAS IP Replacement.

Using Early/Late Notification procedure, the AF sends an/a early/late notification response to the SMF when EAS relocation is completed. The SMF sends the "Outer Header Creation" and "Outer Header Removal" FARs to (target) Local PSA UPF and (target) Local PSA UPF starts the EAS IP address replacement as described in clause 6.3.3.1.

For load balancing purpose, the AF may move some UE(s) from the old Target EAS to the New Target EAS in the same local DN identified by the DNAI. For the abnormal condition of EAS, the AF may move all the UEs being served by the source EAS to a target EAS in the same local DN. For those purposes, the AF needs to include List of UEs, the source/old Target EAS IP address and port number for the impacted DNAI, the (new) Target EAS IP address and port number for the impacted DNAI in the Nnef_TrafficInfluence_Create/Update request.

The additional parameters for enabling the EAS IP Replacement are defined in clause 5.6.7.1 of TS 23.501 [2], clause 4.3.6.3 and 4.3.6.4 of TS 23.502 [3].

# * * * * End of changes * * * *