SA WG2 Meeting #145E	S2-2104592

# April 17th – 28th, 2021 ; Electronic meeting        		 	 				(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2948 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of UPF indication of backhaul QoS to SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_ARCH |  |  |  |  |  | Date: |  |  | 2021-05-03 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The cover page of TS 23.501 CR 2674r1 (S2-2101671) assumes that the AMF may need to be configured to be aware of connections of gNBs that involve satellite backhaul with long delay. Assuming the AMF must be aware of satellite backhaul, the UPF awareness is superfluous and can be removed to reduce the impact area of satellite backhaul in the CN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | AMF informs the SMF of satellite backhaul instead of the UPF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary duplication of configuration and possibly conflicting configurations between the AMF and the UPF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.8.2.15	Reporting of satellite backhaul to SMF

If the AMF is aware that a satellite backhaul with long delay is used towards 5G AN, the AMF may report this to SMF as part of the PDU Session establishment procedure as described in TS 23.502 [3], clause 4.3.2.

NOTE:	In this release, it is assumed that all QoS flows of the UE use the same backhaul as the N2 signalling connection and thus the AMF can deteremine the Satellite backhaul category based on local configuration.