3GPP TSG-SA WG2 Meeting #145E e-meeting	S2-2104626

# Elbonia, 17-28 May, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2850 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: |  |  | UICC apps |  |  |  |  | ME |  | Radio Access Network |  |  |  |  |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | TS23.502 KI#4 NSACF status notification on request |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  |  |  |  |  | Date: |  |  | 2021-05-05 |  |  |  |
| Category: | B |  |  |  |  |  |  |  |  |  | Release: |  |  | Rel-17 |  |  |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |  |  |  |
| Reason for change: |  | This CR is part of the new feature addition to TS 23.502 related to eNS_Ph2 KI#4. The network slice status notification to AF based on event fulfilment trigger was approved at SA2#144E meeting. This CR proposes network slice status retrieval from the NSACFvia NEF on request from the AF. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | New eNS_Ph2 feature addition for KI#4. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | KI#4 is not fully implemented in TS 23.502 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.3.2.x (new), 5.2.21.1, 5.2.21.x (new), 5.2.6.1, 5.2.6.x (new) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * * start of 1st  change * * * *

###### 4.15.3.2.X	Number of UEs and PDU Sessions per network slice status retrieval by AF procedure

AF

NSACF

1. Nnef_SliceStatus_Retrieval Request

NEF

3. Nnsacf_SliceStatus_Retrieval Request

4. Nnsacf_SliceStatus_Retrieval Response

5. Nnef_SliceStatus_Retrieval Response

2. AF authentication

Figure 4.15.3.2.X-1: Number of UEs and PDU Sessions per network slice status retrieval by AF procedure.

- To retrieve information about the number of the UEs registered with a network slice or the number of the PDU Sessions established on a network slice or both, the AF sends Nnef_SliceStatus_Retrieval Request (Event ID, Event Filter) message to the NEF. The Event ID parameter defines the information to be reported, i.e. the number of registered UEs with a network slice or the number of the PDU sessions with a network slice or both. The Event Filter parameter defines the S-NSSAI for which reporting is required.

- The NEF checks whether the AF is authorised. If authorised, the NEF may query the NRF to find the NSACF responsible for the requested S-NSSAI.

- The NEF forwards the request to the NSACF with Nnsacf_SliceStatus_Retrieval Request (Event ID, Event Filter).

- The NSACF returns the Nnsacf_SliceStatus_Retrieval Response (Event ID, Event Filter, Event Reporting information) message to the NEF. In the Event Reporting information parameter the NSACF provides information for the current number of the UEs registered with a network slice (e.g. represented in percentage of the maximum number of the UEs registered with the network slice) or information for the current number of the PDU Sessions on a network slice (e.g. represented in percentage of the maximum number of the PDU Sessions established on the network slice) or both depending on the Event ID.

- The NEF forwards the message to the AF in the Nnef_SliceStatus_Retrieval Response (Event ID, Event Filter, Event Reporting information) message.

* * * * end of 1st change * * * *

* * * * start of 2nd change * * * *

#### 5.2.21.1	General

The following table illustrates the NSACF services.

Table 5.2.21.1-1: List of NSACF services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnsacf_NumberOfUEsPerSlice | AvailabilityCheckAndUpdate | Request/Response | AMF |
|  | EACNotify |  | AMF |
| Nnsacf_SliceStatus | Retrieval | Request/Response | NEF |

* * * * end of 2nd change * * * *

* * * * start of 3rd change * * * *

#### 5.2.21.x	Nnsacf_SliceStatus services

##### 5.2.21.x.1	General

Service Description: The Nnsacf_SliceStatus services provide network slice status information on request from the consumer NF related to the current number of UEs registered with a network slice or the current number of PDU Sessions established on a network slice or both.

##### 5.2.21.x.2	Nnsacf_SliceStatus_Retrieval service operation

Service operation name: Nnsacf_SliceStatus_Retrieval

Description: This service operation is used by the consumer NF to retrieve the network slice status information related to the number of UEs registered for a network slice or the number of PDU Sessions established on a network slice or both.

Inputs, Required: Event ID, Event Filter

The Event ID parameter defines whether the requested information is about the number of UEs registered with a network slice or the number of PDU Sessions established on a network slice or both.

The Event Filter parameter is the S-NSSAI for which the requested information is applicable.

Outputs, Required: Event ID, Event Filter, Event Reporting information.

The Event ID parameter defines the type of the reported information, i.e. information about the number of UEs registered with a network slice or the number of PDUs Sessions established on a network slice or both.

The Event Filter parameter is the S-NSSAI for which the information retrieval is applicable.

The Event Reporting information parameter provides information for the current number of UEs registered with a network slice (e.g. represented in percentage of the maximum number of the UEs registered with the network slice) or information for the current number of PDU Sessions established on a network slice (e.g. represented in percentage of the maximum number of the PDU Sessions established on the network slice) or both.

* * * * end of 3rd change * * * *

* * * * start of 4th change * * * *

### 5.2.6	NEF Services

#### 5.2.6.1	General

The following table shows the NEF Services and Service Operations:

Table 5.2.6.1-1: NF Services provided by the NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_EventExposure | Subscribe | Subscribe/Notify | AF, NWDAF |
|  | Unsubscribe |  | AF, NWDAF |
|  | Notify |  | AF, NWDAF |
| Nnef_PFDManagement | Fetch | Request/Response | SMF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Notify |  | SMF |
|  | Unsubscribe |  | SMF |
|  | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_ParameterProvision | Update | Request/Response | AF |
|  | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_Trigger | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
| Nnef_BDTPNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_TrafficInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | AppRelocationInfo | Subscribe/Notify | AF |
| Nnef_ChargeableParty | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify | Request/Response | AF |
| Nnef_AFsessionWithQoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Revoke | Request/Response | AF |
| Nnef_MSISDN-less_MO_SMS | Notify | Notify | AF |
| Nnef_ServiceParameter | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_APISupportCapability | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe | Subscribe/Notify | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_NIDDConfiguration | Create | Request/Response | AF |
|  | TriggerNotify | Subscribe/Notify | AF |
|  | UpdateNotify | Subscribe/Notify | AF |
|  | Delete | Request/Response | AF |
| Nnef_NIDD | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
|  | GroupDeliveryNotify | Notify | AF |
| Nnef_SMContext | Create | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
|  | DeleteNotify | Subscribe/Notify | SMF |
|  | Delivery | Request/Response | SMF |
| Nnef_AnalyticsExposure | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
|  | Fetch | Request/Response | AF |
| Nnef_UCMFProvisioning | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ECRestriction | Get | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ApplyPolicy | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_Location | LocationUpdateNotify | Notify | AF |
| Nnef_SliceStatus | Retrieve | Request/Response | AF |

* * * * start of 4th change * * * *

* * * * start of 5th change * * * *

#### 5.2.6. x	Nnef_SliceStatus service

#### 5.2.6.x.1	General

Service Description: The Nnef_SliceStatus services provide network slice status information on request from the consumer NF (e.g. AF) related to the number of the UEs registered with a network slice or the number of PDU Sessions established on a network slice or both

##### 5.2.6.x.2	Nnef_SliceStatus_Retrieval service operation

Service operation name: Nnef_SliceStatus_Retrieval

Description: This service operation is used by the consumer NF (e.g. AF) to retrieve the network slice status information related to the number of the UEs registered with a network slice or the number of PDU Sessions established on a network slice or both.

Inputs, Required: Event ID, Event Filter

The Event ID parameter defines whether the requested information is about the number of UEs registered with a network slice or the number of PDU Sessions established on a network slice or both.

The Event Filter parameter is the S-NSSAI for which the requested information is applicable.

Outputs, Required: Event ID, Event Filter, Event Reporting information.

The Event ID parameter defines the type of the reported information, i.e. the number of UEs registered with a network slice or the number of PDUs Sessions established on a network slice or both.

The Event Filter parameter is the S-NSSAI for which the reported information is applicable.

The Event Reporting information parameter provides information for the current number of UEs registered with a network slice (e.g. represented in percentage of the maximum number of the UEs registered with the network slice) or information for the current number of PDU Sessions established on a network slice (e.g. represented in percentage of the maximum number of the PDU Sessions established on the network slice) or both.

* * * * end of 5th change * * * *