3GPP TSG-SA WG2 Meeting #145-e (e-meeting)	S2-2104632

# 17-28 May 2021, Elbonia	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2959 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Grandmaster candidate enabled management information per PTP instance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-04-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Table 5.28.3.1-2 lists the "Grandmaster candidate enabled" bridge management information as applicable for the entire NW-TT. However, whether NW-TT should consider the 5GS clock as a grandmaster clock candidate during BMCA may differ from one configured PTP instance to another. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove "Grandmaster candidate enabled" information from Table 5.28.3.1-2. Add "Grandmaster candidate enabled" information to Table 5.28.3.1-1 such that that it applies per PTP instance per NW-TT. Clarify in clause K.2.2.4 that "Grandmaster candidate enabled" is configured using PMIC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of flexibility to configure 5GS as a grandmaster for selected PTP instances only. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.28.3.1; K.2.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

#### 5.28.3.1	General

Port number of the DS-TT for the PDU Session is assigned by the UPF during PDU session establishment. The port number of the DS-TT port for a PDU Session shall be reported to the SMF from the UPF and further stored at the SMF. The SMF provides the DS-TT port number via PCF to the TSN AF or NEF. TSN AF or NEF maintains an association between the DS-TT port number and the MAC address (with Ethernet type PDU session) or IP address (with IP type PDU Session) of the UE. If a PDU session for which SMF has reported a DS-TT port number to TSN AF or NEF is released, then SMF informs TSN AF or NEF accordingly.

NOTE:	Port number can refer either to Ethernet port or PTP port. In Ethernet type PDU Sessions, it is assumed that the PTP port number is the same as the associated Ethernet port number.

5GS shall support transfer of standardized and deployment-specific port management information transparently between TSN AF or NEF and DS-TT or NW-TT, respectively inside a Port Management Information Container. NW-TT may support one or more ports. In this case, each port uses separate Port Management Information Container. 5GS shall also support transfer of standardized and deployment-specific bridge management information transparently between TSN AF or NEF and NW-TT, respectively inside a Bridge Management Information Container. Table 5.28.3.1-1 and Table 5.28.3.1-2 list standardized port management information and bridge management information, respectively.

If TSN AF is deployed, i.e. if 5GS is integrated with an IEEE TSN network, the port and bridge management information is exchanged between CNC and TSN AF. The port management information, is related to ports located in DS-TT or NW-TT.

If TSN AF is not deployed, the port and bridge management information is exchanged between NEF and DS-TT/NW-TT.

Editor's note:	It is FFS which of the information elements in the tables below are exchanged between NEF and DS-TT/NW-TT.

Table 5.28.3.1-1: Standardized port management information

| Port management information | Applicability (see NOTE 6) |  | Supported operations by TSN AF | Reference |
| --- | --- | --- | --- | --- |
|  | DS-TT | NW-TT | (see NOTE 1) |  |
| General |  |  |  |  |
| Port management capabilities (see NOTE 2) | X | X | R |  |
| Bridge delay related information |  |  |  |  |
| txPropagationDelay | X | X | R | IEEE Std 802.1Qcc [95] clause 12.32.2.1 |
| Traffic class related information |  |  |  |  |
| Traffic class table | X | X | RW | IEEE Std 802.1Q [98] clause 12.6.3 and clause 8.6.6. |
| Gate control information |  |  |  |  |
| GateEnabled | X | X | RW | IEEE Std 802.1Q [98] Table 12-29 |
| AdminBaseTime | X | X | RW | IEEE Std 802.1Q [98] Table 12-29 |
| AdminControlList | X | X | RW | IEEE Std 802.1Q [98] Table 12-29 |
| AdminCycleTime (see NOTE 3) | X | X | RW | IEEE Std 802.1Q [98] Table 12-29 |
| AdminControlListLength (see NOTE 3) | X | X | RW | IEEE Std 802.1Q [98] Table 12-28 |
| Tick granularity | X | X | R | IEEE Std 802.1Q [98] Table 12-29 |
| General Neighbor discovery configuration (NOTE 4) |  |  |  |  |
| adminStatus | D | X | RW | IEEE Std 802.1AB [97] clause 9.2.5.1 |
| lldpV2LocChassisIdSubtype | D | X | RW | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2LocChassisId | D | X | RW | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2MessageTxInterval | D | X | RW | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2MessageTxHoldMultiplier | D | X | RW | IEEE Std 802.1AB [97] Table 11-2 |
| NW-TT port neighbor discovery configuration |  |  |  |  |
| lldpV2LocPortIdSubtype |  | X | RW | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2LocPortId |  | X | RW | IEEE Std 802.1AB [97] Table 11-2 |
| DS-TT port neighbor discovery configuration |  |  |  |  |
| lldpV2LocPortIdSubtype | D |  | RW | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2LocPortId | D |  | RW | IEEE Std 802.1AB [97] Table 11-2 |
| Neighbor discovery information for each discovered neighbor of NW-TT |  |  |  |  |
| lldpV2RemChassisIdSubtype |  | X | R | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemChassisId |  | X | R | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemPortIdSubtype |  | X | R | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemPortId |  | X | R | IEEE Std 802.1AB [97] Table 11-2 |
| TTL |  | X | R | IEEE Std 802.1AB [97] clause 8.5.4 |
| Neighbor discovery information for each discovered neighbor of DS-TT (NOTE 5) |  |  |  |  |
| lldpV2RemChassisIdSubtype | D |  | R | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemChassisId | D |  | R | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemPortIdSubtype | D |  | R | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemPortId | D |  | R | IEEE Std 802.1AB [97] Table 11-2 |
| TTL | D |  | R | IEEE Std 802.1AB [97] clause 8.5.4.1 |
| Stream Parameters (NOTE 11) |  |  |  |  |
| MaxStreamFilterInstances | X |  | R | IEEE Std 802.1Q [98] clause 12.31.1.1 |
| MaxStreamGateInstances | X |  | R | IEEE Std 802.1Q [98] clause 12.31.1.2 |
| MaxFlowMeterInstances | X |  | R | IEEE Std 802.1Q [98] clause 12.31.1.3 |
| SupportedListMax | X |  | R | IEEE Std 802.1Q [98] clause 12.31.1.4 |
| Per-Stream Filtering and Policing information (NOTE 10) |  |  |  |  |
| Stream Filter Instance Table (NOTE 8) |  |  |  | IEEE Std 802.1Q [98] Table 12-32 |
| > StreamFilterInstanceIndex | X | X | RW | IEEE Std 802.1Q [98] Table 12-32 |
| > Stream Identification type | X | X | RW | IEEE 802.1CB [83] clause 9.1.1.6 |
| > Stream Identification Controlling Parameters | X | X | RW | IEEE 802.1CB [83] clauses 9.1.2, 9.1.3, 9.1.4 (NOTE 12) |
| > PrioritySpec | X | X | RW | IEEE Std 802.1Q [98] Table 12-32 |
| > StreamGateInstanceID | X | X | RW | IEEE Std 802.1Q [98] Table 12-32 |
| Stream Gate Instance Table (NOTE 9) |  |  |  | IEEE Std 802.1Q [98] Table 12-33 |
| StreamGateInstanceIndex | X | X | R | IEEE Std 802.1Q [98] Table 12-33 |
| PSFPAdminBaseTime | X | X | RW | IEEE Std 802.1Q [98] Table 12-33 |
| PSFPAdminControlList | X | X | RW | IEEE Std 802.1Q [98] Table 12-33 |
| PSFPAdminCycleTime | X | X | RW | IEEE Std 802.1Q [98] Table 12-33 |
| PSFPTickGranularity | X | X | R | IEEE Std 802.1Q [98] Table 12-33 |
| Time Synchronization Information |  |  |  |  |
| Supported PTP instance types (NOTE 13) | X |  | R | IEEE Std 1588-2019 [126] clause 8.2.1.5.5 |
| Supported transport types (NOTE 14) | X |  | R |  |
| Supported delay mechanisms (NOTE 15) | X |  | R | IEEE Std 1588-2019 [126] clause 8.2.15.4.4 |
| PTP grandmaster capable (NOTE 16) | X |  | R |  |
| gPTP grandmaster capable (NOTE 17) | X |  | R |  |
| Supported PTP profiles (NOTE 18) | X |  | R |  |
| Number of supported PTP instances | X |  | R |  |
| PTP Instance ID | X | X | RW |  |
| > PTP profile (NOTE 19) | X | X | RW |  |
| > Transport type (NOTE 20) | X | X | RW |  |
| > Grandmaster candidate enabled |  | X | RW |  |
| > Grandmaster enabled (NOTE 21) | X |  | RW |  |
| IEEE Std 1588-2019 [126] data sets (NOTE 22) |  |  |  |  |
| > defaultDS.clockIdentity | X | X | RW | IEEE Std 1588-2019 [126] clause 8.2.1.2.2 |
| > defaultDS.clockQuality.clockClass | X | X | RW | IEEE Std 1588-2019 [126] clause 8.2.1.3.1.2 |
| > defaultDS.clockQuality.clockAccuracy | X | X | RW | IEEE Std 1588-2019 [126] clause 8.2.1.3.1.3 |
| > defaultDS.clockQuality.offsetScaledLogVariance | X | X | RW | IEEE Std 1588-2019 [126] clause 8.2.1.3.1.4 |
| > defaultDS.priority1 | X | X | RW | IEEE Std 1588-2019 [126] clause 8.2.1.4.1 |
| > defaultDS.priority2 | X | X | RW | IEEE Std 1588-2019 [126] clause 8.2.1.4.2 |
| > defaultDS.domainNumber | X | X | RW | IEEE Std 1588-2019 [126] clause 8.2.1.4.3 |
| > defaultDS.sdoId | X | X | RW | IEEE Std 1588-2019 [126] clause 8.2.1.4.5 |
| > defaultDS.instanceEnable | X | X | RW | IEEE Std 1588-2019 [126] clause 8.2.1.5.2 |
| > defaultDS.externalPortConfigurationEnabled |  | X | RW | IEEE Std 1588-2019 [126] clause 8.2.1.5.3 |
| > defaultDS.instanceType | X | X | RW | IEEE Std 1588-2019 [126] clause 8.2.1.5.5 |
| > portDS.portIdentity | X | X | RW | IEEE Std 1588-2019 [126] clause 8.2.15.2.1 |
| > portDS.portState | X | X | RW | IEEE Std 1588-2019 [126] clause 8.2.15.3.1 |
| > portDS.logMinDelayReqInterval | X | X | RW | IEEE Std 1588-2019 [126] clause 8.2.15.3.2 |
| > portDS.logAnnounceInterval | X | X | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.1 |
| > portDS.announceReceiptTimeout |  | X | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.2 |
| > portDS.logSyncInterval | X | X | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.3 |
| > portDS.delayMechanism | X | X | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.4 |
| > portDS.logMinPdelayReqInterval | X | X | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.5 |
| > portDS.versionNumber | X | X | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.6 |
| > portDS.minorVersionNumber | X | X | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.7 |
| > portDS.delayAssymetry | X | X | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.8 |
| > portDS.portEnable | X | X | RW | IEEE Std 1588-2019 [126] clause 8.2.15.5.1 |
| > timePropertiesDS.currentUtcOffset | X | X | RW | IEEE Std 1588-2019 [126] clause 8.2.4.2 |
| > timePropertiesDS.timeSource | X | X | RW | IEEE Std 1588-2019 [126] clause 8.2.4.9 |
| > externalPortConfigurationPortDS.desiredState |  | X | RW | IEEE Std 1588-2019 [126] clause 15.5.3.7.15.1 |
| IEEE Std 802.1AS [104] data sets (NOTE 22) |  |  |  |  |
| > defaultDS.clockIdentity | X | X | RW | IEEE Std 802.1AS [104] clause 14.2.2 |
| > defaultDS.clockQuality.clockClass | X | X | RW | IEEE Std 802.1AS [104] clause 14.2.4.2 |
| > defaultDS.clockQuality.clockAccuracy | X | X | RW | IEEE Std 802.1AS [104] clause 14.2.4.3 |
| > defaultDS.clockQuality.offsetScaledLogVariance | X | X | RW | IEEE Std 802.1AS [104] clause 14.2.4.4 |
| > defaultDS.priority1 | X | X | RW | IEEE Std 802.1AS [104] clause 14.2.5 |
| > defaultDS.priority2 | X | X | RW | IEEE Std 802.1AS [104] clause 14.2.6 |
| > defaultDS.timeSource | X | X | RW | IEEE Std 802.1AS [104] clause 14.2.15 |
| > defaultDS.domainNumber | X | X | RW | IEEE Std 802.1AS [104] clause 14.2.16 |
| > defaultDS.sdoId | X | X | RW | IEEE Std 802.1AS [104] clause 14.2.4.3 |
| > defaultDS.externalPortConfigurationEnabled |  | X | RW | IEEE Std 802.1AS [104] clause 14.2.18 |
| > defaultDS.instanceEnable | X | X | RW | IEEE Std 802.1AS [104] clause 14.2.19 |
| > portDS.portIdentity | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.2 |
| > portDS.portState |  | X | R | IEEE Std 802.1AS [104] clause 14.8.3 |
| > portDS.ptpPortEnabled | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.4 |
| > portDS.delayMechanism | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.5 |
| > portDS.isMeasuringDelay | X | X | R | IEEE Std 802.1AS [104] clause 14.8.6 |
| > portDS.asCapable | X | X | R | IEEE Std 802.1AS [104] clause 14.8.7 |
| > portDS.meanLinkDelay | X | X | R | IEEE Std 802.1AS [104] clause 14.8.8 |
| > portDS.meanLinkDelayThresh | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.9 |
| > portDS.delayAssymetry | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.10 |
| > portDS.neighborRateRatio | X | X | R | IEEE Std 802.1AS [104] clause 14.8.11 |
| > portDS.initialLogAnnounceInterval | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.12 |
| > portDS.currentLogAnnounceInterval | X | X | R | IEEE Std 802.1AS [104] clause 14.8.13 |
| > portDS.useMgtSettableLogAnnounceInterval | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.14 |
| > portDS.mgtSettableLogAnnounceInterval | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.15 |
| > portDS.announceReceiptTimeout |  | X | RW | IEEE Std 802.1AS [104] clause 14.8.16 |
| > portDS.initialLogSyncInterval | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.17 |
| > portDS.currentLogSyncInterval | X | X | R | IEEE Std 802.1AS [104] clause 14.8.18 |
| > portDS.useMgtSettableLogSyncInterval | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.19 |
| > portDS.mgtSettableLogSyncInterval | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.20 |
| > portDS.syncReceiptTimeout |  | X | RW | IEEE Std 802.1AS [104] clause 14.8.21 |
| > portDS.syncReceiptTimeoutTimeInterval |  | X | RW | IEEE Std 802.1AS [104] clause 14.8.22 |
| > portDS.initialLogPdelayReqInterval | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.23 |
| > portDS.currentLogPdelayReqInterval | X | X | R | IEEE Std 802.1AS [104] clause 14.8.24 |
| > portDS.useMgtSettableLogPdelayReqInterval | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.25 |
| > portDS.mgtSettableLogPdelayReqInterval | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.26 |
| > portDS.initialLogGptpCapableMessageInterval | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.27 |
| > portDS.currentLogGptpCapableMessageInterval | X | X | R | IEEE Std 802.1AS [104] clause 14.8.28 |
| > portDS.useMgtSettableLogGptpCapableMessageInterval | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.29 |
| > portDS.mgtSettableLogGptpCapableMessageInterval | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.30 |
| > portDS.initialComputeNeighborRateRatio | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.31 |
| > portDS.currentComputeNeighborRateRatio | X | X | R | IEEE Std 802.1AS [104] clause 14.8.32 |
| > portDS.useMgtSettableComputeNeighborRateRatio | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.33 |
| > portDS.mgtSettableComputeNeighborRateRatio | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.34 |
| > portDS.initialComputeMeanLinkDelay | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.35 |
| > portDS.currentComputeMeanLinkDelay | X | X | R | IEEE Std 802.1AS [104] clause 14.8.36 |
| > portDS.useMgtSettableComputeMeanLinkDelay | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.37 |
| > portDS.mgtSettableComputeMeanLinkDelay | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.38 |
| > portDS.allowedLostResponses | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.39 |
| > portDS.allowedFaults | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.40 |
| > portDS.gPtpCapableReceiptTimeout | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.41 |
| > portDS.versionNumber | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.42 |
| > portDS.nup | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.43 |
| > portDS.ndown | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.44 |
| > portDS.oneStepTxOper | X | X | R | IEEE Std 802.1AS [104] clause 14.8.45 |
| > portDS.oneStepReceive | X | X | R | IEEE Std 802.1AS [104] clause 14.8.46 |
| > portDS.oneStepTransmit | X | X | R | IEEE Std 802.1AS [104] clause 14.8.47 |
| > portDS.initialOneStepTxOper | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.48 |
| > portDS.currentOneStepTxOper | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.49 |
| > portDS.useMgtSettableOneStepTxOper | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.50 |
| > portDS.mgtSettableOneStepTxOper | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.51 |
| > portDS.syncLocked | X | X | R | IEEE Std 802.1AS [104] clause 14.8.52 |
| > portDS.pdelayTruncatedTimestampsArray | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.53 |
| > portDS.minorVersionNumber | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.54 |
| > timePropertiesDS.currentUtcOffset | X | X | RW | IEEE Std 802.1AS [104] clause 14.5.2 |
| > externalPortConfigurationPortDS.desiredState |  | R | RW | IEEE Std 802.1AS [104] clause 14.12.2 |
| NOTE 1: R = Read only access; RW = Read/Write access. NOTE 2: Indicates which standardized and deployment-specific port management information is supported by DS-TT or NW-TT. NOTE 3: AdminCycleTime and AdminControlListLength are optional for gate control information. NOTE 4: If DS-TT supports neighbor discovery, then TSN AF sends the general neighbor discovery configuration for DS-TT Ethernet ports to DS-TT. If DS-TT does not support neighbor discovery, then TSN AF sends the general neighbor discovery configuration for DS-TT Ethernet ports to NW-TT using the Bridge Management Information Container (refer to Table 5.28.3.1-2) and NW-TT performs neighbor discovery on behalf on DS-TT. When a parameter in this group is changed, it is necessary to provide the change to every DS-TT and the NW-TT that belongs to the 5GS TSN bridge. It is mandatory that the general neighbor discovery configuration is identical for all DS-TTs and the NW-TTs that belongs to the bridge. NOTE 5: If DS-TT supports neighbor discovery, then TSN AF retrieves neighbor discovery information for DS-TT Ethernet ports from DS-TT. If DS-TT does not support neighbor discovery, then TSN AF retrieves neighbor discovery information for DS-TT Ethernet ports from NW-TT, using the Bridge Management Information Container (refer to Table 5.28.3.1-2), the NW-TT performing neighbor discovery on behalf on DS-TT. NOTE 6: X = applicable; D = applicable when validation and generation of LLDP frames is processed at the DS-TT. NOTE 7: Void. NOTE 8: There is a Stream Filter Instance Table per Stream. NOTE 9: There is a Stream Gate Instance Table per Gate. NOTE 10: TSN AF indicates the support for PSFP to the CNC only if each DS-TT and NW-TT of the 5GS bridge has indicated support of PSFP. DS-TT indicates support of PSFP using port management capabilities, i.e. by indicating support for the Per-Stream Filtering and Policing information and by setting higher than zero values for MaxStreamFilterInstances, MaxStreamGateInstances, MaxFlowMeterInstances, SupportedListMax parameters. When available, TSN AF uses the PSFP information for determination of the traffic pattern information as described in Annex I. The PSFP information can be used at the DS-TT (if supported) and at the NW-TT (if supported) for the purpose of per-stream filtering and policing as defined in IEEE Std 802.1Q [98] clause 8.6.5.1. NOTE 11: TSN AF composes a Stream Parameter Table towards the CNC. It is up to TSN AF how it composes the Stream Parameter Table based on the numerical values as received from DS-TT and NW-TT port(s) and for the bridge for each individual parameter. NOTE 12: The set of Stream Identification Controlling Parameters depends on the Stream Identification type value as defined in IEEE Std 802.1CB [83] Table 9-1 and clauses 9.1.2, 9.1.3, 9.1.4. NOTE 13: Enumeration of supported PTP instance types. Allowed values as defined in IEEE Std 1588-2019 [126] clause 8.2.1.5.5. NOTE 14: Enumeration of supported transport types. Allowed values: IPv4 (as defined in IEEE Std 1588-2019 [126] Annex C), IPv6 (as defined in IEEE Std 1588-2019 [126] Annex D), Ethernet (as defined in IEEE Std 1588-2019 [126] Annex E). NOTE 15: Enumeration of supported PTP delay mechanisms. Allowed values as defined in IEEE Std 1588-2019 [126] clause 8.2.15.4.4. NOTE 16: Indicates whether DS-TT supports acting as a PTP grandmaster. NOTE 17: Indicates whether DS-TT supports acting as a gPTP grandmaster. NOTE 18: Enumeration of supported PTP profiles, each identified by PTP profile ID, as defined in IEEE Std 1588-2019 [126] clause 20.3.3. NOTE 19: PTP profile to apply, identified by PTP profile ID, as defined in IEEE Std 1588-2019 [126] clause 20.3.3. NOTE 20: Transport type to use. Allowed values: IPv4 (as defined in IEEE Std 1588-2019 [126] Annex C), IPv6 (as defined in IEEE Std 1588-2019 [126] Annex D), Ethernet (as defined in IEEE Std 1588-2019 [126] Annex E). NOTE 21: Indicates whether to act as grandmaster or not, i.e. whether to send Announce, Sync and optionally Follow_Up messages. NOTE 22: The IEEE Std 802.1AS [104] data sets apply if the IEEE 802.1AS PTP profile is used; otherwise the IEEE Std 1588-2019 [126] data sets apply. |  |  |  |  |

Table 5.28.3.1-2: Standardized bridge management information

| Bridge management information | Supported operations by TSN AF (see NOTE 1) | Reference |
| --- | --- | --- |
| Information for 5GS Bridge |  |  |
| Bridge Address | R |  |
| Bridge ID | R |  |
| NW-TT port numbers | R |  |
| Traffic forwarding information |  |  |
| Static Filtering Entry (NOTE 3) | RW | IEEE Std 802.1Q [98] clause 8.8.1 |
| General Neighbor discovery configuration (NOTE 2) |  |  |
| adminStatus | RW | IEEE Std 802.1AB [97] clause 9.2.5.1 |
| lldpV2LocChassisIdSubtype | RW | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2LocChassisId | RW | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2MessageTxInterval | RW | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2MessageTxHoldMultiplier | RW | IEEE Std 802.1AB [97] Table 11-2 |
| DS-TT port neighbor discovery configuration for DS-TT ports (NOTE 4) |  |  |
| >DS-TT port neighbor discovery configuration for each DS-TT port |  |  |
| >> DS-TT port number | RW |  |
| >> lldpV2LocPortIdSubtype | RW | IEEE Std 802.1AB [97] Table 11-2 |
| >> lldpV2LocPortId | RW | IEEE Std 802.1AB [97] Table 11-2 |
| Discovered neighbor information for DS-TT ports (NOTE 4) |  |  |
| >Discovered neighbor information for each DS-TT port (NOTE 4) |  |  |
| >> DS-TT port number | R |  |
| >> lldpV2RemChassisIdSubtype | R | IEEE Std 802.1AB [97] Table 11-2 |
| >> lldpV2RemChassisId | R | IEEE Std 802.1AB [97] Table 11-2 |
| >> lldpV2RemPortIdSubtype | R | IEEE Std 802.1AB [97] Table 11-2 |
| >> lldpV2RemPortId | R | IEEE Std 802.1AB [97] Table 11-2 |
| >> TTL | R | IEEE Std 802.1AB [97] clause 8.5.4.1 |
| Stream Parameters (NOTE 5) |  |  |
| MaxStreamFilterInstances | R | IEEE Std 802.1Q [98] |
| MaxStreamGateInstances | R | IEEE Std 802.1Q [98] |
| MaxFlowMeterInstances | R | IEEE Std 802.1Q [98] |
| SupportedListMax | R | IEEE Std 802.1Q [98] |
| Time synchronization information |  |  |
| Supported PTP instance types (NOTE 6) | R |  |
| Supported transport types (NOTE 7) | R |  |
| Supported delay mechanisms (NOTE 8) | R |  |
| PTP grandmaster capable (NOTE 9) | R |  |
| gPTP grandmaster capable (NOTE 10) | R |  |
| Supported PTP profiles (NOTE 11) | R |  |
| Number of supported PTP instances | R |  |
| Time synchronization information for DS-TT ports |  |  |
| > Time synchronization information for each DS-TT port |  |  |
| > DS-TT port number | RW |  |
| >> Time synchronization information for each PTP Instance |  |  |
| >> PTP Instance ID | RW |  |
| >> PTP profile (NOTE 12) | RW |  |
| >> Transport type (NOTE 13) | RW |  |
| >> Grandmaster on behalf of DS-TT enabled (NOTE 14) | RW |  |
| IEEE Std 1588-2019 [126] data sets (NOTE 15) |  |  |
| >> defaultDS.clockIdentity | RW | IEEE Std 1588-2019 [126] clause 8.2.1.2.2 |
| >> defaultDS.clockQuality.clockClass | RW | IEEE Std 1588-2019 [126] clause 8.2.1.3.1.2 |
| >> defaultDS.clockQuality.clockAccuracy | RW | IEEE Std 1588-2019 [126] clause 8.2.1.3.1.3 |
| >> defaultDS.clockQuality.offsetScaledLogVariance | RW | IEEE Std 1588-2019 [126] clause 8.2.1.3.1.4 |
| >> defaultDS.priority1 | RW | IEEE Std 1588-2019 [126] clause 8.2.1.4.1 |
| >> defaultDS.priority2 | RW | IEEE Std 1588-2019 [126] clause 8.2.1.4.2 |
| >> defaultDS.domainNumber | RW | IEEE Std 1588-2019 [126] clause 8.2.1.4.3 |
| >> defaultDS.sdoId | RW | IEEE Std 1588-2019 [126] clause 8.2.1.4.5 |
| >> defaultDS.instanceEnable | RW | IEEE Std 1588-2019 [126] clause 8.2.1.5.2 |
| >> defaultDS.externalPortConfigurationEnabled | RW | IEEE Std 1588-2019 [126] clause 8.2.1.5.3 |
| >> defaultDS.instanceType | RW | IEEE Std 1588-2019 [126] clause 8.2.1.5.5 |
| >> portDS.portIdentity | RW | IEEE Std 1588-2019 [126] clause 8.2.15.2.1 |
| >> portDS.portState | R | IEEE Std 1588-2019 [126] clause 8.2.15.3.1 |
| >> portDS.logMinDelayReqInterval | RW | IEEE Std 1588-2019 [126] clause 8.2.15.3.2 |
| >> portDS.logAnnounceInterval | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.1 |
| >> portDS.announceReceiptTimeout | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.2 |
| >> portDS.logSyncInterval | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.3 |
| >> portDS.delayMechanism | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.4 |
| >> portDS.logMinPdelayReqInterval | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.5 |
| >> portDS.versionNumber | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.6 |
| >> portDS.minorVersionNumber | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.7 |
| >> portDS.delayAssymetry | RW | IEEE Std 1588-2019 [126] clause 8.2.15.4.8 |
| >> portDS.portEnable | RW | IEEE Std 1588-2019 [126] clause 8.2.15.5.1 |
| >> timePropertiesDS.currentUtcOffset | RW | IEEE Std 1588-2019 [126] clause 8.2.4.2 |
| >> timePropertiesDS.timeSource | RW | IEEE Std 1588-2019 [126] clause 8.2.4.9 |
| >> externalPortConfigurationPortDS.desiredState | RW | IEEE Std 1588-2019 [126] clause 15.5.3.7.15.1 |
| IEEE Std 802.1AS [104] data sets (NOTE 15) |  |  |
| >> defaultDS.clockIdentity | RW | IEEE Std 802.1AS [104] clause 14.2.2 |
| >> defaultDS.clockQuality.clockClass | RW | IEEE Std 802.1AS [104] clause 14.2.4.2 |
| >> defaultDS.clockQuality.clockAccuracy | RW | IEEE Std 802.1AS [104] clause 14.2.4.3 |
| >> defaultDS.clockQuality.offsetScaledLogVariance | RW | IEEE Std 802.1AS [104] clause 14.2.4.4 |
| >> defaultDS.priority1 | RW | IEEE Std 802.1AS [104] clause 14.2.5 |
| >> defaultDS.priority2 | RW | IEEE Std 802.1AS [104] clause 14.2.6 |
| >> defaultDS.timeSource | RW | IEEE Std 802.1AS [104] clause 14.2.15 |
| >> defaultDS.domainNumber | RW | IEEE Std 802.1AS [104] clause 14.2.16 |
| >> defaultDS.externalPortConfigurationEnabled | RW | IEEE Std 802.1AS [104] clause 14.2.18 |
| >> defaultDS.sdoId | RW | IEEE Std 802.1AS [104] clause 14.2.4.3 |
| >> defaultDS.instanceEnable | RW | IEEE Std 802.1AS [104] clause 14.2.19 |
| >> portDS.portIdentity | RW | IEEE Std 802.1AS [104] clause 14.8.2 |
| >> portDS.portState | R | IEEE Std 802.1AS [104] clause 14.8.3 |
| >> portDS.ptpPortEnabled | RW | IEEE Std 802.1AS [104] clause 14.8.4 |
| >> portDS.delayMechanism | RW | IEEE Std 802.1AS [104] clause 14.8.5 |
| >> portDS.isMeasuringDelay | R | IEEE Std 802.1AS [104] clause 14.8.6 |
| >> portDS.asCapable | R | IEEE Std 802.1AS [104] clause 14.8.7 |
| >> portDS.meanLinkDelay | R | IEEE Std 802.1AS [104] clause 14.8.8 |
| >> portDS.meanLinkDelayThresh | RW | IEEE Std 802.1AS [104] clause 14.8.9 |
| >> portDS.delayAssymetry | RW | IEEE Std 802.1AS [104] clause 14.8.10 |
| >> portDS.neighborRateRatio | R | IEEE Std 802.1AS [104] clause 14.8.11 |
| >> portDS.initialLogAnnounceInterval | RW | IEEE Std 802.1AS [104] clause 14.8.12 |
| >> portDS.currentLogAnnounceInterval | R | IEEE Std 802.1AS [104] clause 14.8.13 |
| >> portDS.useMgtSettableLogAnnounceInterval | RW | IEEE Std 802.1AS [104] clause 14.8.14 |
| >> portDS.mgtSettableLogAnnounceInterval | RW | IEEE Std 802.1AS [104] clause 14.8.15 |
| >> portDS.announceReceiptTimeout | RW | IEEE Std 802.1AS [104] clause 14.8.16 |
| >> portDS.initialLogSyncInterval | RW | IEEE Std 802.1AS [104] clause 14.8.17 |
| >> portDS.currentLogSyncInterval | R | IEEE Std 802.1AS [104] clause 14.8.18 |
| >> portDS.useMgtSettableLogSyncInterval | RW | IEEE Std 802.1AS [104] clause 14.8.19 |
| >> portDS.mgtSettableLogSyncInterval | RW | IEEE Std 802.1AS [104] clause 14.8.20 |
| >> portDS.syncReceiptTimeout | RW | IEEE Std 802.1AS [104] clause 14.8.21 |
| >> portDS.syncReceiptTimeoutTimeInterval | RW | IEEE Std 802.1AS [104] clause 14.8.22 |
| >> portDS.initialLogPdelayReqInterval | RW | IEEE Std 802.1AS [104] clause 14.8.23 |
| >> portDS.currentLogPdelayReqInterval | R | IEEE Std 802.1AS [104] clause 14.8.24 |
| >> portDS.useMgtSettableLogPdelayReqInterval | RW | IEEE Std 802.1AS [104] clause 14.8.25 |
| >> portDS.mgtSettableLogPdelayReqInterval | RW | IEEE Std 802.1AS [104] clause 14.8.26 |
| >> portDS.initialLogGptpCapableMessageInterval | RW | IEEE Std 802.1AS [104] clause 14.8.27 |
| >> portDS.currentLogGptpCapableMessageInterval | R | IEEE Std 802.1AS [104] clause 14.8.28 |
| >> portDS.useMgtSettableLogGptpCapableMessageInterval | RW | IEEE Std 802.1AS [104] clause 14.8.29 |
| >> portDS.mgtSettableLogGptpCapableMessageInterval | RW | IEEE Std 802.1AS [104] clause 14.8.30 |
| >> portDS.initialComputeNeighborRateRatio | RW | IEEE Std 802.1AS [104] clause 14.8.31 |
| >> portDS.currentComputeNeighborRateRatio | R | IEEE Std 802.1AS [104] clause 14.8.32 |
| >> portDS.useMgtSettableComputeNeighborRateRatio | RW | IEEE Std 802.1AS [104] clause 14.8.33 |
| >> portDS.mgtSettableComputeNeighborRateRatio | RW | IEEE Std 802.1AS [104] clause 14.8.34 |
| >> portDS.initialComputeMeanLinkDelay | RW | IEEE Std 802.1AS [104] clause 14.8.35 |
| >> portDS.currentComputeMeanLinkDelay | R | IEEE Std 802.1AS [104] clause 14.8.36 |
| >> portDS.useMgtSettableComputeMeanLinkDelay | RW | IEEE Std 802.1AS [104] clause 14.8.37 |
| >> portDS.mgtSettableComputeMeanLinkDelay | RW | IEEE Std 802.1AS [104] clause 14.8.38 |
| >> portDS.allowedLostResponses | RW | IEEE Std 802.1AS [104] clause 14.8.39 |
| >> portDS.allowedFaults | RW | IEEE Std 802.1AS [104] clause 14.8.40 |
| >> portDS.gPtpCapableReceiptTimeout | RW | IEEE Std 802.1AS [104] clause 14.8.41 |
| >> portDS.versionNumber | RW | IEEE Std 802.1AS [104] clause 14.8.42 |
| >> portDS.nup | RW | IEEE Std 802.1AS [104] clause 14.8.43 |
| >> portDS.ndown | RW | IEEE Std 802.1AS [104] clause 14.8.44 |
| >> portDS.oneStepTxOper | R | IEEE Std 802.1AS [104] clause 14.8.45 |
| >> portDS.oneStepReceive | R | IEEE Std 802.1AS [104] clause 14.8.46 |
| >> portDS.oneStepTransmit | R | IEEE Std 802.1AS [104] clause 14.8.47 |
| >> portDS.initialOneStepTxOper | RW | IEEE Std 802.1AS [104] clause 14.8.48 |
| >> portDS.currentOneStepTxOper | RW | IEEE Std 802.1AS [104] clause 14.8.49 |
| >> portDS.useMgtSettableOneStepTxOper | RW | IEEE Std 802.1AS [104] clause 14.8.50 |
| >> portDS.mgtSettableOneStepTxOper | RW | IEEE Std 802.1AS [104] clause 14.8.51 |
| >> portDS.syncLocked | R | IEEE Std 802.1AS [104] clause 14.8.52 |
| >> portDS.pdelayTruncatedTimestampsArray | RW | IEEE Std 802.1AS [104] clause 14.8.53 |
| >> portDS.minorVersionNumber | RW | IEEE Std 802.1AS [104] clause 14.8.54 |
| >> timePropertiesDS.currentUtcOffset | RW | IEEE Std 802.1AS [104] clause 14.5.2 |
| >> externalPortConfigurationPortDS.desiredState | RW | IEEE Std 802.1AS [104] clause 14.12.2 |
| NOTE 1: R = Read only access; RW = Read/Write access. NOTE 2: General neighbor discovery information is included only when NW-TT performs neighbor discovery on behalf of DS-TT. When a parameter in this group is changed, it is necessary to provide the change to every DS-TT and the NW-TT that belongs to the 5GS TSN bridge. NOTE 3: If the Static Filtering Entry information is present for uplink traffic, UPF/NW-TT uses Static Filtering Entry information to determine the NW-TT egress port for forwarding UL TSC traffic. The user plane forwarding is further specified in clause 5.8.2.5.3. NOTE 4: DS-TT discovery configuration and DS-TT discovery information are used only when DS-TT does not support LLDP and NW-TT performs neighbor discovery on behalf of DS-TT. NOTE 5: TSN AF indicates the support for PSFP to the CNC only if each DS-TT and NW-TT of the 5GS bridge have indicated support of PSFP. The support of PSFP at the NW-TT ports is expressed by setting higher than zero values for MaxStreamFilterInstances, MaxStreamGateInstances, MaxFlowMeterInstances, SupportedListMax parameters. NOTE 6: Enumeration of supported PTP instance types. Allowed values as defined in IEEE Std 1588-2019 [126] clause 8.2.1.5.5. NOTE 7: Enumeration of supported transport types. Allowed values: IPv4 (as defined in IEEE Std 1588-2019 [126] Annex C), IPv6 (as defined in IEEE Std 1588-2019 [126] Annex D), Ethernet (as defined in IEEE Std 1588-2019 [126] Annex E). NOTE 8: Enumeration of supported PTP delay mechanisms. Allowed values as defined in IEEE Std 1588-2019 [126] clause 8.2.15.4.4. NOTE 9: Indicates whether NW-TT supports acting as a PTP grandmaster. NOTE 10: Indicates whether NW-TT supports acting as a gPTP grandmaster. NOTE 11: Enumeration of supported PTP profiles, each identified by PTP profile ID, as defined in IEEE Std 1588-2019 [126] clause 20.3.3. NOTE 12: PTP profile to apply, identified by PTP profile ID, as defined in IEEE Std 1588-2019 [126] clause 20.3.3. NOTE 13: Transport type to use. Allowed values: IPv4 (as defined in IEEE Std 1588-2019 [126] Annex C), IPv6 (as defined in IEEE Std 1588-2019 [126] Annex D), Ethernet (as defined in IEEE Std 1588-2019 [126] Annex E). NOTE 14: Indicates whether to act as grandmaster on behalf of a DS-TT port or not if 5GS is determined to be the grandmaster clock, i.e. whether to send Announce, Sync and optionally Follow_Up messages on behalf of DS-TT. NOTE 15: The IEEE Std 802.1AS [104] data sets apply if the IEEE 802.1AS PTP profile is used; otherwise the IEEE Std 1588-2019 [126] data sets apply. |  |  |

Exchange of port and bridge management information between TSN AF or NEF and NW-TT or between TSN AF or NEF and DS-TT allows TSN AF or NEF to:

1)	retrieve port management information for a DS-TT or NW-TT port or bridge management information;

2)	send port management information for a DS-TT or NW-TT port or bridge management information;

3)	subscribe to and receive notifications if specific port management information for a DS-TT or NW-TT port changes or bridge management information changes.

Exchange of port management information between TSN AF or NEF and NW-TT or DS-TT is initiated by DS-TT or NW-TT to:

-	notify TSN AF if port management information has changed that TSN AF has subscribed for.

Exchange of bridge management information between TSN AF and NW-TT is initiated by NW-TT to:

-	notify TSN AF or NEF if bridge management information has changed that TSN AF or NEF has subscribed for.

Exchange of port management information is initiated by DS-TT to:

-	provide port management capabilities, i.e. provide information indicating which standardized and deployment-specific port management information is supported by DS-TT.

TSN AF or NEF indicates inside the Port Management Information Container or Bridge Management Information Container whether it wants to retrieve or send port or bridge management information or intends to (un-)subscribe for notifications.

*** Next changes ***

### K.2.2.4	Configuration for PTP grandmaster function

The following options may be supported (per DS-TT) for the 5GS to generate the Sync, Follow_Up and Announce messages for the Master ports on the DS-TT:

a)	NW-TT generates the Sync, Follow_Up and Announce messages on behalf of DS-TT (e.g. if DS-TT does not support this).

b)	DS-TT generates the Sync, Follow_Up and Announce messages in this DS-TT.

TSN AF and NEF may use the elements in port and bridge management information container to determine the PTP grandmaster functionality supported by DS-TT and NW-TT and may configure the DS-TT and NW-TT ports to operate as in option a) or b) as follows:

-	The "PTP grandmaster capable" element and the "gPTP grandmaster capable" element in PMIC are used to indicate the support for PTP or gPTP grandmaster capability, respectively, in each DS-TT. If the TSN AF or NEF determines the DS-TT supports grandmaster capability (PTP or gPTP grandmaster capable is TRUE), then either option a) or b) can be used for the PTP instance(s) in the DS-TT. Otherwise, only option a) can be used for the PTP instance(s) in the DS-TT.

-	To enable option a) for PTP ports in DS-TT, the TSN AF or NEF sets the element "Grandmaster on behalf of DS-TT enabled" TRUE (per PTP instance per DS-TT) in BMIC for the respective DS-TT port, and the TSN AF or NEF sets the element "Grandmaster enabled" FALSE (per PTP instance per DS-TT) in PMIC to the respective DS-TT port.

-	To enable option b) for PTP ports in DS-TT, the TSN AF or NEF sets the element "Grandmaster on behalf of DS-TT enabled" FALSE in BMIC (per PTP instance per DS-TT) for the respective port, and the TSN AF or NEF sets the element "Grandmaster enabled" TRUE (per PTP instance per DS-TT) in PMIC to the respective DS-TT port.

-	To enable either option a) or option b) for a PTP instance, the TSN AF or NEF sets the element "Grandmaster candidate enabled" TRUE (per PTP instance) in PMIC.

-	When option b) is used for one or more PTP ports in DS-TT(s), the TSN AF or NEF shall use the elements in defaultDS in PMIC for the respective DS-TT(s) and in BMIC for NW-TT to ensure that all PTP ports in the DS-TT(s) and NW-TT in particular PTP instance are distributing the same values of grandmasterPriority1, grandmasterClockQuality, grandmasterPriority2, grandmasterIdentity, and timeSource message fields in Announce messages.

*** End of Changes ***