3GPP TSG-WG SA2 Meeting #145E e-meeting	S2-2104697

# Elbonia, May 17 – May 28, 2021	(revision of S2-2103260)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0326 | rev | 2 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | User Data congestion analytics update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Care has to be taken with regards to load and major signalling caused when requesting "any UE". The burden on UPF may be quite high when collecting the required data to generate the "List of top applications". |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new "Sampling ratio" parameter collected by UPF, providing information on the achieved sampling ratio. Allow the UPF to apply data sampling to avoid an overload situation. Additional editorial corrections. Add a statement that "The data collection from UPF shall be configurable with application IDs, Packet Filter Sets, and/or PFDs to reduce the system load from this data collection." |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current specification might result in extremely high UPF load and 5GS signalling load. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8.2, 6.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * * *

### 6.8.2	Input data

The detailed information collected by the NWDAF is defined in Table 6.8.2-1.

NOTE 1:	Performance Measurements defined in TS 28.552 [8] represent resource utilisation but do not, by themselves, indicate the event of congestion or congestion levels. The NWDAF collects measurements from the OAM and how the NWDAF derives Network Status Indication (NSI) is not specified.

Table 6.8.2-1: Data Collected from the NF and OAM related to User Data Congestion Analytics

| Information | Source | Description |
| --- | --- | --- |
| UE Location | AMF | UE location information that NWDAF can use to derive the Area of Interest. |
| Measurements | OAM | Performance Measurements that will be used by the NWDAF to determine congestion levels. Performance Measurements are related to information transfer over the user plane and/or the control plane (e.g. UE Throughput, DRB Setup Management, RRC Connection Number, PDU Session Management, and Radio Resource Utilization as defined in TS 28.552 [8]). The NWDAF may obtain measurements by invoking management services that are defined in TS 28.532 [6] and TS 28.550 [7]. |

Table 6.8.2-2: Data Collected from the UPF or from the AF related to User Data Congestion Analytics

| Information | Source | Description |
| --- | --- | --- |
| Application ID | UPF or AF | Application identifier as defined in TS 23.501 [2] clause 5.8.2 (see NOTE 1, NOTE 2). |
| IP Packet Filter Set | UPF or AF | IP Packet Filter set as defined in TS 23.501 [2] clause 5.8.2 (see NOTE 1, NOTE 2). |
| Measurement period | UPF or AF | Measurement period. |
| Throughput UL/DL | UPF or AF | Average Throughput UL/DL over the measurement period. |
| Throughput UL/DL (peak) | UPF or AF | Peak Throughput UL/DL over the measurement period. |
| Timestamp | UPF or AF | Time when measurements are taken. |
| Achieved sampling ratio | UPF | Sampling ratio achieved by UPF (see NOTE 3). |
| NOTE 1: Application Id and IP Packet Filter Set are mutually exclusive. NOTE 2: The data collection from UPF shall be configurable with application IDs, Packet Filter Sets, and/or PFDs to reduce the system load from this data collection. This configuration is functionally equivalent to the information inside the Packet Detection Rules (PDRs). NOTE 3: UPF may apply data sampling to reduce the load on the UPF. This parameter is provided when no sampling ratio is configured at the UPF or the UPF could not fulfil the configured sampling ratio. NOTE 4: Multiple outputs are provided by the UPF when multiple applications are running at the UPF for the same UE and measurement period. |  |  |

NOTE 2:	Care has to be taken with regards to load and major signalling caused when requesting "any UE". This can be achieved via utilization of event filters (e.g. Area of Interest), Analytics Reporting Information (e.g. maximum number of objects), or preferred sampling ratio provided by NWDAF to the UPF, and/or configuration of UPF data collection for specific application IDs, Packet Filter Sets, and/or PFDs.

NOTE 3:	How NWDAF subscribes to data collection from UPF and how NWDAF collects information from UPF is not defined in this Release of the specification.

Additionally, NWDAF may use statistics or predictions on service experience as specified in clause 6.4.3 as an input, e.g. for service experience in a given area or service experience for some specific applications such as high bandwidth applications.

### 6.8.3	Output analytics

The NWDAF outputs the user data congestion analytics for transfer over the user plane, for transfer over the control plane, or for both. The output may consist of statistics, predictions, or both. The detailed information provided by the NWDAF is defined in Table 6.8.3-1 for statistics and in Table 6.8.3-2 for predictions.

Table 6.8.3-1: User Data Congestion statistics

| Information | Description |
| --- | --- |
| Area of Interest | A list of TAIs or Cell IDs |
| List of user data congestion Analytics (1..max) | List of user data congestion Analytics. See NOTE 5. |
| > Type | User Plane or Control Plane |
| > Applicable Time Window (NOTE 4) | The time period that the analytics applies to |
| > Network Status Indication (NOTE 1, NOTE 4) | Congestion Level |
| > List of top applications in UL (0..NU) (NOTE 1) | The list of applications that contribute the most to the traffic in the UL direction. |
| >> Application ID | Application identifier as defined in TS 23.501 [2] clause 5.8.2 (see NOTE 2). |
| >> IP Packet Filter Set | IP Packet Filter set as defined in TS 23.501 [2] clause 5.8.2 (see NOTE 2). |
| >> Percentage | The application's throughput as a percentage of the total throughput in the Area of Interest. |
| > List of top applications in DL (0..ND) (NOTE 1) | The list of applications that contribute the most to the traffic in the DL direction. |
| >> Application ID | Application identifier as defined in TS 23.501 [2] clause 5.8.2 (see NOTE 2) |
| >> IP Packet Filter Set | IP Packet Filter set as defined in TS 23.501 [2] clause 5.8.2 (see NOTE 2). |
| >> Percentage | The application's throughput as a percentage of the total throughput in the Area of Interest. |
| NOTE 1: This information element is an Analytics subset that can be used in "list of analytics subsets that are requested". NOTE 2: Application Id and IP Packet Filter Set are mutually exclusive. NOTE 3: The listed applications are not necessarily ranked by any order of traffic contribution. NOTE 4: Analytics subset that can be used in "Accuracy level per analytics subset". NOTE 5: The number of user data congestion analytics entries is limited by the maximum number of objects provided as part of Analytics Reporting Information. |  |

Table 6.8.3-2: User Data Congestion predictions

| Information | Description |
| --- | --- |
| Area of Interest | A list of TAIs or Cell IDs. |
| List of user data congestion Analytics (1..max) | List of user data congestion Analytics. See NOTE 5. |
| > Type | User Plane or Control Plane. |
| > Applicable Time Window (NOTE 4) | The time period that the analytics applies to. |
| > Network Status Indication (NOTE 1, NOTE 4) | Congestion Level. |
| > Confidence | Confidence of this prediction. |
| > List of top applications in UL (0..NU) (NOTE 1) | The list of applications predicted to contribute most of the traffic in the UL direction. |
| >> Application ID | Application identifier as defined in TS 23.501 [2] clause 5.8.2 (see NOTE 2). |
| >> IP Packet Filter Set | IP Packet Filter set as defined in TS 23.501 [2] clause 5.8.2 (see NOTE 2). |
| >> Percentage | The application's throughput as a percentage of the total throughput in the Area of Interest. |
| >> Confidence | Confidence of this prediction. |
| > List of top applications in DL (0..ND) (NOTE 1) | The list of applications predicted to contribute most of the traffic in the DL direction. |
| >> Application ID | Application identifier as defined in TS 23.501 [2] clause 5.8.2 (see NOTE 2). |
| >> IP Packet Filter Set | IP Packet Filter set as defined in TS 23.501 [2] clause 5.8.2 (see NOTE 2). |
| >> Percentage | The application's throughput as a percentage of the total throughput in the Area of Interest. |
| >> Confidence | Confidence of this prediction. |
| NOTE 1: This information element is an Analytics subset that can be used in "list of analytics subsets that are requested". NOTE 2: Application Id and IP Packet Filter Set are mutually exclusive. NOTE 3: The listed applications are not necessarily ranked by any order. NOTE 4: Analytics subset that can be used in "Accuracy level per analytics subset". NOTE 5: The number of user data congestion analytics entries is limited by the maximum number of objects provided as part of Analytics Reporting Information. |  |

* * * End of Changes * * * *