3GPP TSG SA WG2 Meeting #145E (e-meeting)		S2-2104780

May 17-28 2021

Title:	[DRAFT] LS Response to LS on multiple TACs per PLMN

Response to:	S2-2103770 / R2-2104377

Release:	Rel-17

Work Item:	5GSAT_ARCH

Source:	SA2

To:	RAN2, CT1

Cc:	RAN3

Contact person:	Stephen Edge

sedge AT qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2104774, S2-2104777

# 1	Overall description

SA2 thanks RAN2 for the LS on multiple TACs per PLMN. SA2 notes that RAN2 has included the following information and a question in this LS:

- The network may broadcast more than one TACs per PLMN in a cell.

- Option 1: AS still reports only one TAC for one PLMN even if more than one TACs per PLMN are broadcasted in an NTN cell.

- Option 2: AS indicates all received TAC(s) for one PLMN to NAS layer.

- Majority of companies think option 1 has larger RAN2 impact, as AS layer has to select one TAC from the broadcast ones to report to NAS layer, thus RAN2 has preference for option 2

- Is option 2, i.e. “AS indicates all received TAC(s) for one PLMN to NAS layer”, feasible from a NAS layer perspective?

SA2 has agreed CRs to TS 23.501 and TS 23.502, as attached, to support broadcast of more than one TAC per PLMN in a cell, where AS indicates all received TAC(s) for one PLMN to the NAS layer as in Option 2. SA2 thus believes that Option 2 is feasible, although there is a small impact to the NAS layer as in the CRs.

For Option 1, SA2 agrees that reporting of only one TAC for one PLMN by AS to NAS would require no change to NAS. However, it is not clear whether and how the NAS layer could be informed of other TACs, e.g. in the case that a reported TAC is not in a UE registration area but another unreported TAC is. SA2 thus questions the effectiveness and feasibility of Option 1.

# 2	Actions

To RAN2 and CT1

ACTION: 	SA2 asks RAN2 and CT1 to take the above answers into account.

# 3	Dates of next TSG SA WG2 meetings

SA2#146E		16-27 August 2021		Electronic Meeting

SA2#147E		18-22 October 2021		Electronic Meeting