3GPP TSG-WG SA2 Meeting #145 e-meeting 	S2-2104813

17-28 May 2021, Elbonia

# Title:	Reply LS on N3IWF FQDN for emergency service

# Reply to:	LS on N3IWF FQDN for emergency service (S2-2102109/C4-211521)

# Release:	Rel-17

# Work Item:	TEI17

Source:	SA2

To:	CT4, CT1

Cc:	-

Contact Person:

Name:	Sebastian Speicher

Tel. Number:

E-mail Address:	speicher@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	S2-2104548 (TS 23.501 CR#2848)

# 1.	Overall Description:

SA2 would like to thank CT4 for the LS on N3IWF FQDN for emergency service. SA2 agreed the attached CR to clarify that separate Visited Country emergency FQDNs are expected to be used for ePDG and N3IWF selection. In addition SA2 would like to provide the following feedback to CT4's questions:

CT4 question 1): Which one does the highlighted "Operator Identifier Emergency FQDN" and "Visited Country Emergency FQDN" refer to?

a) emergency ePDG FQDN

b) either emergency ePDG FQDN or emergency N3IWF FQDN

SA2 reply: SA2 concluded that the configured Operator Identifier Emergency FQDN only refers to the configured Emergency FQDN for ePDG, i.e. that there is no need for a configured Emergency FQDN for N3IWF. The Visited Country Emergency FQDN refers to separate FQDNs for ePDG and N3IWF selection, respectively. SA2 clarified this in the attached CR.

Question 2): If the answer to Q1) is "b", why does emergency N3IWF FQDN need to be defined?

SA2 reply: Re-using the existing visited country emergency FQDN that was defined for ePDG selection also for N3IWF selection for emergency services may lead to backwards compatibility issues for UEs that only support ePDG connectivity (i.e. UEs which do not support N3IWF connectivity) as those UEs are expecting to only receive information about PLMNs that provide emergency services via ePDG when performing the DNS query for the Visited Country Emergency FQDN.

Therefore SA2 decided to clarify that the UE uses a separate visited country emergency FQDN for N3IWF selection for emergency services.

# 2.	Actions:

To CT4 and CT1

ACTION:

SA WG2 kindly requests CT4 and CT1 to take the above into account.

# 3.	Date of Next TSG SA WG2 Meetings:

3GPPSA2#146-e	TBD	Electronic Meeting

3GPPSA2#147-e	TBD	Electronic Meeting