SA WG2 Meeting #145E e-meeting	S2-2104941

Elbonia, May 17 – 28, 2021	S2-2104209r03

Source:	vivo

Title:	TS 23.304: Updates to Policy/Parameter provisioning for ProSe UE-to-Network Relay

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	5G_ProSe / Rel-17

Abstract: This contribution is to update the Policy/Parameter provisioning for ProSe UE-to-Network Relay.

1.	Discussion

Considering the UEs may have different ProSe relay capabilities, thus the Policy/Parameter provisioning for ProSe UE-to-Network Relay also need be classified into L2 and/or L3.

2.	Proposal

It is proposed to capture the following changes into TS 23.304.

# * * * * First change * * * *

#### 5.1.4.1	Policy/Parameter provisioning for ProSe UE-to-Network Relay

The following information is provisioned in the UE in support of the UE assuming the role of a ProSe UE-to-Network Relay:

1)	Authorisation policy for acting as a L3 and/or L2 ProSe UE-to-Network Relay when "served by NG-RAN":

-	PLMNs in which the UE is authorized to relay traffic for L3 and/or L2 Remote UEs.

2)	ProSe Relay Discovery policy/parameters for ProSe UE-to-Network Relay:

-	Includes the parameters that enable the UE to perform ProSe Relay Discovery when provisioned from the PCF in the ME or configured in the UICC:

-	ProSe UE-to-Network Relay Discovery parameters (User Info ID, Relay Service Code(s));

-	For Layer 3 ProSe UE-to-Network Relay, the PDU Session parameters (PDU Session type, DNN, SSC Mode, S-NSSAI, Access Type Preference) to be used for the relayed traffic for each ProSe Relay Service Code;

Editor's note:	Whether the associations between PDU session parameters and ProSe Relay Service Codes are provided to Relay UE and Remote UE as ProSe policy/parameters or as USRP rules will be decided at normative phase. Whether Remote UE provides other assistant info for handling PDU session parameters, e.g. PDU session requirement or Traffic descriptors to Relay UE will be decided at normative phase. The security and privacy aspects of this procedure will be coordinated with SA WG3.

-	Includes security related content for ProSe Relay Discovery for each ProSe Relay Service Code.

Editor's note:  Whether the security parameters can be provided by the PCF and details of security parameters will be determined by SA3 WG.

NOTE 2:	ProSe Relay Discovery policy/parameters can be provided from ProSe Application Server to the ProSe UE-to-Network Relay.

3)	For Layer 3 ProSe UE-to-Network Relay, QoS mapping rules:

-	Includes the rules that determine how the ProSe UE-to-Network Relay maps the 5QI of the QoS flow into a 5G ProSe PQI value over NR PC5;

The following information is provisioned in the UE in support of the UE assuming the role of a Remote UE and thereby enabling the use of a ProSe UE-to-Network Relay:

1)	Authorisation policy for using a L3 and/or L2 ProSe UE-to-Network Relay:

-	Indicates whether the UE is authorised to use a L3 and/or L2 ProSe UE-to-Network Relay.

2)	Policy/parameters for ProSe Relay Discovery:

-	Includes the parameters for ProSe Relay Discovery and for enabling the UE to connect to the ProSe UE-to-Network Relay after discovery when provisioned from the PCF in the ME or configured in the UICC:

-	ProSe UE-to-Network Relay Discovery parameters (User Info ID, Relay Service Code(s));

-	For Layer 3 ProSe UE-to-Network Relay, the PDU Session parameters (PDU Session type, DNN, SSC Mode, S-NSSAI, Access Type Preference) to be used for the relayed traffic for each ProSe Relay Service Code;

-	Includes security related content for ProSe Relay Discovery for each ProSe Relay Service Codes.

Editor's note:  Whether the security parameters can be provided by the PCF and details of security parameters will be determined by SA3 WG.

NOTE 4:	ProSe Relay Discovery policy/parameters can be provided from ProSe Application Server to the Remote UE.

The following information is provisioned in the UE in support of the UE assuming the role of a ProSe UE-to-Network Relay as well as in the UE in support of the UE assuming the role of a Remote UE and thereby enabling the use of a ProSe UE-to-Network Relay:

1)	Radio parameters for ProSe Relay Discovery when the UE is not "served by NG-RAN ":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform ProSe Direct Discovery over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

2)	Radio parameters for ProSe Relay Communication when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform ProSe Direct Communication over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

# * * * * End of changes * * * *