3GPP TSG-WG SA2 Meeting #145E e-meeting  	S2-2105005

# Elbonia, May 17 – 28, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2991 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding PDU session limitation and protocol stacks for trusted WLAN access for N5CW device |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC, TEI17 |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In current specification, the indication that for N5CW device only the PDU session of type IP is supported in this release is missing, hence the TWIF may be configured to start an PDU session of type Ethernet which is currently not supported causing to start a service which will be never used by the N5CW device and from oteh rhand all user traffic send by N3CW wil be discarged and lost by TWIF. Furthermore there are neither control plane nor user plane protocol stacks for trusted WLAN access for N5CW device. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the control and user plane protocol stacks for trusted WLAN access for N5CW device. In this release of the specification, Trusted WLAN Access for N5CW Device only supports IP PDU Session type |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Both control and user plane protocol stacks for trusted WLAN access for N5CW device are missing. The UE may assume a non IP PDU Session Type can be provided while the network provides only IP PDU Session type and the network is not able to tell the UE that it can’t deliver the PDU Session type assumed by the UE. The absence of a protocol stack may mean that ecah of the Ue and network inmplementation assume a different stack, leading to the impossibility to have proper data exchange |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.8.5 ; 8.2.x (new), 8.3.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 4.2.8.5	Access to 5GC from devices that do not support 5GC NAS over WLAN access

##### 4.2.8.5.1	General

The devices that do not support 5GC NAS signalling over WLAN access are referred to as "Non-5G-Capable over WLAN" devices, or N5CW devices for short. A N5CW device is not capable to operate as a 5G UE that supports 5GC NAS signalling over a WLAN access network, however, it may be capable to operate as a 5G UE over NG-RAN.

Clause 4.2.8.5 specifies the 5GC architectural enhancements that enable N5CW devices to access 5GC via trusted WLAN access networks. A trusted WLAN access network is a particular type of a Trusted Non-3GPP Access Network (TNAN) that supports a WLAN access technology, e.g. IEEE 802.11. Not all trusted WLAN access networks support 5GC access from N5CW devices. To support 5GC access from N5CW devices, a trusted WLAN access network must support the special functionality specified below (e.g. it must support a TWIF function).

When a N5CW device performs an EAP-based access authentication procedure to connect to a trusted WLAN access network, the N5CW device may simultaneously be registered to a 5GC of a PLMN. The 5GC registration is performed by the TWIF function (see next clause) in the trusted WLAN access network, on behalf of the N5CW device. The type of EAP authentication procedure, which is used during the 5GC registration to authenticate the N5CW device, is specified in TS 33.501 [29]. In this release of the specification, Trusted WLAN Access for N5CW Device only supports IP PDU Session type.

# * * * * Second change * * * *

### 8.2.x	Control Plane for Trusted WLAN Access for N5CW Device

Figure 8.2.x-1: Control Plane for trusted WLAN access for N5CW device

The EAP protocol applies only for performing EAP-based access authentication procedure to connect to a trusted WLAN access network.

# * * * * Third change * * * *

### 8.3.y	 User Plane for Trusted WLAN Access for N5CW Device

Transport

Non-3GPP (WLAN)

Non-3GPP (WLAN)

Lower layers

Lower layers

N3 Stack

N3 Stack

N5CW Device

TWAP

TWIF

UPF

Yt'

Yw

N3

N9 Stack

Relay

N9 Stack

N9

UPF (PDU Session Anchor)

PDU Layer

PDU Layer

Transport

Transport

Relay

Application

N6

Legend:

-	Transport: this layer refers to the transport of PDUs between the N5CW device and TWIF (see clause 4.2.8.5.4).

In this release of the specification, Trusted WLAN Access for N5CW Device only supports IP PDU Session type.

Figure 8.2.y-1: User Plane for trusted WLAN access for N5CW device

# * * * * End of changes * * * *