SA WG2 Meeting #145E e-meeting	S2-2105011

Elbonia, May 17 – 28, 2021	  (revision of S2-2104443)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2820 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add a new NSSAAF service to support interaction between AUSF and AAA Server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Ericsson, Huawei, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  |  | Date: |  | 2021-05-10 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In order to solve the following FFS related to interaction between AUSF and AAA Server: Editor’s note: Whether an intermediate function is needed between the AUSF and the AAA-S is FFS. Editor’s note: the Interface between AAA Server and SNPN is FFS. It is proposed to re-use the NSSAAF between AUSF and AAA Server. To support the function, a new service (i.e. Nnssaaf_AIW_Authenticate) is introduced in this CR. To make NSSAAF works in network slice scenario and SNPN scenario with no ambiguity, the name of NSSAAF is changed to Network Slice-specific and SNPN Authentication and Authorization Function. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new NSSAAF service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The interface between AAA Server and SNPN is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.9.1, 5.2.20.1, 5.2.20.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

4.2.9.1	General

The Network Slice-Specific Authentication and Authorization procedure is triggered for an S-NSSAI requiring Network Slice-Specific Authentication and Authorization with an AAA Server (AAA-S) which may be hosted by the H-PLMN operator or by a third party which has a business relationship with the H-PLMN, using the EAP framework as described in TS 33.501 [15]. An AAA Proxy (AAA-P) in the HPLMN may be involved e.g. if the AAA Server belongs to a third party.

This procedure is triggered by the AMF during a Registration procedure when some Network Slices require Slice-Specific Authentication and Authorization, when AMF determines that Network Slice-Specific Authentication and Authorization is requires for an S-NSSAI in the current Allowed NSSAI (e.g. subscription change), or when the AAA Server that authenticated the Network Slice triggers a re-authentication.

The AMF performs the role of the EAP Authenticator and communicates with the AAA-S via the Network Slice -specific and SNPN Authentication and Authorization Function (NSSAAF). The NSSAAF undertakes any AAA protocol interworking with the AAA protocol supported by the AAA-S.

The Network Slice-Specific Authentication and Authorization procedure requires the use of a GPSI. In other words, a subscription that contains S-NSSAIs subject to Network Slice-Specific Authentication and Authorization shall include at least one GPSI.

* * * * Second Change * * * *

5.2.20.1	General

The following table illustrates the NSSAAF Services.

Table 5.2.20-1: List of NSSAAF Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnssaaf_NSSAA | Authenticate | Request/Response | AMF |
|  | Re-AuthenticationNotification | Notify | AMF |
|  | RevocationNotification | Notify | AMF |
| Nnssaaf_AIW | Authenticate | Request/Response | AUSF |

* * * * Third Change * * * *

5.2.20.x	Nnssaaf_AIW service

5.2.20.x.1	General

Service Description: The NSSAAF provides Authentication and Authorization service to the requester NF by relaying EAP messages towards a AAA-S or AAA-P and performing related protocol conversion as needed.

5.2.20.x.2	Nnssaaf_AIW_Authenticate service operation

See TS 33.501 [15].

* * * * End of Changes * * * *