SA WG2 Meeting #145E	S2-2105162

17 – 28 May, 2021, Electronic meeting	(was S2-2104228r02)

Source:	Samsung, ZTE

Title:	Update on removal of MBS session configuration

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract: This document updates the removal of MBS session configuration procedure to incorporate broadcast session release procedure including PCF-related steps.

- Introduction

This document modifies the removal of MBS session configuration procedure to incorporate broadcast session release procedure including PCF-related steps.

- Proposal

It is proposed to capture the following changes in TS23.247.

* * * * Start of 1st Change * * * *

7.1.1.2	removal of MBS session configuration

The removal of configuration steps for MBS Session are used by the AF to stop the MBS Session towards 5GC and consist of TMGI de-allocation and session stop procedures, and they apply to both multicast and broadcast communications unless otherwise stated. MBS session release/deactivation procedure may follow removal of the MBS session configuration to release the reserved resources toward NG-RAN.

Figure 7.1.1.2-1: Removal of configuration for MBS Session

Editor´s note:	MBSTF could optionally be in the user plane and the call flow should be updated and aligned with the MBS session configuration procedure.

Editor's note:	The services and messages used in this procedure are FFS.

Editor's note:	Additional interaction between AF/NEF and PCF are FFS.

1.	AF of content provider may request stop contents for the MBS session (MBS Session ID) to NEF.

If dynamic PCC is deployed, the steps 7, 8 and 10 are skipped, otherwise, the steps 2, 3, 4, 5 and 6 are skipped.

Editor’s Note: Whether MBS Policy Association Release Notify can be used to release the MBS Session is FFS, in another word, whether MB-SMF should terminate the MBS policy association regardless of dynamic PCC deployment is FFS.

2.	The NEF/MBSF sends an MBS Session Stop Request (MBS Session ID) message to the PCF that handles the Policy of the MBS Session.

3. The PCF sends message to MB-SMF to release the MBS Policy Association.

4. The MB-SMF sends the MBS Policy Association Release Notify Response (TMGI) message to the PCF.

5.	The PCF de-registers at the BSF that it handles the multicast session.

6a/b.	MB-SMF requests the MB-UPF to release user plane ingress resources.

7.	NEF/MBSF requests MB-SMF to release ingress resources for the MBS distribution session.

8a/b.	MB-SMF requests the MB-UPF to release user plane ingress resources.

9.	[Conditional] If MB-SMF configured the profile with an MBS session ID when the MBS session was configured, the MB-SMF updates its NF profile at NRF to release the MBS Session ID.

10.	MB-SMF responds to the NEF/MBSF.

11.	The NEF/MBSF responds to the AF.

12/13.	[Optional] AF requests NEF/MBSF to de-allocate TMGI(s), and NEF/MBSF forwards request to MB-SMF

NOTE:	Depending on the configuration, MB-SMF may receive requests from AF directly, or via NEF, or via MBSF, or via NEF and MBSF.

14/15.	The MB-SMF responds to the NEF or MBSF and to the AF by sending a de-allocate TMGI Response message.

* * * * End of Changes * * * *