3GPP TSG-SA WG2 Meeting #145E (e-meeting)	S2-2105197

# May 17 – May 28, 2021, Elbonia	(revision of S2-2103303, 4526r01)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2721 | rev | 5 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Send PMF messages to a target QoS flow |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Motorola Mobility, LG Electronics, Broadcom, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When the UE and the UPF decide to perform access performance measurements per QoS flow, it is not specified how they can send PMF messages to a target QoS flow. Further details are provided in S2-2102307 (see open issue 3) and in S2-2104525. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is specified that, when the SMF determines that access performance measurements per QoS flow shall be applied for the MA PDU Session, then the Measurement Assistance Information includes a list of QoS flows on which access performance measurements may be performed. For each QoS flow in this list, addressing information is provided, e.g. UDP ports or MAC addresses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not specified how the UE and UPF can send PMF messages to a target QoS flow. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.32.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This is a revision of S2-2103303 (2721rev1) approved in SA2#144e. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

### 5.32.5	Access Network Performance Measurements

#### 5.32.5.1	General principles

When an MA PDU Session is established, the network may provide the UE with Measurement Assistance Information. This information assists the UE in determining which measurements shall be performed over both accesses, as well as whether measurement reports need to be sent to the network.

Measurement Assistance Information shall include the addressing information of a Performance Measurement Function (PMF) in the UPF, the UE can send PMF protocol messages to:

-	For a PDU Session of IP type, Measurement Assistance Information contains one IP address for the PMF, one UDP port associated with 3GPP access and another UDP port associated with non-3GPP access. PMF messages sent by UE to one of these UDP ports, shall be transmitted to UPF via the default QoS flow.

-	For a PDU Session of Ethernet type, Measurement Assistance Information contains one MAC address associated with 3GPP access and another MAC address associated with non-3GPP access. PMF messages sent by UE to one of these MAC addresses, shall be transmitted to UPF via the default QoS flow.

NOTE 1:	To protect the PMF in the UPF (e.g. to block DDOS to the PMF), the IP addresses of the PMF are only accessible from the UE IP address via the N3/N9 interface.

NOTE 2:	After the MA PDU Session is released, the same UE IP address/prefix is not allocated to another UE for MA PDU Session in a short time.

If the SMF determines that access performance measurements per QoS flow shall be applied for the MA PDU Session, then the Measurement Assistance Information shall also include a list of QoS flows on which access performance measurements may be performed. For each QoS flow in this list, the following information is included:

-	The QFI of the associated QoS flow.

-	For a PDU Session of IP type, one UDP port associated with 3GPP access and another UDP port associated with non-3GPP access. PMF messages sent by UE to one of these UDP ports, shall be transmitted to UPF via the associated QoS flow.

-	For a PDU Session of Ethernet type, one MAC address associated with 3GPP access and another MAC address associated with non-3GPP access. PMF messages sent by UE to one of these MAC addresses, shall be transmitted to UPF via the associated QoS flow.

The QoS rules and the N4 rules provided by SMF to UE and to UPF respectively shall include information (e.g. packet filters containing the UDP port or the MAC address associated with a QoS flow), which enables the UE and UPF to route a PMF message to a specific QoS flow.

The UE and the UPF may need to perform access performance measurements in order to estimate the Round-Trip Time (RTT) and/or the Packet Loss Rate (PLR) that an SDF is expected to experience when transmitted on a certain access type. Based on these measurements and the provisioned ATSSS rules in the UE and MAR rules in the UPF, the UE and the UPF decide how to distribute the traffic of an SDF across the two accesses.

If the UE and the UPF decide to initiate access performance measurements to estimate the RTT and/or the PLR for an SDF, the access performance measurements shall be performed either

(a)	using the default QoS Flow, or

(b)	using the target QoS Flow, which is the QoS Flow that the SDF traffic is transmitted on.

The UE shall perform access performance measurements based on the target QoS Flow only when this is explicitly indicated in the Measurement Assistance Information. Otherwise, the UE shall perform access performance measurements based on the default QoS Flow. The UPF shall perform access performance measurements based on the target QoS Flow only when this is explicitly indicated by SMF. Otherwise, the UPF shall perform access performance measurements based on the default QoS Flow.

Editor's note:	It is FFS how the Measurement Assistance Information indicates whether access performance measurements shall be performed based on (a) or (b). For example, (b) will be used when the Measurement Assistance Information contains different PMF addresses/ports for different QoS Flows, or when the Measurement Assistance Information contains the same PMF address/port for all QoS Flows and a new indicator.

The UE and the UPF may decide not to initiate access performance measurements using PMF over a certain target QoS Flow, when they already have access performance measurements for another target QoS Flow which they determine can be reused

NOTE 3:	How the UE and UPF determine that the performance measurements using a certain target QoS Flow apply to another target QoS Flow is based on implementation e.g. AN resource to QoS Flow mapping in the UE or getting similar access measurements results with other QoS Flow.

The addressing information of the PMF in the UPF is retrieved by the SMF from the UPF during N4 session establishment.

The following PMF protocol messages can be exchanged between the UE and the PMF:

-	Messages to allow for Round Trip Time (RTT) measurements, i.e. when the "Smallest Delay" or "Load-Balancing" steering mode is used;

-	Messages to allow for Packet Loss Rate (PLR) measurements, i.e. when the "Load Balancing" steering mode is used;

-	Messages for reporting Access availability/unavailability by the UE to the UPF.

The PMF protocol is specified in TS 24.193 [109].

The PMF protocol messages used for access availability/unavailability reports shall be sent on the default QoS Flow, i.e. the QoS Flow associated with default QoS rule. The PMF protocol messages used for access performance measurements shall be sent either on the default QoS Flow, or on the target QoS Flow, as specified above.

The QoS Flow associated with default QoS rule for MA PDU Session is Non-GBR QoS Flow.

The UE shall not apply the ATSSS rules and the UPF shall not apply the MAR rules for the PMF protocol messages.

When the UE requests a MA PDU session and indicates it is capable to support the MPTCP functionality with any steering mode and the ATSSS-LL functionality with only the Active-Standby steering mode (as specified in clause 5.32.6.1), the network may send Measurement Assistance Information for the UE to send Access availability/unavailability reports to the UPF. In this case, the UE and UPF shall not perform RTT and PLR measurements using PMF as the UE and UPF can use measurements available at the MPTCP layer.

* * * * End of Changes * * * *