SA WG2 Meeting #S2-145E	S2-2103792

17 - 28 May, 2021, Electronic meeting

Title:	Reply LS on NAS-based busy indication

Response to:	S2-2103768 / R2-2104354

Release:	Release 17

Work Item:	MUSIM / LTE_NR_MUSIM-Core

Source:	SA2

To:	RAN2, CT1, RAN3

Cc:	SA3

Contact Person:

#### Name:	Sašo Stojanovski

E-mail Address:	saso stojanovski intel com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.502 CR2558

1. Overall Description:

SA2 thanks RAN2 for their LS on NAS-based busy indication for Idle and RRC Inactive state (S2-2103768 / R2-2104354).

The Attached CR implements the RAN2 agreement

to enable RAN2 to further analyse the topic and come back to SA2 with any additional feedback as several companies in SA2 have the following concern:-

- The UE resumes from RRC-Inactive when sending the Paging Reject in NAS level. - The RAN is unaware of the content of the NAS message, and forward the NAS message to AMF. The RAN node starts scheduling the DL data or signalling within its buffers for the UE. - Depending upon UE implementation, the UE may discard received packets, which could lead to waste of Uu resources; - This may continue until the UE is released; - RAN receives the N2 release request from the AMF and then release the UE to CM-IDLE/RRC-IDLE.

These companies would like to RAN2 evaluate attached CR and above concerns.

Given that SA2 work on MUSIM is due for completion in June 2021, SA2 agreed to proceed with the agreed TS 23.502 CR2558 including the Editor’s note. A final confirmation from RAN2 is expected to reconfirm the SA2’s working assumption.

Regarding the specific RAN2 questions in the LS:

- Question 1: Are the impacts identified by RAN2 valid?

- Question 2: Are there any other impacts beyond those identified by RAN2?

- Question 3: If the ANS to Q1 and/or to Q2 is yes, can they be specified within Rel-17 timeframe?

SA2 would like to offer the following answers:

-	A1: As indicated above.

-	A2: see the issue description above

-	A3: the attached CR needs only the editor's note to be removed if RAN2 does not change Working assumption. If the RAN2 assumptions change, SA2 needs to update corresponding solution.

2. Actions:

To RAN2 group.

ACTION: 	SA2 respectfully asks RAN2 to consider the above described concern and provide feedback.

To RAN2, CT1 and RAN3 groups.

ACTION: 	SA2 respectfully asks CT1 and RAN3 to take the feedback above into consideration for their work on MUSIM.

3. Date of Next TSG-SA WG2 Meetings:

3GPP SA2#146-e			16 - 27 August 2021			Electronic Meeting

3GPPSA2#147-e	18 - 22 October 2021	Electronic Meeting