3GPP TSG-WG SA2 Meeting #145E e-meeting 	S2-2103816

# Elbonia, May 17th - 28th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0337 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mapping UE IP address and GPSI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | CR 258 rev 1 to 23.288 was agree to allow the AF or NWDAF to map the SUPI to a UE IP address that is used in the data collection from the UE Application. Given the flexibility above, and not having to place additional complexity to the AF in the untrusted domain, the proposal is to have the same procedure for untrusted AF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Th AF in the untrusted may map the GPSI to a UE IP address or the NWDAF may do the mapping instead and provide the UE IP address to the AF. Here we refer to approved CR 258 rev.1 clause 6.2.8.2.4.x. for the NWDAF providing the UE IP address to the AF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification of the conclusions in KI#8. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.2.8.2.4.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---------------------------------First Change---------------------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[5]	Void.

[6]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[7]	3GPP TS 28.550: "Management and orchestration; Performance Assurance".

[8]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[9]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS)".

[10]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[11]	ITU-T Recommendation P.1203.3: "Parametric bitstream-based quality assessment of progressive download and adaptive audiovisual streaming services over reliable transport - Quality integration module".

[12]	3GPP TS 38.215: "NR; Physical layer measurements".

[13]	Void.

[14]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[15]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[16]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[17]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes".

[18]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3".

[19]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[20]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; stage 2".

[21]	3GPP TS 28.201: "Charging management; Network slice performance and analytics charging in the 5G System (5GS); stage 2".

[22]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[23]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[24]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[25]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[xx]	3GPP TS 29.503 : "5G System; Unified Data Management Services; Stage 3".

---------------------------------Second Change-----------------------------------

6.2.8.2.4.x	AF correlates NWDAF request for a GPSI and Data collected from the UE Application for a UE IP address

If the AF receives the Naf_EventExposure_Subscribe from NWDAF, via NEF, including Target for Event Reporting set to GPSI, the AF request the NEF to provide the allocated IPv4 address or IPv6 prefix or both as described in Figure 6.2.8.2.4.z-1.  Note that the NWDAF can also provide the UE IP address to the AF as described in 6.2.8.2.4.x.

Figure 6.2.8.2.4.z-1.: AF in untrusted domain correlates UE data collection and NWDAF request

- Same step as step 0 in figure 6.2.8.2.4.z-1.

- The AF receives a request to retrieve input data as described in clause 6.2.8.2.3 including a GPSI. The AF requests NEF to provide the IPv4address or IPv6 prefix or both serving the PDU session for this GPSI towards the AF using Nnef_UEid_Get_Request.

- The NEF is configured with the DNN,S-NSSAI to access this AF, the NEF finds the SMF serving the PDU session(s) for this GPSI, DNN, S-NSSAI using Nudm_UECM_Get_Request including type of requested information set to SMF Registration Info and the S-NSSAI and DNN, as defined in clause 5.3.2.5.7 in TS 29.503 [xx]. Note that if there are more than one DNN,S-NSSAI combination to access this AF, the NEF will find the SMF(s) serving the PDU session(s) to any of these DNN,S-NSSAIs combinations.

3. The UDM provides the SMF id(s) and the PDU Session id(s) using Nudm_UECM_Get_Response to the NEF.

4. The NEF sends Nsmf_EvenExposure_Subscribe to each of the SMF(s) identified in step 3, including the Target for Event Reporting set to the PDU Session id(s) provided in step 2 and the Event ID set to IP address/prefix allocation/change.

5.	Each SMF provides the allocated IPv4 address or IPv6 prefix or both to the NEF.

6.	The NEF provides the allocated IPv4 address or IPv6 prefix or both provided by each SMF to the AF.

7.	The AF correlates the UE data that includes the UE IP address and the NWDAF request for the GPSI using the retrieved IPv4 address(es) or IP v6 prefix(es).

-----------------------------End of Changes---------------------------------------