3GPP TSG-SA WG2 Meeting #145-e	S2-2103856r01

# Electronic meeting, 2021‑05‑17 -- 2021‑05‑28

Source: Ericsson, Samsung Title: Flow consistency in 23.548 clause 6.2.3.2.2 Document for: Agreement Agenda Item: 8.3 Work Item / Release: eEDGE_5GC /Rel-17 Abstract of the contribution: This contribution makes sure the message flow in 23.548, clause 6.2.3.2.2 is concistent and simplifies the DNS handling in EASDF

# 1	Discussion

The editor’s Note in S 23.548 Rel-0.1.0, clause 6.2.3.2.2:

Editor’s note:	It is FFS whether it is aligned with the usage of notification in SBI. This behaviour looks like SMF is requesting a message DNS handling rule (i.e. where to forward the DNS message or whether to include ECS option, what to include ECS option.). Normally, the Notification message is just sending ack or nack. The NF service model should be re-visited.

Was not fully resolved. It is not clear how the EASDF knows that it should wait for an update from SMF when it notifies SMF in step 8 and step 15. Samsung agree with this view.

Ericsson sees four possible solutions,

1	The rule sent by SMF includes if EASDF should wait or not for further instructions

2	Wait for the Response to the Notification and update the EASDF context before notification response is sent

3	Notification is not used, instead the SMF has a service that is invoked by EASDF

4	The Notication response contains instructions

5	Serializing the events, by including DNS message in the notification from EASDF to SMF. Then EASDF can forget the message. SMF does what is needs to do onn the message, e.g. addig ECS option, and then sends the DNS message in a new opertion to EASDF, which the forward it to DNS server.

4 is as we see it not in line with SBI.Samsung agrees with it.

3 will require new SMF service and EASDF. Samsung thinks it can be a potential option to be discussed. For the case where the EASDF meets the situation when the SMF instruction (or DNS message handling for FQDN that can be newly initiated, which does not require additional transaction between the SMF and EASDF) is required.

2 will mean that the notification request and response are not atomic. Samsung totally agrees with this opinion.

1 is not so clean since it requires state synchronization betwee SMF and EASDF and it also uses an update operation to trigger an action other than only modifying the rules. Samsung thinks that delaying the DNS response CANNOT be an option. It rules the all application layer logic relying on it. Many application layer framework, and edge computing service providers are relying on the existing DNS mechanism as well as its performance. It should NOT be an option.

5 would mean that rules are simpler and less states to be sychronized beween SMF and EASDF. Thus 5 is proposed.

Samsung thinks the proposed solution is NOT aligned with SBI either because it create another method (e.g. DNSsend), which is NOT aligned with SBI design practice.

In summary, option 2 can be viable option. We need to have a further discussion.

Also,we see the Neasdf_DNSContext_Update as a way to modifying the rule set for the EASDF context which can happen at anytime, whereas step 11 and 18 are really about triggering of sending a DNS message. This is not so nice to have the same operation for 2 different purposes

# 2	Proposal

The discussion above is reflected below in proposed changes

Start Changes

##### 6.2.3.2.2	EAS Discovery Procedure with EASDF

For the case that the UE DNS Query is to be handled by EASDF, the following applies.

-	The AF may provide EAS deployment information to UDR, including the list of FQDNs supported by applications for each DNAI, the IP address range(s) corresponding to each DNAI, and the DNS server identifier (consisting of IP address and port) for each DNAI, as defined in clause 5.6.7 of TS 23.501[2]. The AF may update the information as described in clause 4.3.6.2 of TS 23.502[3].

-	During the PDU Session establishment procedure, the SMF gets the EAS deployment information via the PDU Session related policy information from PCF or the SMF is preconfigure with the EAS deployment information, and the SMF selects an EASDF and provides its address to the UE as the DNS Server to be used for the PDU Session. The UE sends DNS Query to the EASDF. The SMF may configure the EASDF with DNS message handling rules to forward DNS messages of the UE to a relevant DNS server and/or report when detecting DNS messages. A DNS message handling rule includes information used for DNS message detection and associated action(s). It is defined as following:

-	Precedence of the DNS message handling rule

-	DNS message detection template (includes at least one of the following):

-	DNS message type = DNS Query or DNS Response or EASDF reference

-	If DNS message type = DNS Query

-	Source IP address (i.e. UE IP address)

-	Array of (FQDN ranges)

-	If DNS message type = DNS Response

-	Array of FQDN ranges and/or array of EAS IP address ranges

-	the EASDF reference corresponds to a reference provided by the EASDF in a Neasdf_DNSContext_Notify request

NOTE 1:	For DNS message type = Query, the UE IP address provided at DNS context creation is considered if not provided explicitly as part of the template.

-	Action template (including one)the possible actions are:

-	Report information from DNS message content to SMF (Neasdf_DNSContext_Notify request). The report can be aAction = report and forward the DNS message or Action = report, store the DNS message and wait for further  instruction .

-	Send the DNS message to the UE

-	Send the DNS message to a specific DNS server and optionally add an EDNS Client Subnet option

-	Send message to a preconfigured DNS server/resolver and optionally add an EDNS Client Subnet option

Multiple actions are possible.

-	when the rule is to add EDNS Client Subnet option the rule Includes the information to build this option in the DNS message (The information for the EASDF to build the ECS option is included in the Action template within the DNS handling rule)

Editor’s Notes:	It is FFS whether the information for the EASDF to build the ECS option may contain more than one IP address to deal with following cases: 1) some EAS may be deployed on the part of the DN with public IP addressing space while other EAS may be deployed on the part of the DN with private IP addressing space, 2) there may be multiple candidate L-PSA (+ULCL) UPF: for example some expensive L-PSA at the DU site related with fairly specific applications that are demanding in terms of delays and some more vanilla L-PSA + ULCL UPF that can provide traffic offload but that are more central and cheaper due to a better pooling effect in a more central location, 3) there may be multiple L-PSA UPFs: for example some applications are supposed to be accessed via one DNAI with one L-PSA UPF and the other applications are supposed to be accessed via another DNAI with another L-PSA UPF.

-	Replacement of the DNS message target address with the indicated DNS Server Address; if no DNS Server Address is provided  by the SMF, then the EASDF is to forward the DNS message to a locally preconfigured DNS server/resolver.

-	Buffer the DNS message and report DNS message content to the SMF

-	Send the buffered DNS response message to UE.

When the EASDF forwards a DNS request, it shall always ensure it receives the DNS answer (putting its own address as the source of the request)

The SMF may use following information to create DNS message handling rules associated with a PDU session:

-	Local configuration associated with the (DNN, S-NSSAI) of the PDU Session, and/or

-	EAS deployment information provided by the AF, and/or

-	Information derived from the UE location such as candidate L-PSA (s).

-	PDU Session information, like PDU Session L-PSA(s) and ULCL/BP

NOTE 2:	For example, the SMF can derive the IP address for ECS based on the N6 IP address(es) associated with serving L-PSA(s) locally configured or in the NRF.

NOTE 3:	Providing in DNS ECS option an IP address associated with the L-PSA UPF protects the privacy of the (IP address of the) UE.

The following are example of actions done by EASDF:

-	If the FQDN in a DNS Query matches a DNS message handling rule provided by SMF. The actions may be one of following. :

-	Option A: The EASDF includes the EDNS Client Subnet (ECS) option into the DNS Query message as defined in RFC 7871[6], and sends the DNS Query message to the DNS server for resolving the FQDN. The DNS server may resolve the EAS IP address considering the ECS option, and sends the DNS Response to the EASDF.

-	Option B: The. EASDFsends the DNS Query message to the indicated Local DNS server which is responsible for resolving the FQDN within the corresponding Local DN. The EASDF receives the DNS Response message from the Local DNS server.

NOTE 4:	Option B does not support the scenario where the EASDF has no direct connectivity with the local DNS servers.

.

-	If the DNS Query from the UE does not match a DNS message handling rules provided by the SMF, then the EASDF may simply forward the DNS Query towards a preconfigured DNS server/resolver for DNS resolution.

-	If the EASDF receives a DNS Response message matching a DNS message hadling rule for which the EASDF should report to the the SMF and if the reporting condition is met, the EASDF may inlude the EAS information (i.e. EAS IP address(es), optionally the EAS FQDN, included in DNS Answer Fieldand the corresponding IP address within the ECS DNS option)  in the DNS message report (Neasdf_DNSContext_Notify request).. The SMF may then trigger UL CL/BP and L-PSA insertion as specified in clause 6.3.3 in TS 23.501 [2] based on the Notification. In the case of Action = “report, store the DNS message and wait for further instruction” the notification from EASDF contains an EASDF context reference that the SMF is to provide to EASDF when providing further action (e.g. Send the DNS message to the UE)

The information to build the EDNS Client Subnet (ECS) option or the Local DNS server address provided by the SMF to the EASDF are part of the DNS message handling . This information is related to DNAI(s) for that FQDN(s) for the UE location.

The SMF may provide DNS message handling rules to handle DNS queries from the UE to the EASDF when the SMF establishes the association with the EASDF for the UE, and may update the rules at any time when the association exists. For the selection of the candidate DNAI for an FQDN for the UE, the SMF may consider the UE location, network topology and information of EAS deployment information received as part of PDU Session related policy information for the PDU Session while it is provided/modified/deleted as defined in TS 23.503 [4] clause 6.4 or be preconfigured into the SMF. The DNS configuration information is provisioned by the AF via the procedure of AF influence on traffic routing as defined in in clause 5.6.7.1 of TS 23.501 [2] and in clause 4.3.6.2 of TS 23.502 [3].. After the UE mobility, if the provided Information for ECS option or the Local DNS server address needs be updated, the SMF may send an update to DNS message forwarding rules to the EASDF.

Editor's note:	The sentence above means that the EASDF gets the FQDN(s) from SMF. How the SMF getting the ECS option needs to be clarified.

Once the UL CL/BP and L-PSA have been inserted, the SMF may decide that the DNS messages for the FQDN are to be handled by local DNS resolver/server from now on. This option is further described in clause 6.2.3.2.3.

Figure 6.2.3.2.2-1: EAS discovery procedure with EASDF

1.	UE sends PDU Session Establishment Request to the SMF as shown in step 1 of clause 4.3.2.2.1 of TS 23.502 [3].

2.	During the PDU Session Establishment procedure, the SMF selects EASDF as described clause 6.3 of TS 23.501 [2]. The SMF includes the IP address of the EASDF as DNS server in PDU Session Establishment Accept message as in step 11 of clause 4.3.2.2.1 of TS 23.502 [3]. The UE configures the EASDF as DNS server for that PDU Session.

3.	The SMF invokes Neasdf_DNSContext_Create Request (UE IP address, DNN, callback URI, DNS message handling rules) to the selected EASDF.

This step is performed before step 11 of PDU Session Establishment procedure in clause 4.3.2.2.1 of TS 23.502 [3].

The EASDF creates a DNS context for the PDU Session, and stores the UE IP address, DNS message handling rule(s) into the context. If a rule has a reporting action, callback URI for this ruls is provided.

The EASDF is provisioned with the DNS message handling rule(s), before the DNS Query message is received at the EASDF or as a consequence of the DNS Query reporting.

4.	The EASDF invokes the service operation Neasdf_DNSContext_Create Response.

Editor's note:	How to guarantee that the UE uses the EASDF's IP address for the subsequent DSN Query in step 8 is FFS.

5.	The SMF may invoke Neasdf_DNSContext_Update Request (EASDF Context ID, DNS message handling rules) to EASDF. The update may be triggered by UE mobility, e.g. when UE moves to a new location, or by a reporting by EASDF of a DNS Query with certain FQDN, or, the update may be triggered by insertion/removal of Local PSA, e.g. to update rules to handle DNS messages from the UE or by new PCC rule information.

6.	The EASDF responds with Neasdf_DNSContext_Update Response.

7.	The UE sends DNS Query message to the EASDF.

8.	If the DNS Query message matches a DNS message handling rule for reporting, the EASDF sends the DNS message to SMF by invoking Neasdf_DNSContext_Notify Request.

9.	The SMF responds with Neasdf_DNSContext_Notify Response.

10.

Editor's note:	It is FFS whether UL CL / BP insertion should be mentioned at this step.

11.	If DNS message handling rule for the FQDN received in the report need to be updated, e.g. provide updates to information to build the ECS option information, the SMF invokes Neasdf_DNSContext_Update Request (DNS message handling rules) to EASDF.

For Option A, the DNS handling rule includes corresponding IP address to be used to build the ECS option. For Option B, the DNS handling rule includes corresponding local DNS Server IP address. The EASDF may as well be instructed by the DNS handling rule to simply forward the DNS Query to a pre-configured DNS server/resolver.

NOTE 5:	The DNS handling rules are flexible, e.g. instead of step 11 as described, SMF could have already

12.	The EASDF responds with Neasdf_DNSContext_Update Response.

13.	The EASDF handles the DNS Query message received from the UE as the following:

-	For Option A, the EASDF adds the ECS option into the DNS Query message as specified in RFC 7871[6] and sends it to C-DNS server;

-	For Option B, the EASDF sends the DNS Query message to the Local DNS server.

If no DNS message detection template within the DNS message handling rule provided by the SMF matches the requested FQDN in the DNS Query, the EASDF may simply send a DNS Query to a pre-configured DNS server/resolver.

14.	EASDF receives DNS Responses from the DNS system and determines that a DNS Response can be sent to the UE.

15.	The EASDF may send an DNS message reporting to the SMF by invoking Neasdf_DNSContext_Notify request including EAS information if the EAS IP address or the FQDN in the DNS Response message matches the reporting condition provided by the SMF. The DNS message reporting may contain multiple EAS IP address if the EASDF has received multiple EAS IP address(es) from the DNS servers it has contacted.

In the case of Action = “report, store the DNS message and wait for further instruction” the notification from EASDF contains an EASDF context reference

Per the received DNS message handling rule, the EASDF does not send the DNS Response message to the UE but waits for SMF instructions (in step 18) , i.e. buffering the DNS Response message.

16.	The SMF invokes Neasdf_DNSContext_Notify Response service operation.

17.	The SMF may perform UL CL/BP and Local PSA selection and insert UL CL/BP and Local PSA.

Based on received EAS information received from the EASDF and other UPF selection criteria, as specified in clause 6.3.3 in TS 23.501 [2], the SMF may determine the DNAI and determine the associated N6 traffic routing information for the DNAI. The SMF may perform UL CL/BP and Local PSA selection and insertion as described in TS 23.502 [3].

18.	The SMF invokes Neasdf_DNSContext_Update Request (DNS message handling rules,. EASDF context reference received in step 15)

The DNS message handling rule indicates the EASDF to send a DNS Response buffered in Step 15 to UE.

19.	The EASDF responds with Neasdf_DNSContext_Update Response.

20.	The EASDF sends the DNS Response to UE.

Next Changes

## 7.1	EASDF Services

### 7.1.1	General

The following table illustrates the EASDF Services and Service Operations.

Table 7.1.1-1: NF services provided by the EASDF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Neasdf_DNSContext | Create | Request/Response | SMF |
|  | Update | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
|  | Notify | Subscribe/Notify | SMF |

### 7.1.2	Neasdf_DNSContext service

#### 7.1.2.1	General

Service description: This service enables the consumer to create, update, or delete DNS context in EASDF and to Subscribe to DNS signalling related reporting from EASDF.

DNS contexts in EASDF include rules on how EASDF is to handle DNS messages.

#### 7.1.2.2	Neasdf_DNSContext_Create service operation

Service operation name: Neasdf_DNSContext_Create.

Description: Create a DNS context in EASDF.

Input, Required: UE IP address, DNN, callback URI, DNS message handling rule (DNS message detection, Action(s)).

DNS message detection and Actions(s) are specified in clause 6.2.3.2.2.

Input, Optional: None.

Output, Required: IP address of the EASDF, EASDF Context ID, Success or Failure.

Output, Optional: None.

#### 7.1.2.3	Neasdf_DNSContext_Update service operation

Service operation name: Neasdf_DNSContext_Update.

Description: Update the DNS context in EASDF.

Input, Required: EASDF Context ID, DNS handling rules.

Input, Optional:. EASDF context reference

Output, Required: Success or Failure.

Output, Optional: None.

#### 7.1.2.4	Neasdf_DNSContext_Delete service operation

Service operation name: Neasdf_DNSContext_Release.

Description: Delete the DNS context in EASDF.

Input, Required: EASDF Context ID.

Input, Optional: None.

Output, Required: Success or Failure.

Output, Optional: None.

#### 7.1.2.5	Neasdf_DNSContext_Notify service operation

Service operation name: Neasdf_DNSContext_Notify.

Description: EASDF reports DNS signalling related information to the consumer when receiving DNS Query or DNS Response.

Input, Required: DNS message content reporting information specified in clause 6.2.3.2.2).

Input, Optional: EASDF context reference.

Output, Required: Success or Failure.

Output, Optional: None.