3GPP TSG-SA2 # 145E (e-meeting)	S2-2104165

# May 17 – May 28, 2021                                       (revision of S2-2103286)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0314 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Historical Data and Analytics storage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-4-5 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is no procedures for storing historical data of NWDAF, DCCF, MFAF into ADRF. SA2#144E left how ADRF triggers data and/or analytics collection, as captured in the editor's note: Editor's note: Additional step for ADRF to subscribe to data towards NF is FFS. And in the current procedures, ADRF can only passively receive data and/or analytics from consumers, and there is no mechanism to avoid storing the same data and/or analytics repeatedly. Because the same data and/or analytics may be consumed by multiple consumers, and based on the configuration or consumer request, these data and/or analytics may be sent to ADRF by multiple consumers. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of procedures to store historical data of NWDAF, DCCF, MFAF into ADRF. If there is no matching data in ADRF, it will store the historical data, if there is matching data in ADRF, it will refuse to store the historical data. The first change is related to how ADRF triggers the storage of historical data and/or analytics. The second change relates to ADRF avoids receiving the same data and/or analytics from consumers repeatedly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedures for historical data handling not specified. ADRF can only store data and/or analytics when requested by the consumer. ADRF receives the same data and/or analytics repeatedly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.6.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

6.X.2	Historical Data and Analytics storage

The procedure depicted in figure 6.x.2-1 is used by consumers (e.g. NWDAF, DCCF or MFAF) to store historical data and/or analytics, i.e. data and/or analytics related to past time period that has been obtained by the consumer. After the consumer obtains data and/or analytics, consumer may store historical data and/or analytics in an ADRF. Whether the consumer directly contacts the ADRF or goes via the DCCF or via the Messaging Framework is based on configuration.

Figure 6.x.2-1: Historical Data and Analytics storage

1.	The consumer sends data and/or analytics to the ADRF by invoking the Nadrf_DataManagement_StorageRequest(collected data, analytics) service operation.

2.	The ADRF stores the data and/or analytics sent by the consumer. The ADRF may, based on implementation, determines whether the same data and/or analytics is already stored or being stored based on the information sent in step 1 by the consumer. And if the data and/or analytics is already stored and/or being stored in the ADRF(e.g. may stored by other consumer), the ADRF refuse the request from consumer and not store the data and/or analytics sent by the consumer.

3. The ADRF sends Nadrf_DataManagement_StorageRequest Response message to the consumer indicating that data and/or analytics is stored, including when the ADRF may have determined at step 2 that data or analytics is already stored.

* * *End of Changes * * *