3GPP TSG-WG SA2 Meeting #145E e-meeting  	S2-2104184r09

# Elbonia, May 17 – 28, 2021	(revision of S2-2102976)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2755 | rev | 2 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | De-registration for onboarding registered UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the TR 23700-07, On-boarding network shall restrict the usage only for on-boarding service, and the on-boarding UE should be de-registrated after finishing the remote provisioning. When the remote provisioning for SNPN credentials is finished the UE or the ON-SNPN shall initiate de-registration from the ON-SNPN. AMF shall initiate the de-registration when the expiration of the on-boarding timer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on the TR conclusion, add the description that the on-boarding network shall restrict the usage only for on-boarding service, and the on-boarding UE should be de-registrated after finishing the remote provisioning. AMF shall initiate the de-registration when the expiration of the on-boarding timer. 1. Change the NOTE for deregistration from ON-SNPN into normative text 2. Add description about deregistration when ON is a PLMN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing new features as per conclusion of TR 23.700.07 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.X.2.7 (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** Start of changes *****

##### 5.30.2.X.2.7	Deregistration from the ON-SNPN for onboarding registered UE

Once remote provisioning of SO-SNPN credentials is completed, the UE should initiate deregistration from the ON-SNPN.

Based on ON-SNPN policies, the AMF may start  an implementation specific timer once the UE has registered to the ON-SNPN for the purpose of onboarding. Expiry of this timer triggers the AMF to deregister the onboarding registered UE from the ON-SNPN.

NOTE X1:	This specific timer is used to prevent onboarding registered UEs from staying at the ON-SNPN indefinitely.

.

NOTE X2:	When Onboarding network is a PLMN and the UE’s subscription only allows for Remote Provisioning, then based on PLMN policies, the AMF can start an implementation specific timer once the UE has registered to the PLMN. Expiry of this timer triggers the AMF to deregister the UE from the PLMN. This specific timer is used to prevent registered UEs that only allow for Remote Provisioning from staying at the PLMN indefinitely.

***** End of changes *****